IEEE P802.11
Wireless LANs

| D0.1 comment resolution for #292 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-09-10 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Thomas Derham | Orange |  |  | thomas.derham@orange-ftgroup.com |
| Assaf Kasher | Intel Corporation |  |  | assaf.kasher@intel.com |
| Kapseok Chang | ETRI |  |  | kschang@etri.re.kr |
| Seung Eun Hong | ETRI |  |  | iptvguru@etri.re.kr |

Abstract

This document proposes resolution for comment 292 in the Spatial Reuse subgroup for Draft 0.1 of TGad.

| 292 | 7.3.2.22.11 | 56 | 5-7 | The Measurement for Time Block fields are set to the ANIPI or average RSNI value. The current draft does not describe how to encode these values in 1 byte field. | Please specify how to encode the ANIPI or average RSNI value. |
| --- | --- | --- | --- | --- | --- |

Proposed Resolution: Counter

TGad Editor: in 7.3.2.22.11, page 56, line 7, insert the sentences “ANIPI is set to the average noise plus interference power value measured during the indicated Measurement Duration using the same units and accuracy as defined for ANPI in 11.10.8.4. Average RSNI is set according to 7.3.2.41, where RCPI is defined in the RCPI measurement clause in Clause 21.”

TGad Editor: in 7.3.2.22.12, page 57, line 16, insert the sentence “ANIPI is set to the average noise plus interference power value measured during the indicated Measurement Duration using the same units and accuracy as defined for ANPI in 11.10.8.4. RCPI indicates the received channel power of the corresponding Link Measurement Request frame in a dBm scale, as defined in the RCPI measurement clause in Clause 21.”

TGad Editor: insert a new subclause after 21.3.9: “Received channel power indicator (RCPI) measurement” with text identical to 20.3.22.6.

TGad Editor: insert a new line in table 64, after the RSSI line:

| RCPI | Is a measure of the received RF power averaged over all the receive chains in the data portion of a received frame. Refer to 21.3.10 (Received channel power indicator (RCPI) measurement) for the definition of RCPI. | N | Y |
| --- | --- | --- | --- |