IEEE P802.11
Wireless LANs

| Key Descriptor Version Usage |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-09-10 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Dan Harkins | Aruba Networks | 1322 Crossman ave, Sunnyvale, CA | +1 408 227 4500 | dharkins at arubanetworks dot com |

Abstract

This submission proposes changes to the definition of how EAPOL-Key frames are processed and to deprecate use of the Key Descriptor Version.

Editor: modify section 11.5.2 as indicated:

    - EAPOL-Key frames

Editor: Modify figure 11-27 so “Key MIC” length is “variable” and not 16 octets. Modify figure 11-28 such that “B0  B2” is above the “Key Descriptor Value” and “B3” is the sole value above “Key Type”.

- Key Information. This field is 2 octets and specifies characteristics of the key. See Error: Reference source not found.

The bit convention used is as in 7.1 of IEEE Std 802.1X-2004. The subfields of the Key Information field are as follows:

- Key Descriptor Version (bits 0–2) is deprecated. It shall be ignored upon receipt of all EAPOL-Key frames. It shall be set to zero on all transmitted EAPOL-Key frames except under the following circumstances for backwards compatibility: .

- The value 1 shall be used for all EAPOL-Key frames to a STA when the negotiated AKM is 00-0F-AC:1 or 00-0F-AC:2 (11r)and the pairwise cipher is TKIP or "Use Group Cipher" for Key Descriptor 1.(#1115) This value indicates the following:

- HMAC-MD5 is the EAPOL-Key MIC.

- ARC4 is the EAPOL-Key encryption algorithm used to protect the Key Data field.

- The MIC is 16 octets

- The value 2 shall be used for all EAPOL-Key frames to a STA when the negotiated AKM is 00-0F-AC:1 or 00-0F-AC:2 and(11r) either the pairwise or the group cipher is an enhanced data cryptographic encapsulation mechanism other than TKIP(#1115) for Key Descriptor 2. This value indicates the following:

- HMAC-SHA1-128 is the EAPOL-Key MIC. HMAC is defined in IETF RFC 2104; and SHA1, by FIPS PUB 180-2-2002(#1142). The output of the HMAC-SHA1 shall be truncated to its 128 MSBs (octets 0–15 of the digest output by HMAC-SHA1), i.e., the last four octets generated shall be discarded.

- The NIST AES key wrap is the EAPOL-Key encryption algorithm used to protect the Key Data field. IETF RFC 3394 defines the NIST AES key wrap algorithm.

- The MIC is 16 octets

- The value 3 shall be used for all EAPOL-Key frames to a STA when the negotiated AKM is 00-0F-AC:3, 00-0F-AC:4, 00-0F-AC:5, or 00-0F-AC:6(11w). This value indicates the following:(11r)

- AES-128-CMAC is the EAPOL-Key MIC. AES-128-CMAC is defined by FIPS SP800-38B and also found in RFC 4493[B37](#1258).(#1229) The output of the AES-128-CMAC shall be 128 bits.(11r)

- The NIST AES key wrap is the EAPOL-Key encryption algorithm used to protect the Key Data field. IETF RFC 3394 defines the NIST AES key wrap algorithm.(11r)

- The MIC is 16 octets.

- Key MIC. The length of this field depends on the negotiated AKM per section 11.5.2a (EAPOL-Key frame construction and processing).

The key wrap algorithm selected depends on the negotiated AKM per section 11.5.2a (EAPOL-Key frame construction and processing).

Editor: add new section 11.5.2a and assign the appropriate table number to TBD

11.5.2a EAPOL-Key frame construction and processing

EAPOL-Key frames are constructed and processed according to the AKM negotiated at association time. The negotiated AKM determines what algorithm is used to construct and verify a MIC, the size of the MIC, and the algorithm used to wrap and unwrap the Key Data field.

Table TBD indicates the particular algorithms to use when constructing and processing EAPOL-Key frames. The AKM of “Deprecated” indicates an AKM of 00-0F-AC:1 or 00-0F-AC:2 when either TKIP or “Use Group Cipher” is the negotiated pairwise cipher. For all other AKMs the negotiated pairwise cipher suite does not influence the algorithms used to process EAPOL-Key frames.

|  | Integrity algorithm | Size of MIC | Key-wrap algorithm |
| --- | --- | --- | --- |
| Deprecated | HMAC-MD5 | 16 | ARC4 |
| 00-0F-AC:1 | HMAC-SHA1-128 | 16 | NIST AES Key Wrap |
| 00-0F-AC:2 | HMAC-SHA1-128 | 16 | NIST AES Key Wrap |
| 00-0F-AC:3 | AES-128-CMAC | 16 | NIST AES Key Wrap |
| 00-0F-AC:4 | AES-128-CMAC | 16 | NIST AES Key Wrap |
| 00-0F-AC:5 | AES-128-CMAC | 16 | NIST AES Key Wrap |
| 00-0F-AC:6 | AES-128-CMAC | 16 | NIST AES Key Wrap |

Table TBD

Editor, make the following change to 11.5.4.2, 11.5.4.3, 11.5.4.4, 11.5.4.5, 11.5.5.2, 11.5.5.3, 11.5.9.2.2, 11.5.9.2.3, 11.5.9.2.4, 11.5.9.2.5, 11.5.9.2.6, 11.5.9.4 and 11.5.9.5.1.

Key Information:

Key Descriptor Version = 0 (11r)

References: