IEEE P802.11
Wireless LANs

| CID 156, 353-367 – Definition of MIB Objects |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-09-08 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Daniel R. Borges | Apple Inc. | 1 Infinite Loop MS 306-2HN Cupertino, CA, 94107 | +1 (415) 425-7347 | drborges@apple.com |

Abstract

This document proposes MIB Management Object definitions for CID 156, 353, 354, 355, 356, 357, 358, 359, 360, 361, 362, 363, 364, 365, 366, and 367.

# Annex D

In the Major Section of Annex D, insert the following comment to the end of SMT attributes after dot11HTStationConfigTable:

-- dot11mmWaveSTAConfigTable ::= { dot11smt 18 }

In the Major Section of Annex D, insert the following comment to the end of dot11mac section after dot11ResourceInfoTable:

-- dot11mmWaveOperationTable	::= { dot11mac 8 }

-- dot11mmWaveCountersTable	::= { dot11mac 9 }

In the Major Section of Annex D, insert the following comment to the end of dot11phy section after dot11TransmitBeamformingConfigTable:

-- dot11mmWavePHYConfigTable		 ::= { dot11phy 19 }

-- dot11mmWavePHYTxRateTable		 ::= { dot11phy 20 }

-- dot11mmWavePHYRxRateTable		 ::= { dot11phy 21 }

-- dot11mmWaveBeamformingConfigTable ::= { dot11phy 22 }

After the definition of the dot11HTStationConfigTable, insert the dot11mmWaveSTAConfigTable as defined below:

-- ********************************************************************

-- * dot11mmWaveSTAConfig TABLE

-- ********************************************************************

dot11mmWaveSTAConfigTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11mmWaveSTAConfigEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The dot11mmWaveSTAConfig Table"

::= { dot11smt 18 }

dot11mmWaveSTAConfigEntry OBJECT-TYPE

SYNTAX Dot11mmWaveSTAConfigEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11mmWaveSTAConfig Table.

ifIndex - Each IEEE 802.11 interface is represented by an

ifEntry.  Interface tables in this MIB module are indexed

by ifIndex."

INDEX { ifIndex }

::= { dot11mmWaveSTAConfigTable 1 }

Dot11mmWaveSTAConfigEntry ::=

SEQUENCE {	dot11MultibandSupportActivated	TruthValue,

}

-- CID 156

dot11MultibandSupportActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Multiband Support Enabled

This attribute, when true, indicates that the capability

of the station to operate in multiple bands is enabled.

The capability is disabled, otherwise. The default value

of this attribute is false."

DEFVAL { false }

::= {  dot11mmWaveSTAConfigEntry 1 }

After the definition of the dot11mmWavePHYConfigTable, insert the dot11mmWaveBeamformingConfigTable as defined below:

-- ********************************************************************

-- * dot11mmWaveBeamformingConfig TABLE

-- ********************************************************************

dot11mmWaveBeamformingConfigTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11mmWaveBeamformingConfigEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The dot11mmWaveBeamformingConfig Table"

::= { dot11phy 22 }

dot11mmWaveBeamformingConfigEntry OBJECT-TYPE

SYNTAX Dot11mmWaveBeamformingConfigEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11mmWaveBeamformingConfig Table.

ifIndex - Each IEEE 802.11 interface is represented by an

ifEntry.  Interface tables in this MIB module are indexed

by ifIndex."

INDEX { ifIndex }

::= { dot11mmWaveBeamformingConfigTable 1 }

Dot11mmWaveBeamformingConfigEntry ::=

SEQUENCE {	dot11MaxBFTime				Unsigned32,

dot11BFTXSSTime				Unsigned32,

dot11MaximalSectorScanUSec		Unsigned32,

dot11ABFTRSSS				Unsigned32,

dot11RSSRetryLimit			Unsigned32,

dot11RSSBackoff				Unsigned32,

dott11BFRetryLimit			Unsigned32

}

-- CID 353

dot11MaxBFTime OBJECT-TYPE

SYNTAX Unsigned32 (1..16)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Maximum Beamforming Time"

DEFVAL { 4 }

::= { dot11mmWaveBeamformingConfigEntry 1 }

-- CID 354, CID360

dot11BFTXSSTime OBJECT-TYPE

SYNTAX Unsigned32 (1..4294967295)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Beamforming Transmission SS Time (in Microseconds)"

DEFVAL { 256 }

::= { dot11mmWaveBeamformingConfigEntry 2 }

-- CID 355

dot11MaximalSectorScanUSec OBJECT-TYPE

SYNTAX Unsigned32 (1..4294967295)

UNITS "microseconds"

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Maximal Sector Scan (in Microseconds)"

DEFVAL { 5000 }

::= { dot11mmWaveBeamformingConfigEntry 3 }

-- CID 356

dot11ABFTRSSS OBJECT-TYPE

SYNTAX Unsigned32 (1..4294967295)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"A-BFT Responder Sector Sweep"

DEFVAL { 4 }

::= { dot11mmWaveBeamformingConfigEntry 4 }

-- CID 357

dot11RSSRetryLimit OBJECT-TYPE

SYNTAX Unsigned32 (1..4294967295)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Responder Sector Sweep Retry Limit"

DEFVAL { 3 }

::= { dot11mmWaveBeamformingConfigEntry 5 }

-- CID 358

dot11RSSBackoff OBJECT-TYPE

SYNTAX Unsigned32 (1..4294967295)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Responder Sector Sweep Backoff"

DEFVAL { 8 }

::= { dot11mmWaveBeamformingConfigEntry 6 }

-- CID 359

dott11BFRetryLimit OBJECT-TYPE

SYNTAX Unsigned32 (1..4294967295)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Beamforming Retry Limit"

DEFVAL { 3 }

::= { dot11mmWaveBeamformingConfigEntry 7 }

After the definition of the dot11ResourceInfoTable, insert the dot11mmWaveOperationTable as defined below:

-- ********************************************************************

-- * dot11mmWaveOperation TABLE

-- ********************************************************************

dot11mmWaveOperationTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11mmWaveOperationEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The dot11mmWaveOperation Table"

::= { dot11mac 8 }

dot11mmWaveOperationEntry OBJECT-TYPE

SYNTAX Dot11mmWaveOperationEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11mmWaveBeamformingConfig Table.

ifIndex - Each IEEE 802.11 interface is represented by an

ifEntry.  Interface tables in this MIB module are indexed

by ifIndex."

INDEX { ifIndex }

::= { dot11mmWaveOperationTable 1 }

Dot11mmWaveOperationEntry ::=

SEQUENCE {	dot11MaxLostBeacons			Unsigned32,

dot11MinBIHeaderDuration		Unsigned32,

dot11PSRequestSuspensionInterval	Unsigned32,

dot11AssocRespConfirmTime		Unsigned32,

dot11BroadcastSTAInfoDuration		Unsigned32,

dot11NbrOfChangeBeacons			Unsigned32,

dot11ImplicitHandoverLostBeacons	Unsigned32,

dot11ReplayTPATime			Unsigned32

}

-- CID 361

dot11MaxLostBeacons OBJECT-TYPE

SYNTAX Unsigned32 (1..4294967295)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Maximum Number of Lost Beacons"

DEFVAL { 4 }

::= { dot11mmWaveOperationEntry 1 }

-- CID 362

dot11MinBIHeaderDuration OBJECT-TYPE

SYNTAX Unsigned32 (1..4294967295)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Minimum Beacon Interval Header Duration (in Microseconds)"

DEFVAL { 5000 }

::= { dot11mmWaveOperationEntry 2 }

-- CID 363

dot11PSRequestSuspensionInterval OBJECT-TYPE

SYNTAX Unsigned32 (1..4294967295)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"PS Request Suspension Interval"

DEFVAL { 4 }

::= { dot11mmWaveOperationEntry 3 }

-- CID 364

dot11AssocRespConfirmTime OBJECT-TYPE

SYNTAX Unsigned32 (1..4294967295)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Association Response Confirmation Time (in Microseconds)"

DEFVAL { 1000 }

::= { dot11mmWaveOperationEntry 4 }

-- CID 365

dot11BroadcastSTAInfoDuration OBJECT-TYPE

SYNTAX Unsigned32 (1..4294967295)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Broadcast STA Information Duration"

DEFVAL { 32 }

::= { dot11mmWaveOperationEntry 5 }

-- CID 367

dot11NbrOfChangeBeacons OBJECT-TYPE

SYNTAX Unsigned32 (1..4294967295)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Number of Change Beacons"

DEFVAL { 4 }

::= { dot11mmWaveOperationEntry 6 }

-- CID 368

dot11ImplicitHandoverLostBeacons OBJECT-TYPE

SYNTAX Unsigned32 (1..4294967295)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Implicit Handover Lost Beacons"

DEFVAL { 8 }

::= { dot11mmWaveOperationEntry 7 }