IEEE P802.11
Wireless LANs

| Normative Text for Network Channel Control |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-09-13 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Chen Sun | NICT | 3-4, Hikarino-oka, Yokosuka, Kanagawa, Japan, 239-0847 | +81 45 847 5062 | sun@nict.go.jp |
| Zhou Lan | NICT | 3-4, Hikarino-oka, Yokosuka, Kanagawa, Japan, 239-0847 |  | lan@nict.go.jp |
| Chin Sean Sum | NICT | 3-4, Hikarino-oka, Yokosuka, Kanagawa, Japan, 239-0847 |  | sum@nict.go.jp |
| Yohannes D. Alemseged | NICT | 3-4, Hikarino-oka, Yokosuka, Kanagawa, Japan, 239-0847 |  | yohannes@nict.go.jp |
| Ha Nguyen Tran | NICT | 3-4, Hikarino-oka, Yokosuka, Kanagawa, Japan, 239-0847 |  | haguen@nict.go.jp |
| Junyi Wang | NICT | 3-4, Hikarino-oka, Yokosuka, Kanagawa, Japan, 239-0847 |  | Junyi.wang@nict.go.jp |
| Hiroshi Harada | NICT | 3-4, Hikarino-oka, Yokosuka, Kanagawa, Japan, 239-0847 |  | harada@nict.go.jp |

Abstract

This document is a normative text proposal for IEEE 802.11af/D0.05.  A TVBD that does not use geo-location capability of access to the TV bands database shall operate in client mode, where the transmissions of the TVBD, including frequencies of operation, are under control of the master device. (cf. 47 CFR-PART15, 15.703, Definitions) The Network Channel Control frame is given to allow a mater device to control the network channel usage in an operating class of a client device in the context of 802.11af. The frame allows a dependent STA to seek control for one or multiple channels at the same time. Further, the enabling STA should be able to set up control for one or multiple channels at the same time. This allows for more flexible operation on network channels for IEEE 802.11 products in TV bands. The frame can be exchanged between a master device and a client device using the public action frame or transport protocols such as the generic advertisement service (GAS) protocol.

The proposal addresses comments 113 and 114 in IEEE 802.11-10/0595r9. This document is the revision of IEEE 802.11-10/072r4

NOTE—IEEE 802.11 terms differ from FCC 47CFR [B8], Section 15.703 Definitions in many particulars, but generally an enabling STA is a Master Mode station, and each dependent STA has only one enabling STA at any time. Generally, dependent STAs are Client stations. (cf. 802.11af/D0.03)

6. Layer Management

6.3 MLME SAP Interface

TGaf Editor: Insert a new subclause at the end of clause 10.3 as follows:

6.3.af2 Network channel control management

The following MLME primitives support the signaling of network channel control management.

6.3.af2.1 MLME-NETWORKCHANNELCONTROL.request

6.3.af2.1.1 Function

This primitive requests that a (Protected) Network Channel Control public action frame be sent by a STA.

6.3.af2.1.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-NETWORKCHANNELCONTROL.request (

Dialog Token,

NetworkChannelEnablment,

Protected,

VendorSpecificInfo

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Dialog Token | Integer | 0-255 | The dialog token to identify the network channel control transaction |
| NetworkChannelControl | A set of information subfields | As described in 8.5.8.af2 | As described in 8.5.8.af2 |
| Protected | Boolean | true, false | Specifies whether the request is sent using a Robust Management frame. If true, the request is sent using the Protected Network Channel Control frame. If false, the request is sent using the Network Channel Control frame. |
| VendorSpecificInfo | A set of elements | As defined in 7.3.2.26 (Vendor Specific element) | Zero or more elements. |

6.3.af2.1.3 When generated

This primitive is generated by the STA management entity (SME) to request that a (protected) Network Channel Control public action frame be sent to a STA that is associated to the enabling STA.

6.3.af2.1.4 Effect of receipt

On receipt of this primitive, the MLME constructs and transmits a (Protected) Network Channel Control public action frame.

6.3.af2.2 MLME-NETWORKCHANNELCONTROL.confirm

6.3.af2.2.1 Function

This primitive reports the result of a request to network channel control.

6.3.af2.2.2 Semantics of the service primitive

The primitive parameters as follows:

MLME- NETWORKCHANNELCONTROL.confirm (

Dialog Token,

ResultCode,

VendorSpecificInfo

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Dialog Token | Integer | 0-255 | The dialog token to identify the network channel control transaction |
| ResultCode | Enumeration | SUCCESS, INVALID_PARAMETERS, UNSPECIFIED_FAILURE | Reports the result of a network channel control request |
| VendorSpecificInfo | A set of elements | As defined in 7.3.2.26 (Vendor Specific element) | Zero or more elements. |

6.3.af2.2.3 When generated

This primitive is generated by the MLME when a Network Channel Control request completes.  Possible unspecified failure causes include an inability to schedule a Network Channel Control public action frame.

6.3.af2.2.4 Effect of receipt

The SME is notified of the results of the network channel control procedure.

6.3.af2.3 MLME-NETWORKCHANNELCONTROL.indication

6.3.af2.3.1  Function

This primitive indicates that a (Protected) Network Channel Control public action frame was received from an STA.

6.3.af2.3.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-NETWORKCHANNELCONTROL.indication (

PeerMACAddress,

Dialog Token,

NetworkChannelControl

Protection,

VendorSpecificInfo

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerMACAddress | MACAddress | Any valid individual MAC address | The address of the peer MAC entity from which the Network Channel Control public action frame was received. |
| Dialog Token | Integer | 0-255 | The dialog token to identify the network channel control transaction |
| NetworkChannelControl | A set of information subfields | As described in 8.5.8.af2 | As described in 8.5.8.af2 |
| Protected | Boolean | true, false | Specifies whether the request was received using a Robust Management frame. If true, the request was received using the Protected Network Channel Control public action frame. If false, the request was received using the Network Channel Control public action frame. |
| VendorSpecificInfo | A set of elements | As defined in 7.3.2.26 (Vendor Specific element) | Zero or more elements. |

6.3.af2.3.3 When generated

This primitive is generated by the MLME when a valid (Protected) Network Channel Control public action frame is received.

6.3.af2.3.4 Effect of receipt

On receipt of this primitive, the SME decides whether to accept the switch request.

6.3.af2.4 MLME-NETWORKCHANNELCONTROL.response

6.3.af2.4.1  Function

This primitive is used to schedule an accepted network channel control.

6.3.af2.4.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-NETWORKCHANNELCONTROL.response (

Dialog Token,

NetworkChannelEnablment

VendorSpecificInfo

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Dialog Token | Integer | 0-255 | The dialog token to identify the network channel control transaction |
| NetworkChannelControl | A set of information subfields | As described in 8.5.8.af2 | As described in 8.5.8.af2 |
| VendorSpecificInfo | A set of elements | As defined in 7.3.2.26 (Vendor Specific element) | Zero or more elements. |

6.3.af2.4.3 When generated

This primitive is generated by the SME to schedule an accepted network channel control request.

6.3.af2.4.4 Effect of receipt

On receipt of this primitive, the MLME schedules the network channel control.

8 Frame formats

8.4 Management frame body components

8.4.5 Registered Location Query Protocol elements

Change Table 8-35af1 as shown:

Table 8-35af1—Registered Location Query Protocol info ID definitions

| Info Name | Info ID | RLQP Info Element (clause) |
| --- | --- | --- |
| Reserved | 0 | N/A |
| DSE Enablement | 1 | 7.3.5.1 |
| Channel Power Management | 2 |  |
| Network Channel Control | 3 |  |
| Reserved | 43-220 | N/A |
| Vendor Specific | 221 | 7.3.2.26 |
| Reserved | 222-255 | N/A |

Add the following section after 8.4.5.2:

- 8.4.5.3 Network Channel Control element

The Network Channel Control element of RLQP is used to request/respond to network channel control using the GAS protocol rather than using dedicated Public Action frames. The element is in the format shown in Figure 7-35af1.

|  |  |  |  |  |  |  | These three fields are repeated, as determined by the Length field |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Info ID |  | Length |  | Requester STA address | Responder STA Address | Operating Class | Channel Number | Transmit Power Constraint | … |
| Octets: | 1 |  | 1 |  | 6 | 6 | 1 | 1 | 1 |  |

Figure 8-35af4—Network Channel Control frame body format

The Info ID field shall be set to the value for DSE Control defined in Table 8-35af1.

The Length field indicates the length of the remaining frame fields in octets, and the value is variable.

The RequesterSTAAddress field is the MAC address of the requesting STA that initiates the Network Channel Control process.

The ResponderSTAAddress field is the MAC address of the responding STA that grants Network Channel Control.

The Operating Class field indicates the channel set for which the network channel control request applies. The Operating Class and Channel Number fields together specify the channel frequency and channel bandwidth for which the channel control applies. Valid values for the Operating Class field are shown in Annex J.

The Network Channel Number field of control request indicates the network channel numbers for which dependent STA requests. The Network Channel Number field of the control response indicates the network channel granted by the enabling STA. The dependent STA can select a set of network channels from the list of available channels and the enabling STA can grant a subset of the network channels selected by the dependent STA. The channel number is defined within an operating class as shown in Annex J.

The Transmit Power Constraint field gives the maximum allowable transmit power in dBm for TV bands operation. The field can be ignored by the enabling STA if the Network Channel Control frame is sent from a dependent STA to an enabling STA as a control request.

- 8.5.8 Public Action details

- 8.5.8.1 Public Action frames

Change Table 8-113 as shown:

Table 8-113—Public Action field values

| Action field value | Description |
| --- | --- |
| 0 | Reserved |
| 1 | DSE enablement |
| 2 | DSE deenablement |
| 3 | DSE Registered Location Announcement |
| 4 | Extended Channel Switch Announcement |
| 5 | DSE measurement request |
| 6 | DSE measurement report |
| 7 | Measurement Pilot |
| 8 | DSE power constraint |
| <ANA> | Network Channel Control |
| <ANA>-255 | Reserved |

TGaf editor: Insert the following new clauses after the last subclause of 8.5.8:

- 8.5.8.af2 Network Channel Control frame format

The Network Channel Control frame is a Public Action frame. It is transmitted by a STA and a dependent STA as part of the procedure to enable control of a dependent STA by an enabling STA for a particular 802.11 channel in TV bands under an operating class. The format of the Network Channel Control frame body is shown in Figure 8-47af2.

|  |  |  |  |  |  |  | These three fields are repeated, as determined by the Length field |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Category | Action Value |  | Length |  | Requester STA address | Responder STA Address | Operating Class | Channel Number | Transmit Power Constraint | … |
| 1 Octets: | 1 |  | 1 |  | 6 | 6 | 1 | 1 | 1 |  |

Figure 8-47af2—Network Channel Control frame body format

The Category field is set to the value indicating the Public category, as specified in Table 8-36 (Category values) in 8.4.1.11 (Action field).

The Action Value field is set to the value indicating Network Channel Control frame, as specified in

Table 8-131 (Public Action field values) in 8.5.8.1 (Public Action frames).

The Length field indicates the length of the remaining frame fields in octets, and the value is variable.

The RequesterSTAAddress field is the MAC address of the requesting STA that initiates the Network Channel Control process.

The ResponderSTAAddress field is the MAC address of the responding STA that grants Network Channel Control.

The Operating Class field indicates the channel set for which the network channel control request applies. The Operating Class and Channel Number fields together specify the channel frequency and channel bandwidth for which the channel control applies. Valid values for the Operating Class field are shown in Annex J.

The Network Channel Number field of control request indicates the network channel numbers for which dependent STA requests. The Network Channel Number field of the control response indicates the network channel granted by the enabling STA. The dependent STA can select a set of network channels from the list of available channels and the enabling STA can grant a subset of the network channels selected by the dependent STA. The channel number is defined within an operating class as shown in Annex J.

The Transmit Power Constraint field gives the maximum allowable transmit power in dBm for TV bands operation. The field can be ignored by the enabling STA if the Network Channel Control frame is sent from a dependent STA to an enabling STA as a control request.

8.5.11 Protected Dual of Public Action frames

8.5.11.1 Protected Dual of Public Action details

Insert the following row (ignoring the header row) in Table 8-143 Protected Dual of Public Action field values in the correct position to preserve ordering by the “Action field value” column.

Table 8-143 – Protected Dual of Public Action field values

| Action field value | Description |
| --- | --- |
| <ANA> | Protected Channel Power Management Announcement |
| <ANA> | Network Channel Control |

Insert the following clause at the end of 8.5.11.7, before 8.5.11.8, as shown:

8.5.11.7af2 Protected Network Channel Control Frame format

The Protected Network Channel Control frame format is the same as the Network Channel Control frame format (see 8.5.8.af2).  It is used instead of the Network Channel Control frame when Management Frame Protection is negotiated.

10. MLME

Insert the following new subclause as 10.12.7, as shown:

10.12.8 Network Channel Control

Some regulations require that  a client STA shall operate on network channels under the control of  a master STA. In some regulatory domains, a master STA operates as an enabling STA. A client STA operates as a dependent STA. Network channel control utilizes a two-message transaction sequence to realize such control. The Network Channel Control frame is used between the master STA and the client STA.  The first message asserts identity and requests network channel control. The client STA may request control on multiple network channels. The second message returns the network channel control result. The master STA may grant permission of multiple network channels to the dependent STA by using the Network Channel Control public action frame (cf. 8.5.8af2 Network Channel Control frame format).

For dependent STAs employing RLQP, the dependent STA may request its preferred operating channels from the list of available channels to its enabling STA by transmitting the Network Channel Control Request frame. An enabling STA that receives the Network Channel Control Request frame shall respond with the Network Channel Control Response frame by which the responder confirms the Network Channel Control request. When the responder transmits the Network Channel Control Response frame, it shall transmit the confirmed channel list and the maximum transmit power levels as well. The channel list the responder confirms may be the same channels listed in the Network Control Channel Request frame or the subsets of that list in the Network Channel Control Request frame.

Whenever the available channel list has been changed by the update of the database information or detection of the primary service signals, the dependent STAs may transmit the Network Channel Control Request frame.

NOTE—The TVB enabler, which is provided by the operator, has the capability to obtain the channel list as well as network channel control, which realizes the control of frequencies of operation for TVBD..

Annex B

(normative)

Protocol Implementation Conformance Statement (PICS) proforma

TGaf editor: Insert the following new rows into B.4.af:

### B.4.af TVWS operation extensions

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| *WS1 | Fixed STA TVWS Operation | 11.11.3, Annex I, Annex J.2.4 | CFaf:O | Yes, No. N/A |
| *WS2 | Master STA TVWS Operation | 11.11.3, 11.11.4, Annex I, Annex J.2.4 | CFaf:O | Yes, No. N/A |
| *WS3 | Client STA TVWS Operation | 11.11.5, Annex I, Annex J.2.4 | CFaf:O | Yes, No. N/A |
| WS3.1 | Dependent STA TVWS Behavior | 11.11.5, Annex I, Annex J.2.4 | WS3:M | Yes, No. N/A |
| WS6 | Network Channel Control | 6.3.2.af2, 8.4.5.3, 8.5.11.7af2, 10.12.8 | CFaf:M | Yes, No. N/A |