IEEE P802.11
Wireless LANs

| MLME Primitives |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-09-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Dorothy Stanley | Aruba Networks | 1332 Crossman Ave Sunnyvale, CA 94089 | +1-630-363-1389 | dstanley@arubanetworks.com |
| Stephen McCann | Research in Motion | 200 Bath Road, Slough, Berkshire, SL1 3XE, UK | +44 1753 667099 | smccann@rim.com |

Abstract

This document contains proposed text defining MLME primitives for the TGae draft.

*

Probe Response and received Beacon Frames:

6.3.3.3 MLME-SCAN.confirm

6.3.3.3.2 Semantics of the service primitive

Insert the following new row in the BSSDescription parameter table:

| Name | Type | Valid range | Description | IBSS Adoption |
| --- | --- | --- | --- | --- |
| MFQ Policy | MFQ Policy element | As defined in 8.4.2.ae1(MFQ Policy element) | The values from the MFQ Policy element. if such an element was present in the Probe Response or Beacon frame, else null. | Adopt |

Change 6.3.6 as follows:

Association Response Frame:

6.3.6 Associate

6.3.7.3 MLME-ASSOCIATE.confirm

6.3.7.3.2 Semantics of the service primitive

Change the primitive parameter list as shown:

MLME-ASSOCIATE.confirm (
ResultCode,
CapabilityInformation,
AssociationID,
SupportedRates,
RCPI.request,
RSNI.request,
RCPI.response,
RSNI.response,
RRMEnabledCapabilities,
EDCAParameterSet,
Content of FT Authentication Elements, 
SupportedOperatingClasses,
HT Capabilities,
Extended Capabilities, 
20/40 BSS Coexistence,
BSSMaxIdlePeriod,
TIMBroadcastResponse,

MFQ Policy,
VendorSpecificInfo)

Insert the following new row before VendorSpecificInfo in the parameter table:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| MFQ Policy | MFQ Policy element | As defined in 8.4.2.ae1(MFQ Policy element) | The values from the MFQ Policy element that the STA indicates. This element is optionally present when the MIB attribute dot11MFQPolicyActivated is set to true, and is not present otherwise |

6.3.7.5 MLME-ASSOCIATE.response

6.3.7.5.2 Semantics of the service primitive

Change the primitive parameter list as shown:

MLME-ASSOCIATE.response	(
PeerSTAAddress,
ResultCode,
CapabilityInformation,
AssociationID,
EDCA Parameter Set,
RCPI,
RSNI,
RM Enabled Capabilities,
Content of FT Authentication Elements,
SupportedOperatingClasses,
DSERegisteredLocation,
HT Capabilities,
Extended Capabilities, 
20/40 BSS Coexistence,
BSSMaxIdlePeriod,
TIMBroadcastResponse,
MFQ Policy,
VendorSpecificInfo)

Insert the following new row before VendorSpecificInfo in the parameter table:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| MFQ Policy | MFQ Policy element | As defined in 8.4.2.ae1(MFQ Policy element) | The values from the MFQ Policy element. if such an element was present in the Association Response, else null. |

ReAssociation Response Frame:

Change  6.3.8.3.2 as follows:

6.3.8.3 MLME-REASSOCIATE.confirm

6.3.8.3.2 Semantics of the service primitive

Change the primitive parameters as shown:

MLME-REASSOCIATE.confirm	(
Resultcode,
CapabilityInformation,
AssoicationID,
SupportedRates,
EDCAParameterSet,
RCPI.request,
RSNI.request,
RCPI.response,
RSNI.response,
Content of FT Authentication Elements,
SupportedOperatingClasses,
HT Capabilities,
Extended Capabilities, 
20/40 BSS Coexistence,
BSSMaxIdlePeriod,
TIMBroadcastResponse,
FMSResponse,
DMSResponse,
QoSMapSet,
MFQ Policy,
VendorSpecificInfo)

Insert the following new row before VendorSpecificInfo in the parameter table:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| MFQ Policy | MFQ Policy element | As defined in 8.4.2.ae1(MFQ Policy element) | Indicates the MFQ Policy parameters of the AP. This parameter is present if the MIB attribute dot11MFQPolicyActivated is set to true, and is not present otherwise. |

6.3.8.4 MLME-REASSOCIATE.indication

6.3.8.4.2 Semantics of the service primitive

Change the primitive parameter list as shown:

MLME-REASSOCIATE.indication	(
PeerSTAAddress,
CurrentAPAddress,
CapabilityInformation,
ListenInterval,
SSID,
SupportedRates,
RSN,
QoSCapability,
RCPI,
RSNI,
RRMEnabledCapabilities,
Content of FT Authentication Information Elements,
SupportedOperatingClasses,
DSERegisteredLocation,
HT Capabilities,
Extended Capabilities, 
20/40 BSS Coexistence,
QoSTrafficCapability,
TIMBroadcastRequest,
FMSRequest,
DMSRequest,
MFQ Policy,
VendorSpecificInfo)

Insert the following new row before VendorSpecificInfo in the parameter table:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| MFQ Policy | MFQ Policy element | As defined in 8.4.2.ae1(MFQ Policy element) | The values from the MFQ Policy element if such an element was present in the Association Response, else null. |

6.3.8.5 MLME-REASSOCIATE.response

6.3.8.5.2 Semantics of the service primitive

Change the primitive parameter list as shown:

MLME-REASSOCIATE.response	(
PeerSTAAddress,
ResultCode,
CapabilityInformation,
AssociationID,
Content of FT Authentication Information Elements,
SupportedRegulatoryClasses,
HT Capabilities,
Extended Capabilities, 
20/40 BSS Coexistence,
BSSMaxIdlePeriod,
TIMBroadcastResponse,
FMSResponse,
DMSResponse,
QoSMapSet,
MFQ Policy,
VendorSpecificInfo)

Insert the following new row before VendorSpecificInfo in the parameter table:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| MFQ Policy | MFQ Policy element | As defined in 8.4.2.ae1(MFQ Policy element) | The values from the MFQ Policy element if such an element was present in the Re-Association Response, else null. |

6.3.11 Start

6.3.11.2 MLME-START.request

6.3.11.2.2 Semantics of the service primitive

Change the primitive parameter list as shown:

MLME-START.request(
			SSID,
			BSSType,

BeaconPeriod,

DTIMPeriod,

CF parameter set,

PHY parameter set,

IBSS parameter set,

ProbeDelay,

CapabilityInformation,

BSSBasicRateSet,

OperationalRateSet,

Country,

IBSS DFS Recovery Interval,

EDCAParameterSet,

DSERegisteredLocation,

HT Capabilities,

HT Operation,

BSSMembershipSelectorSet,

BSSBasicMCSSet,

HTOperationalMCSSet,

Extended Capabilities,

20/40 BSS Coexistence,

Overlapping BSS Scan Parameters,

MultipleBSSID,
InterworkingInfo,

AdvertisementProtocolInfo,

RoamingConsortiumInfo,
Mesh ID,
Mesh Configuration,
MFQ Policy,

VendorSpecificInfo)

Insert the following row before the VendorSpecificInfo row of the untitled table defining the primitive parameters in 6.3.11.2.2 :

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| MFQ Policy | MFQ Policy element | As defined in 8.4.2.ae1(MFQ Policy element) | This element is optionally present when the MIB attribute dot11MFQPolicyActivated is set to true, and is not present otherwise. |

Insert the following new sub-clauses:

6.3.ae1  MFQ Policy

The following MLME primitives support the signaling of MFQ Policy.

6.3.ae1.1 MLME-MFQPOLICYQUERY.request

6.3.ae1.1.1 Function

This primitive supports the signalling of MFQ Policy Queries between peer SMEs. An SME queries the MFQ Policy within a peer SME.

6.3.ae1.1.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-MFQPOLICYQUERY.request (

Peer MAC Address,

Dialog Token,

MFQ Policy parameters

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MAC Address | Any valid individual MAC Address | The address of the peer MAC entity to which the MFQ Policy Query request is sent. |
| Dialog Token | Integer | 1-255 | The dialog token to identify the MFQ Policy Query transaction. |
| MFQ Policy parameters | MFQ Policy element | As defined in 8.4.2.ae1(MFQ Policy element) | This element is optionally present when the MIB attribute dot11MFQPolicyActivated is set to true, and is not present otherwise. |

6.3.ae1.1.3 When generated

This primitive is generated by the SME to request that a MFQ Policy Query Request frame be sent to a peer entity to query for MFQ Policy information.

6.3.ae1.1.4 Effect of receipt

On receipt of this primitive, the MLME constructs a MFQ Policy Query Request frame containing the set of MFQ Policy parameters. This frame is then scheduled for transmission.

6.3.ae1.2 MLME-MFQPOLICYQUERY.confirm

6.3.ae1.2.1 Function

This primitive reports the result of a MFQ Policy Query Request.

6.3.ae1.2.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-MFQPOLICYQUERY.confirm (

Dialog Token,

Result Code,

Peer MAC Address,

MFQ Policy parameters

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Dialog Token | Integer | 1-255 | The dialog token to identify the MFQ Policy Query transaction. |
| Result Code | Enumeration | SUCCESS, INVALID_ PARAMETERS, TIMEOUT, TRANSMISSIO N_FAILURE, UNSPECIFIED_ FAILURE | Reports the outcome of the MFQ Policy Query Request. |
| Peer MAC Address | MAC Address | Any valid individual MAC Address | The address of the peer MAC entity to which the MFQ Policy Query request is sent. |
| MFQ Policy parameters | MFQ Policy element | As defined in 8.4.2.ae1(MFQ Policy element) | This element is optionally present when the MIB attribute dot11MFQPolicyActivated is set to true, and is not present otherwise. |

6.3.ae1.2.3 When generated

This primitive is generated by the MLME when the MFQ Policy Query  completes.

6.3.ae1.2.4 Effect of receipt

On receipt of this primitive, the SME evaluates the result code.

6.3.ae1.3 MLME-MFQPOLICYQUERY.indication

6.3.ae1.3.1 Function

This primitive indicates that a MFQ Policy Query Request frame has been received from a peer entity.

6.3.ae1.3.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-MFQPOLICYQUERY.indication (

Peer MAC Address,

Dialog Token,

MFQ Policy parameters

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MAC Address | Any valid individual MAC Address | The address of the peer MAC from which the MFQ Policy Query request was received |
| Dialog Token | Integer | 1-255 | The dialog token to identify the MFQ Policy Query transaction. |
| MFQ Policy parameters | MFQ Policy element | As defined in 8.4.2.ae1(MFQ Policy element) | This element is optionally present when the MIB attribute dot11MFQPolicyActivated is set to true, and is not present otherwise. |

6.3.ae1.3.3 When generated

This primitive is generated by the MLME when a valid MFQ Policy Query Request frame is received from a peer entity.

6.3.ae1.3.4 Effect of receipt

On receipt of this primitive, the MFQ Policy Query Request frame can be made available for SME processes.

6.3.ae1.4 MLME-MFQPOLICYQUERY.response

6.3.ae1.4.1 Function

This primitive requests the transmission of MFQ Policy Query Response frame to a peer entity, in response to a received MFQ Policy Query Request frame.

6.3.ae1.4.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-MFQPOLICYQUERY.response (

Peer MAC Address,

Dialog Token,

MFQ Policy parameters

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MAC Address | Any valid individual MAC Address | The address of the peer MAC entity to which the MFQ Policy Query request is sent. |
| Dialog Token | Integer | 1-255 | The dialog token to identify the MFQ Policy Query transaction. |
| MFQ Policy parameters | MFQ Policy element | As defined in 8.4.2.ae1(MFQ Policy element) | This element is optionally present when the MIB attribute dot11MFQPolicyActivated is set to true, and is not present otherwise. |

6.3.ae1.4.3 When generated

This primitive is generated by the SME to request that a MFQ Policy Query Response frame be sent to a

peer entity to provide MFQ Policy information.

6.3.ae1.4.4 Effect of receipt

On receipt of this primitive, the MLME constructs a MFQ Policy Query Response frame containing the

set of MFQ Policy elements specified. This frame is then scheduled for transmission.

6.3.ae1.5 MLME-MFQPOLICYCONFIG.request

6.3.ae1.5.1 Function

This primitive supports the signaling of MFQ Policy Configuration between peer SMEs. An SME configures the MFQ Policy within a peer SME.

6.3.ae1.5.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-MFQPOLICYCONFIG.request (

Peer MAC Address,

Dialog Token,

MFQ Policy parameters

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MAC Address | Any valid individual MAC Address | The address of the peer MAC entity to which the MFQ Policy Config request is sent. |
| Dialog Token | Integer | 1-255 | The dialog token to identify the MFQ Policy Config transaction. |
| MFQ Policy parameters | MFQ Policy element | As defined in 8.4.2.ae1(MFQ Policy element) | This element is optionally present when the MIB attribute dot11MFQPolicyActivated is set to true, and is not present otherwise. |

6.3.ae1.5.3 When generated

This primitive is generated by the SME to request that a MFQ Policy Config Request frame be sent to a peer entity to configure MFQ Policy information.

6.3.ae1.5.4 Effect of receipt

On receipt of this primitive, the MLME constructs a MFQ Policy Config Request frame containing the set of MFQ Policy parameters. This frame is then scheduled for transmission.

6.3.ae1.6 MLME-MFQPOLICYCONFIG.confirm

6.3.ae1.6.1 Function

This primitive reports the result of a MFQ Policy Config Request.

6.3.ae1.6.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-MFQPOLICYCONFIG.confirm (

Dialog Token,

Result Code,

Peer MAC Address,

MFQ Policy parameters

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Dialog Token | Integer | 1-255 | The dialog token to identify the MFQ Policy Config transaction. |
| Result Code | Enumeration | SUCCESS, INVALID_ PARAMETERS, TIMEOUT, TRANSMISSIO N_FAILURE, UNSPECIFIED_ FAILURE | Reports the outcome of the MFQ Policy Config Request. |
| Peer MAC Address | MAC Address | Any valid individual MAC Address | The address of the peer MAC entity to which the MFQ Policy Config request is sent. |
| MFQ Policy parameters | MFQ Policy element | As defined in 8.4.2.ae1(MFQ Policy element) | This element is optionally present when the MIB attribute dot11MFQPolicyActivated is set to true, and is not present otherwise. |

6.3.ae1.6.3 When generated

This primitive is generated by the MLME when the MFQ Policy Config completes.

6.3.ae1.6.4 Effect of receipt

On receipt of this primitive, the SME evaluates the result code.

6.3.ae1.7 MLME-MFQPOLICYCONFIG.indication

6.3.ae1.7.1 Function

This primitive indicates that a MFQ Policy Config Request frame has been received from a peer entity.

6.3.ae1.7.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-MFQPOLICYCONFIG.indication (

Peer MAC Address,

Dialog Token,

MFQ Policy parameters

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MAC Address | Any valid individual MAC Address | The address of the peer MAC from which the MFQ Policy Config request was received |
| Dialog Token | Integer | 1-255 | The dialog token to identify the MFQ Policy Config transaction. |
| MFQ Policy parameters | MFQ Policy element | As defined in 8.4.2.ae1(MFQ Policy element) | This element is optionally present when the MIB attribute dot11MFQPolicyActivated is set to true, and is not present otherwise. |

6.3.ae1.7.3 When generated

This primitive is generated by the MLME when a valid MFQ Policy Config Request frame is received from a peer entity.

6.3.ae1.7.4 Effect of receipt

On receipt of this primitive, the MFQ Policy Config Request frame can be made available for SME processes.

6.3.ae1.8 MLME-MFQPOLICYCONFIG.response

6.3.ae1.8.1 Function

This primitive requests the transmission of MFQ Policy Config Response frame to a peer entity, in response to a received MFQ Policy Config Request frame.

6.3.ae1.8.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-MFQPOLICYCONFIG.response (

Peer MAC Address,

Dialog Token,

MFQ Policy parameters

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MAC Address | Any valid individual MAC Address | The address of the peer MAC entity to which the MFQ Policy Config request is sent. |
| Dialog Token | Integer | 1-255 | The dialog token to identify the MFQ Policy Config transaction. |
| MFQ Policy parameters | MFQ Policy element | As defined in 8.4.2.ae1(MFQ Policy element) | This element is optionally present when the MIB attribute dot11MFQPolicyActivated is set to true, and is not present otherwise. |

6.3.ae1.8.3 When generated

This primitive is generated by the SME to request that a MFQ Policy Config Response frame be sent to a

peer entity to convey MFQ Policy information..

6.3.ae1.8.4 Effect of receipt

On receipt of this primitive, the MLME constructs a MFQ Policy Config Response frame containing the

set of MFQ Policy elements specified. This frame is then scheduled for transmission.

6.3.ae1.9 MLME-MFQPOLICYSET.request

6.3.ae1.9.1 Function

This primitive supports the signaling of MFQ Policy from the SME to the MLME.

6.3.ae1.5.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-MFQPOLICYSET.request (

Dialog Token,

MFQ Policy parameters

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Dialog Token | Integer | 1-255 | The dialog token to identify the MFQ Policy Set transaction. |
| MFQ Policy parameters | MFQ Policy element | As defined in 8.4.2.ae1(MFQ Policy element) | This element is optionally present when the MIB attribute dot11MFQPolicyActivated is set to true, and is not present otherwise. |

6.3.ae1.5.3 When generated

This primitive is generated by the SME to request that a MFQ Policy Set Request frame be sent to the MLME to set MFQ Policy information.

6.3.ae1.5.4 Effect of receipt

On receipt of this primitive, the MLME uses the set of MFQ Policy parameters within the MFQ Policy Set Request frame.

6.3.ae1.10 MLME-MFQPOLICYSET.confirm

6.3.ae1.10.1 Function

This primitive reports the result of a MFQ Policy Set Request frame.

6.3.ae1.10.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-MFQPOLICYSET.confirm (

Dialog Token,

Result Code,

MFQ Policy parameters

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Dialog Token | Integer | 1-255 | The dialog token to identify the MFQ Policy Set transaction. |
| Result Code | Enumeration | SUCCESS, INVALID_ PARAMETERS, UNSPECIFIED_ FAILURE | Reports the outcome of the MFQ Policy Set Request. |
| MFQ Policy parameters | MFQ Policy element | As defined in 8.4.2.ae1(MFQ Policy element) | This element is optionally present when the MIB attribute dot11MFQPolicyActivated is set to true, and is not present otherwise. |

6.3.ae1.10.3 When generated

This primitive is generated by the MLME when the MFQ Policy Set Request completes.

6.3.ae1.10.4 Effect of receipt

On receipt of this primitive, the SME evaluates the result code.

10.ae1.1 Management frame QoS policy implementation dependencies

Insert the following paragraphs as the end of the sub-clause:

A non-AP STA may query the MFQ QoS policy of a peer MFQ STA using the MFQ Policy Query Request frame. The peer MFQ STA responds with the MFQ Policy using the MFQ Policy Query Response frame.

The MLME-MFQPOLICYQUERY primitives (See 6.3.ae1.1 to 6.3.ae1.4) are used by an MFQ STA to query MFQ Policy.

On the receipt of an MLME-MFQPOLICYQUERY.indication primitive, a MFQ STA that is using the default MFQ policy shall not include the Management Frame QoS Policy parameters in the MLME-MFQPOLICYQUERY.response primitive.

10.ae1.2 Management frame QoS policy config procedures

Insert the following paragraph as the last paragraph of the sub-clause:

An MFQ STA SME uses the MLME-MFQPOLICYSET primitive to configure the MLME with an MFQ QoS policy (See 6.3.ae1.9).

10.ae1.2.3 Management frame QoS policy reconfiguration request and response

Insert the following paragraph as the second paragraph of the sub-clause:

The SME of peer MFQ STA uses the MLME-MFQPOLICYCONFIG primitives (See 6.3.ae1.5 to 6.3.ae1.9) to exchange the MFQ Policy Config Request and MFQ Policy Config Response frames.

Insert the following MIB variable in the Dot11StationConfigEntry” of the “dotStationConfig TABLE”

dot11MFQPolicyActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

" This is a control variable.

It is written by the SME or external management entity.

This attribute, when true, indicates that the station capability for MFQ Policy is enabled. False indicates the station has no MFQ Policy capability or that the capability is present but disabled. "

DEFVAL {false}

::= { dot11MFQPolicyActivated  <ANA>}

In 8.4.2.29, include the changes indicated below:

#### 8.4.2.29 Extended Capabilities element

Insert two new items in table 8-88, and update the reserved values as appropriate:

(11k)

| Table 8-88—Capabilities field |  |  |
| --- | --- | --- |
| Bit | Information | Notes |
| <ANA> | MFQActivated | This bit is set to 1 if dot11MFQPolicyActivated is set to true. Otherwise, it is set to 0 |
| <ANA> (11n) | MFQReconfiguration | This bit is set to 1 if dot11MFQReconfigurationActivated is set to true. Otherwise, it is set to 0 |

And change instances of “MFQImplemented” to “MFQActivated” throughout the draft, excluding MIB variable names.

References:

- IEEE P802.11v D14.0

- IEEE P802.11REVmb D5.0

- IEEE P802.11s D7.0

- IEEE P802.11u D12.0