IEEE P802.11
Wireless LANs

| Network channel control with spectrum mask |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2011-09-19 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Chen Sun | NICT | 3-4, Hikarino-oka, Yokosuka, Kanagawa, Japan, 239-0847 | +81 45 847 5062 | sun@nict.go.jp |
| Zhou Lan | NICT | 3-4, Hikarino-oka, Yokosuka, Kanagawa, Japan, 239-0847 |  | lan@nict.go.jp |
| Yohannes Alemseged | NICT | 3-4, Hikarino-oka, Yokosuka, Kanagawa, Japan, 239-0847 |  | yohannes@nict.go.jp |
| Hiroshi Harada | NICT | 3-4, Hikarino-oka, Yokosuka, Kanagawa, Japan, 239-0847 |  | harada@nict.go.jp |

Abstract

This document addresses the comments received in the letter ballot based on D1.0. The resolutions to the comments are provided by giving the editor's instruction based on D1.03.

| CID | Page | Clause | Comment | Proposed Change |
| --- | --- | --- | --- | --- |
| 1183 | 39.01 | 8.4.5.3 | The database can generate a tailored list of available channels based on the spectrum mask information for given geolocation. | Add field in the channel availability query frame to allow mode II devices to give the spectrum mask information of both Mode II devices and client devices to the database as mentioned in the OFCOM document. |
| 1184 | 40.44 | 8.4.5.4 | If the mode II devices know the spectrum mask of the client STAs, the mode II devices can set the transmit power and the number of channels for client STAs according to their out-of-band emission characteristics for each WLAN channel as in the OFCOM approach. | Add field in the network channel control frame to allow client devices to send spectrum mask information to the mode II devices as in document doc.:IEEE 802.11-10/1232. |

- Discussion

The ECC Report 159 Section 9.3.3 Technical information to be communicated to the geo-location database stated "Providing information about the type of device, such as the device class will allow information to be returned according to device capabilities and interference characteristics.  The database could then take into account its known transmission parameters in returning appropriate frequencies and allowed maximum transmission power.  Different classes of devices, with different technical characteristics, can exhibit different interference characteristics (e.g. antenna type, antenna height, type of technology and modulation) allowing different EIRP limits. For example, devices classes which would have good out-of-band emission characteristics might be able to transmit with higher power levels on some frequencies and/or locations. Defining device classes and their characteristics is a topic for standardization."

This allows the database (or its proxy if allowed by regulation) to generate a tailored list of available channels based on the device's spectrum mask. Thus, the spectrum mask information shall be provided to the database. Comment 1183 addresses this issue.

In OFCOM "Implementing Geolocation--Summary of consultation responses and next steps" published in Sep. 1, 2011, it says

"3.31 Compliance with the R&TTE Directive and related issues. One respondent commented on our approach towards the emissions mask where there are no “hard” values set in the specification. Instead, manufacturers are required to supply, or otherwise refer to, typical masks for each device type which are then taken into account by the database algorithms. The respondent felt this would not work because manufacturers cannot be obliged to provide such a mask and there is no way to check that what they provide is accurate.

3.32 We have considered this point and agree that we cannot oblige manufacturers to provide a mask. However, we do not believe that such action will be necessary. It is our belief that manufacturers will be incentivised to provide accurate device masks on a voluntary basis as the most efficient way to ensure that their devices have access white space spectrum."

To address this OFCOM approach and to make the 11af standard suitable also for European market, we propose to allow the devices to give their spectrum mask information as defined in the TLV format called Spectrum Mask Descriptor. Note that, although the FCC regulation does not have such operation, the concept of calculating the constrained maximum transmit power can be extended to the RLSS or GDC enabling STA to allow them to optimize the spectrum usage based on the devices' emission mask. The comment CIDs 1183 and 1184 ask for adding the spectrum mask descriptor to the NCC and CAQ frame. We believe that adding the spectrum mask descriptor to NCC is sufficient at this stage. This allows us to use CAQ to fulfill the FCC regulation and use NCC with spectrum mask in addition to CAQ to fulfill UK as well as European regulations.

- Proposed Revised for CIDs1183 & 1184 as per the discussion and editing instructions in 11-11/1232r6.

8.2.6 TLV Values

- TGaf Editor: Insert the following paragraph in 8.2.6

- 6.2.6.1.6 Spectrum Mask Descriptor

This parameter describes the spectrum masks used for devices. The Spectrum Mask Descriptor is shown in Table 8-14m (Spectrum Mask Descriptor definition) and Table 8-14n (Spectrum Mask Descriptor value fields).

Table 8-14m – Spectrum Mask Descriptor definition

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| <ANA> | variable | Compound TLVs in Table 8-14n (Spectrum Mask Descriptor value fields) | UK |

Table 8-14n – Spectrum Mask Descriptor value fields

| Name | Subtype (1 octet) | Length (octets) | Value | Scope |
| --- | --- | --- | --- | --- |
| Bandwidth | 1 | 1 | Bandwidth gives the bandwidth of the spectrum mask. The field is coded as an unsigned integer in units of 0.5 MHz. | UK |
| Maximum Total Transmit Power | 2 | 1 | Maximum Total Transmit Power field gives the maximum total transmit power. The field is coded as a signed integer in units of dBm. | UK |
| Spectrum Mask | 3 | 5 | The Spectrum Mask field describes the transmit spectrum of a device by giving five unsigned integers (each byte for each integer) representing power spectral density of the emissions, measured in dB below the maximum output power, of the transmitter for any frequency removed from the center frequency between 45-50%, between 50-55%, between 55-100%, between 100-150%, and above 150% of the channel bandwidth, respectively. | UK |

8.4.5.4 RLQP Network Channel Control element

- TGaf Editor: Change Figure 8-430l as follows

|  |  |  |  |  |  | - These three fields are repeated, as determined by the Number of Network Channel Control Triplets field |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - RLQP ID | - Length | - Requester STA Address | - Responder STA Address | Reason Result - Code | Number - Of NetworkChannel Control Triplets | - Operating Class | - Channel Number | - Maximum Transmit Power | - Spectrum Mask Descriptor |
| Octets: - 1 | - 2 | - 6 | - 6 | - 1 | - 1 | - 1 | - 1 | - 1 | - variable |

- Figure 8-430l—Network Channel Control frame body format

- TGaf Editor: Insert the following paragraph as the last paragraph of 8.4.5.4 as follows.

The Spectrum Master descriptor field is given in 6.2.6.1.6 (Spectrum Mask Descriptor).

8.5.8.30 Network Channel Control frame format

- TGaf Editor: Change Figure 8-459i as follows

|  |  |  |  |  |  |  | These three fields are repeated, as determined by the Number of Network Channel Control Triplets field |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Category | Public Action | Length | Requester STA address | Responder STA Address | Reason Result Code | Number Of NetworkChannelTriplets | Operating Class | Channel Number | Maximum Transmit Power | Spectrum Mask Descriptor |
| 1 | 1 | 2 | 6 | 6 | 1 | 1 | 1 | 1 | 1 | variable |

- Figure 8-459i—Network Channel Control frame Action field format

- TGaf Editor: Insert the following paragraph as the last paragraph of 8.5.8.30 as follows.

The Spectrum Master Descriptor field is given in 6.2.6.1.6 (Spectrum Mask Descriptor).

10.12.7 Network channel control (NCC)

- TGaf Editor: Change the first two paragraphs of 10.12.7 as follows.

Network channel control utilizes a two-message transaction sequence to allow a NCC responding STA to control the frequency usage in TV bands of a NCC requesting STA's WLAN network channels. The first message sent by the NCC requesting STA asserts identity and requests NCC. The NCC requesting STA may select its preferred frequencies from the WSM and request usage of the selected frequencies for multiple WLAN network channels usage by providing spectrum mask information (6.2.6.1.6 (Spectrum Mask Descriptor)) to the NCC responding STA. The second message sent by the NCC responding STA returns the NCC result. The NCC responding STA may grant permission of using the selected frequencies for multiple WLAN network channels to the NCC requesting STA by using the NCC response frame (cf. 8.5.8.30 (Network Channel Control frame format)). When responding, the NCC responding STA shall provide the confirmed WLAN network channels and the transmit power constraints as well. The WLAN network channels that the network channel control responding STA confirms may be the same network channels listed in the NCC Request frame or the subsets of that list in the NCC Request frame.

A NCC requesting STA employing RLQP may send a NCC frame to the RLSS to request its preferred frequencies given by the WSM for WLAN network channels. The NCC requesting STA accomplishes this by transmitting a RLQP element with the RLQP ID for NCC in the Query Request field in a Gas Initial Request frame. The RLSS uses the spectrum mask information whenever available to decide spectrum usage. The NCC responding STA after receiving this frame forwards the NCC request to the RLSS. The RLSS after receiving the NCC request shall respond and send a NCC response via the NCC responding STA to the NCC requesting STA as a NCC response frame. When responding, the RLSS shall provide the confirmed WLAN network channels and the transmit power constraints as well. The WLAN network channels that the RLSS confirms may be the same network channels listed in the NCC Request frame or the subsets of that list in the NCC Request frame.