IEEE P802.11
Wireless LANs

| Proposed Edits to Clause aa.4.1 – CID 254 LB164 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-11-9 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Graham Smith | DSP Group | 2941 Sunrise Blvd, Suite 100 Rancho Cordova, CA 96742 | +1 916 851 9191 x209 | Graham.Smith@dspg.com |

Abstract

This document contains changes to the normative text of aa.4.1 as a response to CID 254 of LB164, based upon D1.02.

Discussion on aa.4.1

Two sharing schemes are suggested:  Proportional Sharing and On Demand Sharing.

In each sharing scheme, the purpose is to keep the total allocated traffic to a value such that over-allocation does not occur.  The absolute maximum allocation is 1second per second (100%).  In order to provide for some protection to non-QoS traffic, then it is suggested that the actual allocation should be less than 100%, e.g. 90%.  In the following descriptions of the two suggested sharing schemes, this value is referred to as "MAV" (Maximum Allocation Value).  It is suggested that in order to provide some protection to non-QoS traffic, each AP should select a value for MAV between 0.9, minimum and 0.95 maximum.  There is no requirement that each overlapping AP must select the same MAV.

EXAMPLES:

Proportional Sharing

AP 1 	Potential Traffic = 0.4	MAV = 0.9

AP 2 	Potential Traffic = 0.8	MAV = 0.95

Access Factor = 0.8 + 0.4 = 1.2

AP1 Share = 0.4/1.2 x 0.9 = 0.3

AP2 Share = 0.8/1.2 x 0.95  = 0.63

Peak Traffic = 0.93 i.e. at least 7% still available (at very worse case,TSPECs always pessimistic)

Worse case would be that the traffic in AP2 is >> traffic in AP1, then always we have at least 5% free.

If we force AP2 to MAV = 0.9

Then

AP1 Share = 0.4/1.2 x 0.9 = 0.3

AP2 Share = 0.8/1.2 x 0.9 = 0.6

Peak Traffic = 0.9	How important is this, not much and alternative is to specify a complicated procedure to reduce MAV to the lowest value.

On Demand Sharing

Same settings for AP1 and AP2

Let AP1 set up traffic 0.4

AP2 can add traffic such that total traffic is 0.95, i.e. 0.55

Total traffic is 0.95

i.e. worse case is always the traffic capacity equal to the largest MAV, which is 0.95.

TSPECs are ALWAYS pessimistic, and there will ALWYS be surplus capacity anyway.

The more I think about it, I am tempted to keep the original text with a MAV of 1.

MOTION #1

Retain original text and respond to CID 254 as follows:

"This proposal actually improves the non-QOS data situation by providing the means to prevent the over allocation of QoS traffic and squeeze out non-QoS traffic completely.  In no way does this proposal make it worse for non-QoS traffic.  In addition TSPECs are necessarily pessimistic and in practice there is always room for non-QoS traffic.  The allocation policy for EDCA Admission Control and HCCA is considered out of scope and there is no present limit imposed.  In these OBSS Sharing schemes, there is a limit imposed that in practice will always provide bandwidth for non-QoS traffic.

MOTION #2 (If Motion 1 fails)

Accept text in 10/1324r1 for inclusion in Draft 2.0?

Alternative Text

aa.4.1 Sharing Schemes

Two sharing schemes are suggested(#390):  Proportional Sharing and On Demand Sharing.

In each sharing scheme, the purpose is to keep the total allocated traffic to a value such that over-allocation does not occur.  The absolute maximum allocation is 1second per second (100%).  In order to provide for some protection to non-QoS traffic, then it is suggested that the actual allocation should be less than 100%, e.g. 90%.  In the following descriptions of the two suggested sharing schemes, this value is referred to as "MAV" (Maximum Allocation Value).  It is suggested that in order to provide some protection to non-QoS traffic, each AP should select a value for MAV between 0.9, minimum and 0.95 maximum.  There is no requirement that each overlapping AP must select the same MAV.

The (#390)Proportional Sharing scheme is as follows:

- The AP examines the Access Factor in the QLoad Reports from each overlapping BSS, including its own QLoad, and determines the maximum.

- If the maximum Access Factor is less than or equal to MAV, the AP may allocate up to its advertised Potential Traffic Self(#283)(#284) traffic.

- If the maximum Access Factor is greater than MAV, then the AP may only allocate up to a value of its Potential Traffic Self(#283)(#284) divided by the maximum Access Factor, multiplied by MAV.

In the proportional sharing scheme, before an AP allocates a new Medium Time or schedules a new TXOP in response to an ADDTS Request, it shall check that this addition will not exceed its sharing limit, as follows:

- If the Access Factor is less than or greater than MAV, then the AP may allocate up to its advertised Potential Traffic Self(#283)(#284) (composite stream calculated as MAX traffic = µtot + 2 σtot )

- If the Access Factor is greater than MAV, the AP carries out the following:

- Calculate the peak traffic value of the Potential Traffic Self(#283)(#284), using:

Peak = MEAN + 2 STDEV

- Divide this value by the maximum Access Factor.  This is termed the maximum allowable Potential Traffic Self(#283)(#284) traffic

- Calculate the resulting value of the Allocated Traffic Self  if the new TSPEC is accepted, as explained in Error: Reference source not found (Ed), and then calculate the resulting peak value using

Peak = MEAN + 2 STDEV

- If the resulting peak value, calculated in step 3 is greater than the maximum allowable Potential Traffic Self(#283)(#284) traffic, then the TS(#61) Request shall be rejected.

- If the resulting peak value, calculated in step 3 is less than the maximum allowable Potential Traffic Self(#283)(#284) traffic, and the TS(#62) Request is for EDCA Admission, then it shall be accepted.

- If the resulting peak value, calculated in step 3 is less than the maximum allowable Potential Traffic Self(#283)(#284) traffic and the new allocation is for an HCCA TS(#63), the AP must further check the HCCA Access Factor

- If the HCCA Access Factor is less than or equal to MAV, then the AP may allocate up to its advertised HCCA Peak

- If the HCCA Access Factor is greater than MAV, an AP may only allocate the new stream if the resulting HCCA Peak is less than or equal to the value of the HCCA Peak divided by the HCCA Access Factor multiplied by MAV

- The AP must then check that it is possible to schedule TXOPs using the HCCA TXOP Advertisement as described in Error: Reference source not found (Ed)

If the new stream is allocated, then the AP updates the appropriate fields in its QLoad element (#574).

The (#390)On-Demand sharing scheme, is as follows:

- Before allocating a new stream, the AP examines the Allocated Traffic Shared values in the QLoad Reports from each overlapping BSS, including its own QLoad, and selects the maximum Allocated Traffic Shared value which has the highest peak value, using:

Peak = MEAN + 2 STDEV.
The AP also notes the number of AC_VI and AC_VO streams in this maximum Allocated Traffic Shared Field.

- The AP adds the requested new stream (new) to the selected  maximum Allocated Traffic Shared value (max) determined in step 1, using:

MEAN = MEANnew + MEANmax

STDEV = sqrt(STDEV2new + STDEV2max)

- The AP then calculates the peak value for the new composite stream calculated in step 2 , using:

Peak = MEAN + 2 STDEV

- Using the values of the AC_VI and AC_VO streams noted in step 1, plus the stream (#604) represented by the new stream, the AP determines the new EDCA BW Factor

- Multiply the peak value calculated in step 3 by the EDCA BW Factor, determined in step 4.  This is the new Peak Traffic requirement

- If this Peak Traffic requirement value calculated in step 5 is greater than MAV, then the AP must refuse to allocate the new stream

- If the peak value calculated in step 5 is less than or equal to MAV, and the new allocation is for an EDCA Admission ADDTS, then the AP may allocate that new traffic

- If the peak value calculated in step 4 is less than or equal to 1, and the new allocation is for an HCCA ADDTS, the AP must further check the HCCA Access Factor

- If the HCCA Access Factor is less than or equal to MAV, then the AP may allocate up to its advertised HCCA Peak

- If the HCCA Access Factor is greater than MAV, an AP may only allocate the new stream if the resulting HCCA Peak is less than or equal to the value of the HCCA Peak divided by the HCCA Access Factor, multiplied by MAV

- The AP must also check that it is possible to schedule TXOPs using the HCCA TXOP Advertisement as described in Error: Reference source not found (Ed)

If the new stream is allocated, then the AP shall update the appropriate fields in its QLoad element (#574).

The Allocated Traffic Self field in the QLoad element of a particular AP can be used by other overlapping APs as an indication of whether an AP has either over allocated or is using an on demand sharing scheme.

If, using either the proportional or on-demand sharing schemes, an AP determines that the acceptance of a TSPEC would exceed the available allocation, or a non-AP STA has a TSPEC refused, the non-AP STA could always request a new TSPEC that represents a lower Medium Time or TXOP.  This may be used, for example, where the non-AP STA or AP has the ability to send a more compressed stream.