IEEE P802.11
Wireless LANs

| Link metric comment resolution text |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-12-8 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Kazuyuki Sakoda | Sony Corporation | 5-1-12 Kita-Shinagawa, Shinagawa-ku, Tokyo, 141-0001 Japan | +81-3-5448-4018 | KazuyukiA.Sakoda(at)jp.sony.com |
| Michael Bahr | Siemens AG, Corporate Technology | Otto-Hahn-Ring 6, 80200 München |  | bahr et siemens dod com |

Abstract

This document provides suggested text changes in accordance with the resolutions to CID 82 and CID 237 from the initial sponsor ballot.

## Summary of the intention of this document

- Clarify the link metric request/report functions and relative MLME-SAP interface

## Suggested changes to the draft spec

Apply the following changes.
Corresponding changes to D7.03 are indicated in the following text with “Track Changes” on, to clarify the direction to the editor. Please update the part indicated by the “Track Changes” only.

- Management frame body components

- Information elements

- Mesh Link Metric Report element

The Mesh Link Metric Report element is transmitted by a mesh STA to a neighbor peer mesh STA to indicate the quality of the link between the transmitting mesh STA and the neighbor peer mesh STA.

The contents of the element are shown in Mesh Link Metric Report element.

| Element ID | Length | Flags | Link Metric |  |
| --- | --- | --- | --- | --- |
| Octets: 1 | 1 | 1 | variable |  |
| - Mesh Link Metric Report element |  |  |  |  |

The Element ID is set to the value given in Error: Reference source not found for this element.

The Length field indicates the number of octets in the Information field (fields following the Element ID and Length fields).

The format of the Flags field is shown in Figure xxx (Flags field format).

| B0 | B1 B7 |
| --- | --- |
| Request | Reserved |
| Bits: 1 | 7 |
| Figure 11s-00—Flags field format |  |

The Flags field is set as follows:

- Bit 0: Request subfield (0 = not a request, 1 = link metric report request). A Request subfield equal to 1 indicates that the recipient of Mesh Link Metric Report element is requested to send a link metric report to the transmitter of the Mesh Link Metric Report element.

- Bit 1–7: Reserved.

The Link Metric field indicates the value of the link metric associated with the mesh link between the peer mesh STA transmitting the Mesh Link Metric Report andthe neighbor mesh STA receiving the Mesh Link Metric Report.

The length and the data type of the Link Metric field are determined by the active path selection metric identifier (see Error: Reference source not found).The length and the data type for the Airtime Link Metric are given in Table 11C-4 in. 11C.8 (Airtime Link Metric).

- Mesh Action frame details

- Mesh Action fields

Several Action frame formats are defined for mesh BSS operation. A Mesh Action field, in the octet field immediately after the Category field, differentiates the formats. The Mesh Action field values associated with each frame format are defined in Mesh Action field valuesinput.doc.

| - Mesh Action field valuesinput.doc |  |
| --- | --- |
| Mesh Action field value | Description |
| 0 | Mesh Link Metric Report |
| 1–63 | Reserved |
| 64 | HWMP Mesh Path Selection |
| 65–127 | Reserved |
| 128 | Gate Announcement |
| 129–191 | Reserved |
| 192 | Congestion Control Notification |
| 193 | MCCA Setup Request |
| 194 | MCCA Setup Reply |
| 195 | MCCAOP Advertisement Request |
| 196 | MCCAOP Advertisements |
| 197 | MCCA Teardown |
| 198 | TBTT Adjustment Request |
| 199 | TBTT Adjustment Response |
| 200–255 | Reserved |

- Mesh Link Metric Report frame format

The Mesh Link Metric Report frame is transmitted by a mesh STA to a neighbor peer mesh STA to report metric information on the link between the two mesh STAs. It is also transmitted by a mesh STA to a neighbor peer mesh STA to request metric information on the link between the two mesh STAs from the recipient. This frame is transmitted using an individually addressed frame. The format of the Mesh Link Metric Report frame Action field is shown in Error: Reference source not found.

| - Mesh Link Metric Report frame Action field formatinput.doc |  |  |
| --- | --- | --- |
| Order | Information | Notes |
|  | Category |  |
|  | Mesh Action |  |
|  | Mesh Link Metric Report element |  |

The Category field is set to the value in Error: Reference source not found for category Mesh Action.

The Mesh Action field is set to the value in Mesh Action field valuesinput.doc representing Mesh Link Metric Report.

The Mesh Link Metric Report field is set as described in 7.3.2.98 (Mesh Link Metric Report element).

- Layer management

- MLME SAP interface

- Mesh link metric

- Introduction

This set of primitives supports the measurement of link metric and the signaling of link metric reporting.

- MLME-MeshLinkMetricRead.request

- Function

This primitive requests to read a measured link metric value between the local MAC entity and a specific peer MAC entity.

- Semantics of the service primitive

The primitive parameters are as follows:

MLME-MeshLinkMetricRead.request(

peerMAC,

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| peerMAC | MAC Address | Any valid individual MAC address | Specifies the address of the peer MAC entity for which the link metric value is measured |

- When generated

This primitive is generated by the SME to read the measured link metric value for the specified peer MAC entity.

- Effect of receipt

On receipt of this primitive, the MLME reports the measured link metric value. The MLME subsequently issues an MLME- MeshLinkMetricMeasure.confirm that reflects the results of this request.

- MLME-MeshLinkMetricRead.confirm

- Function

This primitive reports the results of a link metric read request.

- Semantics of the service primitive

The primitive parameters are as follows:

MLME- MeshLinkMetricRead.confirm(

ResultCode,

ForwardLinkMetricValue,

ReverseLinkMetricValue

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, INVALID_FWLINK_METRIC, INVALID_RVLINK_METRIC, INVALID_PARAMETERS, or NOT_SUPPORTED | Indicates the result of the link metric measurement request. SUCCESS indicates that both forward and reverse link metrics are available. INVALID_FWLINK_METRIC indicates that only reverse link metric has a valid value. INVALID_RVLINK_METRIC indicates that only forward link metric has a valid value. INVALID_RVLINK_METRIC, Either INVALID_PARAMETERS or NOT_SUPPORTED indicates that the request was not processed properly. |
| ForwardLinkMetricValue | Link Meteric | As defined in Error: Reference source not found | A metric value reported from the specified peer MAC entity previously. |
| ReverseLinkMetricValue | Link Metric | As defined in Error: Reference source not found | A metric value computed for the corresponding link. |

- When generated

This primitive is generated by the MLME as a result of an MLME-MeshLinkMetricRead.request primitive to request a measured link metric value.

- Effect of receipt

The SME is notified of the results of the link metric read request.

- MLME-MeshLinkMetricReport.request

- Function

This primitive requests to submit a link metric report  to the specified peer mesh STA.

- Semantics of the service primitive

The primitive parameters are as follows:

MLME-MeshLinkMetricReport.request(

peerMAC,

ReverseLinkMetricRequestFlag,

MeshLinkMetricReport,

VendorSpecificInfo

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| peerMAC | MAC Address | Any valid individual MAC address | Specifies the address of the peer MAC entity to which the Mesh Link Metric Report is sent. |
| ReverseLinkMetricRequestFlag | Enumeration | REPORT_ONLY, or REPORT_AND_REQUEST | Indicates whether the mesh STA requests a link metric report from the peer MAC entity. |
| MeshLinkMetricReport | Mesh Link Metric Report element | As defined in Error: Reference source not found | A metric value computed for the corresponding link. |
| VendorSpecificInfo | A set of elements | As defined in 7.3.2.26 | Zero or more elements. |

- When generated

This primitive is generated by the SME to request that a Mesh Link Metric Report frame to be sent to a peer entity to report a link metric value and to request a Mesh Link Metric Report from the peerMAC entity if ReverseLinkMetricRequestFlag is equal to REPORT_AND_REQUEST.

- Effect of receipt

On receipt of this primitive, the MLME constructs a Mesh Link Metric Report frame.The Request subfield in the Flags field of the Mesh Link Metric Report element is set depending on the parameter given by the ReverseLinkMetricRequestFlag. If ReverseLinkMetricRequestFlag is equal to REPORT_ONLY, the Request subfield is set to 0. If ReverseLinkMetricRequestFlag is equal to REPORT_AND_REQUEST, the Request subfield is set to 1. This frame is then scheduled for transmission. The MLME subsequently issues an MLME-MeshLinkMetricReport.confirm that reflects the results of this request.

- MLME-MeshLinkMetricReport.confirm

- Function

This primitive reports the results of a link metric report request.

- Semantics of the service primitive

The primitive parameters are as follows:

MLME- MeshLinkMetricReport.confirm(

ResultCode,

peerMAC

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, INVALID_PARAMETERS , TRANSMISSION_FAILURE,or UNSPECIFIED_FAILURE | Reports the outcome of a request to submit a link metric report. |
| peerMAC | MAC Address | Any valid individual MAC address | The address of the peer MAC entity to which the Mesh Link Metric Report is sent. |
|  | Mesh Link Metric Report element | As defined in Error: Reference source not found | A metric value reported from the specified peer MAC entity. |
| VendorSpecificInfo | A set of elements | As defined in 7.3.2.26 | Zero or more elements. |

- When generated

This primitive is generated by the MLME as a result of an MLME-MeshLinkMetricReport.request primitive to indicate the result of that request.

- Effect of receipt

The SME is notified of the results of the MLME_MeshLinkMetricReport.request.

- MLME-MeshLinkMetricReport.indication

- Function

This primitive indicates that a Mesh Link Metric Report frame has been received from a peer entity. This Mesh Link Metric Request Report can be in response to an earlier MLME-MeshLinkMetricReport.request primitive with ReverseLinkMetricRequestFlag equal to REPORT_AND_REQUEST.

- Semantics of the service primitive

The primitive parameters are as follows:

MLME- MeshLinkMetric.indication(

peerMAC,

ReverseLinkMetricRequestFlag,

MeshLinkMetricReport,

VendorSpecificInfo

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| peerMAC | MAC Address | Any valid individual MAC address | Specifies the address of the peer MAC entity from which the Mesh Link Metric Report frame was received. |
| ReverseLinkMetricRequestFlag | Enumeration | REPORT_ONLY, or REPORT_AND_REQUEST | Indicates if the peer MAC entity requests a link metric report. |
| MeshLinkMetricReport | Mesh Link Metric Report element | As defined in Error: Reference source not found | A metric value reported from the specified peer MAC entity. |
| VendorSpecificInfo | A set of elements | As defined in 7.3.2.26 | Zero or more elements. |

- When generated

This primitive is generated by the MLME as a result of the receipt of a Mesh Link Metric Report frame from a specific peer MAC entity.

- Effect of receipt

The SME is notified of the receipt of the link metric report from the specified peer MAC entity. When ReverseLinkMetricRequestFlag is equal to REPORT_AND_REQUEST, the peer MAC entity requests a Mesh Link Metric Report from this mesh STA.

|  |  | Error: Reference source not found |  |
| --- | --- | --- | --- |

- Mesh path selection and metric framework

- General

The term “mesh path selection” is used to describe selection of multi-hop paths between mesh STAs at the link layer. Mesh path selection creates forwarding information that is utilized for MSDU/MMPDU forwarding as described in 9.22 (Mesh forwarding framework).

- Extensible path selection framework

This standard allows for alternative and flexible implementations of path selection protocols and metrics.

A mesh STA may include multiple protocol implementations (that is, the default protocol, vendor-specific protocols, etc.) as well as multiple metric implementations, but only one path selection protocol and only one path selection metric shall be used by a mesh STA at a time.

As described in Error: Reference source not found and Error: Reference source not found, mesh STAs use the Mesh Configuration element (7.3.2.96 (Mesh Configuration element)) to announce the active path selection protocol and active path selection metric of the MBSS. This allows a neighbor mesh STA to identify if it should become a member of the MBSS and how it should establish mesh peerings with its members. This standard does not force an existing MBSS that is using a protocol other than the default protocol to switch to the default protocol when a new mesh STA requests mesh peering establishment. While it is possible, in principle, to implement such behavior, an algorithm to coordinate such reconfiguration is beyond the scope of this standard.

(CID230)Path selection protocol and path selection metric are identified by a unique identifier as defined in 7.3.2.96.2 (Active Path Selection Protocol Identifier) and 7.3.2.96.3 (Active Path Selection Metric Identifier), respectively. Also, each path selection protocol and each path selection metric specifies the following:

- Data type of metric values

- Length of the metric field

- Operator for aggregation of link metrics to a path metric; the symbol  is used to identify an arbitrary operator for aggregation

- Comparison operator for determining a better or worse path; how this is performed depends on the actual comparison operator

- Initial value of the path metric (path selection metric only)

The standard defines a default mandatory path selection protocol (HWMP, Error: Reference source not found) and a default mandatory path selection metric (Airtime Link Metric, Airtime Link Metric). Both shall be implemented on all mesh STAs to ensure interoperability.

- Link metric reporting

A mesh STA may submit a link metric report to or request a link metric report from its neighbor peer mesh STA by transmitting a Mesh Link Metric Report frame. A mesh STA receiving a Mesh Link Metric Report element with the Request subfield of the Flags field equal to 1 shall reply with a Mesh Link Metric Report frame containing the link metric value for the corresponding link.

Upon reception of a Mesh Link Metric Report frame, the mesh STA may update its local link metric information using the link metric information received.

- Airtime Link Metric

This subclause defines a default link metric that may be used by a path selection protocol to identify an efficient radio-aware path. The extensibility framework allows this metric to be overridden by any path selection metric as specified in the mesh profile.

Airtime reflects the amount of channel resources consumed by transmitting the frame over a particular link. This measure is approximate and designed for ease of implementation and interoperability.

The airtime for each link is calculated as:

Where O and Bt are constants listed in Airtime cost constants, and the input parameters r and ef are the data rate in Mb/s and the frame error rate for the test frame size Bt respectively. The rate r represents the data rate at which the mesh STA would transmit a frame of standard size Bt based on current conditions and its estimation is dependent on local implementation of rate adaptation. The frame error rate ef is the probability that when a frame of standard size Bt is transmitted at the current transmission bit rate r, the frame is corrupted due to transmission error; its estimation is a local implementation choice. Frame failures due to exceeding Mesh TTL should not be included in this estimate as they are not correlated with link performance.

The Airtime Link Metric shall be encoded as an unsigned integer in units of 0.01 TU.

| - Airtime cost constants |  |  |
| --- | --- | --- |
| Parameter | Recommended value | Description |
| O | Varies depending on PHY | Channel access overhead, which includes frame headers, training sequences, access protocol frames, etc. |
| Bt | 8192 | Number of bits in test frame |

Parameters of the Airtime Link Metric for extensible path selection framework gives the parameters of the Airtime Link Metric for the extensible path selection framework.

| - Parameters of the Airtime Link Metric for extensible path selection framework |  |
| --- | --- |
| Path Selection Metric ID | See Table s7-43bs (Path selection metric identifier values) in 7.3.2.96.3 (Active Path Selection Metric Identifier) |
| Data type | Unsigned integer, 0 metric value< 4 294 967 296 |
| Length of metric field | 4 octets |
| Operator for metric aggregation | addition (+) |
| Comparison operator | less than, equal to, greater than as used with integers - metric a is better than metric b iff a < b - metric a is equal to metric b iff a = b - metric a is worse than metric b iff a > b |
| Initial value of path metric | 0 |

An example of the Airtime Link Metric is shown in Annex Error: Reference source not found.

- Mesh protocol capabilities

- General mesh support

| Item | Protocol capability | Reference | Status | Support |
| --- | --- | --- | --- | --- |
| *MP1 | Support of mesh capability | 5.2.13 (Mesh BSS: IEEE 802.11 wireless mesh network) | PC39:M | Yes No N/A  |
| MP1.1 | Announcement of mesh profile and supplemental information for the mesh discovery | Error: Reference source not found, Error: Reference source not found | MP1:M | Yes No N/A  |
| MP1.2 | Candidate peer mesh STA determination | Error: Reference source not found | MP1:M | Yes No N/A  |
| MP1.3 | Becoming a member of a mesh BSS | Error: Reference source not found | MP1:M | Yes No N/A  |
| *MP1.4 | Mesh Peering Management | Error: Reference source not found | MP1:M | Yes No N/A  |
| MP1.4.1 | Processing of Mesh Peering Open frame | Error: Reference source not found | MP1.4:M | Yes No N/A  |
| MP1.4.2 | Processing of Mesh Peering Confirm frame | Error: Reference source not found | MP1.4:M | Yes No N/A  |
| MP1.4.3 | Processing of Mesh Peering Close frame | Error: Reference source not found | MP1.4:M | Yes No N/A  |
| MP1.4.4 | MPM finite state machine | Error: Reference source not found | MP1.4:M | Yes No N/A  |
| *MP1.5 | Authenticated Mesh Peering Exchange | Error: Reference source not found | MP1:O | Yes No N/A  |
| MP1.5.1 | Protected Mesh Peering Management frame processing | Error: Reference source not found | MP1.5:M | Yes No N/A  |
| MP1.5.2 | AMPE finite state machine | Error: Reference source not found | MP1.5:M | Yes No N/A  |
| MP1.5.3 | MGTK distribution | Error: Reference source not found | MP1.5:M | Yes No N/A  |
| *MP2 | Mesh Control field | 7.1.3.6.3 (Mesh Control field) | MP1:M | Yes No N/A  |
| MP2.1 | Address Extension recognition | 7.1.3.6.3 (Mesh Control field) | MP2:M | Yes No N/A  |
| MP2.2 | Decrement of Mesh frame TTL | 7.1.3.6.3 (Mesh Control field) | MP2:M | Yes No N/A  |
| MP3 | Mesh STA beaconing | Error: Reference source not found | MP1:M | Yes No N/A  |
| *MP4 | Mesh STA synchronization | Error: Reference source not found | MP1:M | Yes No N/A  |
| MP4.1 | Neighbor Offset Protocol | Error: Reference source not found | MP4:M | Yes No N/A  |
| *MP4.2 | Mesh Beacon Collision Avoidance | Error: Reference source not found | MP4:O | Yes No N/A  |
| MP4.2.1 | Beacon timing advertisement | Error: Reference source not found | MP4.2:M | Yes No N/A  |
| MP4.2.2 | TBTT selection | Error: Reference source not found | MP4.2:M | Yes No N/A  |
| MP4.2.3 | TBTT adjustment | Error: Reference source not found | MP4.2:M | Yes No N/A  |
| MP4.2.4 | Frame transmission across reported TBTT | Error: Reference source not found | MP4.2:O | Yes No N/A  |
| MP4.2.5 | Delayed beacon transmission | Error: Reference source not found | MP4.2:O | Yes No N/A  |
| *MP5 | MCCA | 9.9a.3 (MCF coordinated channel access (MCCA)) | MP1:O | Yes No N/A  |
| MP5.1 | MCCAOP Advertisement | 9.9a.3.8 (MCCAOP advertisements) | MP5:M | Yes No N/A  |
| MP5.2 | Neighbor MCCAOP Recognition | 9.9a.3.4 (Neighborhood MCCAOP times at a mesh STA) - 9.9a.3.6 (MCCA Access Fraction (MAF)) | MP5:M | Yes No N/A  |
| MP5.3 | MCCAOP Setup | 9.9a.3.7 (MCCAOP setup procedure) | MP5:M | Yes No N/A  |
| MP5.4 | Access during MCCAOPs | 9.9a.3.11 (Access during MCCAOPs) | MP5:M | Yes No N/A  |
| *MP6 | Intra Mesh Congestion Control | Error: Reference source not found | MP1:O | Yes No N/A  |
| MP6.1 | Local congestion monitoring and detection | Error: Reference source not found | MP6:M | Yes No N/A  |
| MP6.2 | Congestion Control signaling | Error: Reference source not found | MP6:M | Yes No N/A  |
| MP6.3 | Local rate control | Error: Reference source not found | MP6:M | Yes No N/A  |
| MP7 | MBSS channel selection | 11.9.7 (Selecting and advertising a new channel), 11.9a.3 (Selecting and advertising a new channel and/or operating class) | MP1:M | Yes No N/A  |
| *MP8 | Mesh power save operation (operation in light or deep sleep mode) | Error: Reference source not found | MP1:O | Yes No N/A  |
| MP8.1 | Mesh power modes | Error: Reference source not found, Error: Reference source not found | MP8:M | Yes No N/A  |
| MP8.2 | STA power state transitions | Error: Reference source not found | MP8:M | Yes No N/A  |
| MP8.3 | Mesh Awake Window and frame transmission | Error: Reference source not found | MP8:M | Yes No N/A  |
| *MP9 | Mesh power save support | Error: Reference source not found | MP1:M | Yes No N/A  |
| MP9.1 | TIM transmission | Error: Reference source not found | MP9:M | Yes No N/A  |
| MP9.2 | Link-specific mesh power modes | Error: Reference source not found | MP9:M | Yes No N/A  |
| MP9.3 | Non-peer power modes | Error: Reference source not found | MP9:M | Yes No N/A  |
| MP9.4 | Frame transmission to mesh STA in light sleep mode | Error: Reference source not found, Error: Reference source not found | MP9:M | Yes No N/A  |
| MP9.5 | Frame transmission to mesh STA in deep sleep mode | Error: Reference source not found, Error: Reference source not found | MP9:M | Yes No N/A  |
| MP10 | Airtime Link Metric computation | Airtime Link Metric | MP1:M | Yes No N/A  |
| MP11 | Link metric reporting | Link metric reporting | MP1:M | Yes No N/A  |
|  |  | Link metric reporting |  |  |
|  |  | Link metric reporting |  |  |
| MP12 | Proxy operation | Error: Reference source not found | MP1:O | Yes No N/A  |
| MP13 | Gate announcement | Error: Reference source not found | MP1:O | Yes No N/A  |
| *MP14 | MSDU/MMPDU forwarding | 9.22 (Mesh forwarding framework) | MP1:O | Yes No N/A  |
| MP14.1 | Individually addressed MSDU forwarding | 9.22.2.2 (Addressing of individually addressed Mesh Data frames and data forwarding) | MP14:M | Yes No N/A  |
| MP14.2 | Group addressed MSDU forwarding | 9.22.2.3 (Addressing of group addressed Mesh Data frames and data forwarding) | MP14:M | Yes No N/A  |
| MP14.3 | MMPDU forwarding | 9.22.2.4 (Addressing of Management frames and MMPDU forwarding) | MP14:M | Yes No N/A  |

References:

[1] Draft Amendment: Mesh Networking.  doc.: IEEE P802.11s/D7.03, November 2010.