IEEE P802.11
Wireless LANs

| GCR for mesh |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2011-01-19 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Ivan Pustogarov | IITP RAS | Bolshoy Karentniy Str. 19, Moscow, Russia | +74956503617 | Ivan.pustogarov@iitp.ru |
| Alexander Safonov | IITP RAS | Bolshoy Karentniy Str. 19, Moscow, Russia | +74956503617 | safa@iipt.ru |

Abstract

This document proposes changes to IEEE 802.11aa_D2.0 draft to extend GCR for Mesh BSS.

This submission was supported by FP7 project “FLAVIA”.

3. Definitions

EDITORIAL NOTE—The subclause numbering of definitions is of the form ―3.aa<x>‖ where <x> is an increasing number. The 802.11 technical editor will assign numbers when merging this list into the baseline

document.

Change definition 3.135 as follows:

3.135 service period (SP): A contiguous time during which one or more downlink unicast frames are transmitted to a quality of service (QoS) station (STA) and/or one or more transmission opportunities  (TXOPs) are granted to the same STA. SPs can be scheduled or unscheduled. For a non-access point (non- AP) STA, there can be at most one non-Groupcast with Retries (non-GCR) SP active at any time.

Insert new definitions 3.aa1 through 3.aa10 retaining the alphabetic ordering:

3.aa1 No retry/no acknowledgment (Ack): A retransmission policy for group addressed frames in which each frame is transmitted once and without acknowledgement.

3.aa2 Active from power save (Active-PS): A delivery method for group addressed frames whereby group addressed frames are transmitted when all associated non-access point (non-AP) stations (STAs) are in Active mode or after a beacon that causes the associated non-AP stations that are in power save (PS) mode to be awake.

3.aa3 Groupcast with Retries (GCR) service : A means for transmission and retransmission of medium access control (MAC) service data units (MSDUs) to a destination that is a group address that provides greater reliability by using individually addressed (re)transmissions and group addressed retransmissions, comprising this service, concealed from GCR-incapable stations.

3.aa4 Groupcast with Retries (GCR) group address: A group address subject to a GCR agreement

between the access point (AP) and at least one station (STA) within the infrastructure basic service set (BSS) or between peer mesh stations (STAs) in the mesh BSS.

3.aa5 Groupcast with Retries (GCR) frame: A group addressed frame subject to a GCR agreement between the access point (AP) and at least one station (STA) within the infrastructure basic service set (BSS) or between peer mesh stations (STAs) in the mesh BSS.

3.aa6 Groupcast with Retries (GCR) Service Period (GCR-SP) frame: A frame subject to the GCR service when delivery method is GCR-SP.

3.aa7 Groupcast with Retries (GCR) Service Period (GCR-SP) medium access control (MAC) service data unit (MSDU): An MSDU subject to the GCR service with delivery method  equal to GCR-SP.

3.aa8 Groupcast with Retries (GCR) Service Period (GCR-SP) aggregate medium access control (MAC) service data unit (A-MSDU): An A-MSDU subject to the GCR service with delivery method equal to GCR-SP.

3.aa9 Active Groupcast with Retries (GCR) Service Period (GCR-SP): A delivery method for a group addressed stream subject to a GCR agreement wherein the frames are transmitted at any time without regard to the power state of the non-access point (non-AP) stations (STAs)  in the group; i.e. a continuous Service Period.

3.aa10 Overlapping Access Points (APs): Overlapping APs are APs that are on the same channel and can receive each other‘s Beacons

3.aa11 Groupcast with Retries (GCR) transmission opportunity (TXOP): An interval of time when an access point (AP) or a mesh station (STA) has the right to initiate frame exchange sequences onto the wireless medium (WM) for the purpose of transmitting multiple frames that are subject to the GCR service.

5. General description

5.2 Components of the IEEE 802.11 architecture

EDITORIAL NOTE: This is clause 5.2.13.3  in 802.11s_D7.03

Change clause 5.2.13.3 as follows:

5.2.13.3 Mesh STA

A STA that belongs to a mesh BSS is termed a mesh station (mesh STA). Mesh STAs are QoS STAs that support mesh services, i.e., they participate in interoperable formation and operation of a Mesh Basic Service Set (MBSS). A mesh STA implements a subset of the QoS functionality.This subset is as follows:

— Use of QoS frame format, EDCA (as a part of MCF), Block Acknowledgement (optional), and No Acknowledgement (optional).

— Use of TSPEC (optional), TCLAS (optional), and TCLAS Processing elements (optional) to establish DMS or GCR agreements.

The QoS functionality depending on a Hybrid Coordinator (HC) does not apply in a mesh BSS. In particular a mesh STA does not implement the following subset of the QoS functionality:

— HCCA, traffic stream (TS) management, admission control, automatic power save delivery (APSD), and direct-link setup (DLS).

Insert the following subclauses after 5.2.11:

5.2.aa12 Robust Audio Video Streaming

Robust Audio Video Streaming improves audio video streaming performance over 802.11 for consumer and enterprise applications.

5.2.aa12.1 Groupcast with Retries

The Groupcast with Retries (GCR) Service allows a STA to request greater reliability for one or  more  group  addressed  streams  that  the  STA  receives.  Greater  reliability  is  provided  via transmission as individually addressed frames, unsolicited retries, or the Block Ack mechanism. A non- AP STA may request delivery when all associated non-AP STAs are in Active mode, so that the AP transmits the frames via EDCA within regular Service Periods.

6. MAC service definition

6.1 Overview of MAC services

6.1.1 Data service

6.1.1.3 Interpretation of service class parameter in MAC service primitives in a STA

EDITORIAL NOTEThis is clause 5.1.1.5 in REVmb D6.0

Change 6.1.1.3 as follows:

In QoS STAs, the value of the service class parameter in the MAC service primitive (see 6.2) may be a noninteger value of QoSAck or QoSNoAck.

When an MSDU is received from the MAC_SAP and the recipient STA is a QoS STA with the service class set to

-  QoSAck, the MSDU is transmitted using a QoS data frame with the Ack Policy subfield in the QoS Control field set to either Normal Acknowledgment (Normal Ack) or Block Ack.

- QoSNoAck, the MSDU is transmitted using a QoS data frame with the Ack Policy subfield in the QoS Control field set to No Acknowledgment (No Ack). If the sender STA is an AP and the frame has a group DA, then the MSDU is buffered for transmission and is also sent to the DS.

If the sender STA is an AP and the frame is a group addressed MSDU, then the MSDU is buffered for transmission and is also sent to the DS.

When an MSDU is received from the MAC_SAP and the recipient STA is not a QoS STA, the MSDU is transmitted using a non-QoS data frame.

When  a  QoS  data  frame  is  received  from  another  STA,  the  service  class  parameter  in  MA- UNITDATA.indication primitive is set to

   QoSAck, if the frame is a QoS data frame with the Ack Policy subfield in the QoS Control field set to either Normal Ack or Block Ack., or the frame is a GCR frame.

   QoSNoAck, if the frame is a QoS data frame with the Ack Policy subfield in the QoS Control field set to No Ack. This service class is also used where the DA parameter is a group address. unless the frame is to be delivered via the GCR service.

When   a   non-QoS   data   frame   is   received   from  a   STA,   the   service   class   parameter   in   MA- UNITDATA.indication primitive is set to

   QoSAck, if the frame is an individually addressed frame and is acknowledged by the STA.

   QoSNoAck, if the frame is a group addressed frame andor is not acknowledged by the STA.

NOTE— that the group addressed frames sent by a non-QoS STA are not acknowledged regardless of the service class parameter in MA-UNITDATA.indication primitive.

NOTE— GCR frames are only transmitted by a QoS AP or a mesh STA.

7. Frame formats

7.1 MAC frame formats

7.1.3 Frame fields

7.1.3.5.2 EOSP (end of service period) subfield

Insert the following paragraph at the end of 7.1.3.5.2:

If dot11RobustAVStreamingImplemented is true then the HCsets the EOSP field to 1 in a GCR-SP group addressed  frame  in  order  to  indicate  that  no  more  GCR-SP  frames  of that  group  address  are  to  be transmitted by the AP until the next scheduled SP for this GCR-SP stream. The EOSP field is set to 0 in a group addressed frame delivered using the Active GCR-SP procedures described in 11.22.15.aa2.8.

7.2 Format of individual frame types

7.2.2 Data frames

7.2.2.1 Data frame format

Change the third paragraph of 7.2.2.1 as follows:

A QoS STA always uses QoS data frames for data transmissions to other QoS STAs. A QoS STA uses frames with the QoS subfield of the Subtype field set to 0 for data transmissions to non-QoS STAs. A non- QoS STA always uses frames with the QoS subfield of the Subtype field set to 0 for data transmissions to other STAs. All STAs use frames with the QoS subfield of the Subtype field set to 0 for non-concealed GCR broadcast data frames unless a transmitting STA knows that all STAs in a BSS have QoS capability, in which case the transmitting STAs use QoS data frames. All STAs use frames with the QoS subfield of the Subtype field set to 0 for non-concealed GCR multicast data frames unless it is known to the transmitter that all STAs in the BSS that are members of the multicast group have QoS capability, in which case STAs use QoS data frames. QoS APs or mesh STAs use frames with the QoS subfield of the Subtype field set to 1 for concealed GCR frames as described in 11.22.15.aa2.5.

7.3 Management frame body components

7.3.2 Information elements

7.3.2.27 Extended Capabilities information element

Insert the rows for Bit <ANA>, and change the Reserved row in Table 7-35a as follows (note that the

entire table is not shown here):

EDITORIAL NOTE—The <ANA> will be replaced with a number assigned by the 802.11 Assigned

Numbers Authority once that assignment has been made.

Table 7-35a—Capabilities field

| Bit | Information | Notes |
| --- | --- | --- |
| <ANA> | Robust AV Streaming | The STA sets the Robust AV Streaming field to 1 when the MIB attribute dot11RobustAVStreamingImplemented is true , and sets it to 0 otherwise. See 11.aa23. |
| <ANA> | Advanced GCR | The STA sets the Advanced GCR field to 1 when the MIB attribute dot11GCRActivated is true , and sets it to 0 otherwise. See 9.2.7.3. |
| <ANA> | Mesh Robust AV Streaming | The STA sets the Mesh Robust AV Streaming field to 1 when the MIB attribute dot11MeshRobustAVStreamingImplemented is true , and sets it to 0 otherwise. See 11.aa23. |
| <ANA> | Mesh Advanced GCR | The STA sets the mesh Advanced GCR field to 1 when the MIB attribute dot11MeshGCRActivated is true , and sets it to 0 otherwise. See 9.2.7.3. |
| <ANA> | SCS | The STA sets the SCS field to 1 when the MIB attribute dot11SCSActivated is true , and sets it to 0 otherwise. See 11.aa23.2 (SCS Procedures) |
| <ANA> | QLoad Report | When dot11QLoadReportActivated is true , the QLoad Report field is set to 1 to indicate the AP supports QLoad Report as described in 11.aa24.1. |
| <ANA> | Alternate EDCA | The STA sets the Alternate EDCA field to 1 when the MIB attribute dot11AlternateEDCAActivated is true , and sets it to 0 otherwise. See 9.1.3.1. |
| (<ANA> +1) — n*8 | Reserved | All other bits are reserved, and are set to 0 on transmission and ignored on reception. |

7.3.2.30 TSPEC element

Change the first paragraph of 7.3.2.30 as follows:

The TSPEC element contains the set of parameters that define the characteristics and QoS expectations of a traffic flow, in the context of a particular STA, for use by the HC and non-AP STA(s) or a mesh STA and its peer mesh STAs in support of QoS traffic transfer using the procedures defined in 9.2.7.3.2 and 11.4. The element information format comprises the items as defined in this subclause, and the structure is defined in Figure 7-82.

Change the Reserved row in Table 7-41 as follows:

Table 7-41—Setting of Schedule subfield

| APSD | Schedule | Usage |
| --- | --- | --- |
| 0 | 0 | No Schedule |
| 1 | 0 | Unscheduled APSD |
| 0 | 1 | Scheduled PSMP or GCR- SPReserved |
| 1 | 1 | Scheduled APSD |

Change paragraphs 6 and 7 of 7.3.2.30 as follows:

The Minimum Service Interval field is 4 octets long and contains an unsigned integer that specifies the minimum interval, in microseconds, between the start of two successive SPs. If the TSPEC element is included within a GCR Request element that has the GCR delivery method  set to GCR-SP, a Minimum Service Interval field equal to 0 indicates that Service Periods up to the Maximum Service Interval are requested, including the continuous service period used by the Active GCR-SP delivery method.

The Maximum Service Interval field is 4 octets long and contains an unsigned integer that specifies the maximum interval, in microseconds, between the start of two successive SPs.  The Maximum Service Interval field is greater than or equal to the Minimum Service Interval. If the TSPEC element is included within a GCR Request element that has the GCR delivery method set to GCR-SP, a Maximum Service Interval field equal to 0 indicates that the continuous service period used by the Active GCR-SP delivery method is requested.

Change paragraph 10 of 7.3.2.30 as follows:

The Service Start Time field is 4 octets and contains an unsigned integer that specifies the time, expressed in microseconds, when the first scheduled SP starts. The service start time indicates to AP the time when a non-AP STA first expects to be ready to send frames and a power-saving non-AP STA will be awake to receive frames. This may help the AP to schedule service so that the MSDUs encounter small delays in the MAC and help the power-saving non-AP STAs to reduce power consumption. The field represents the four lower order octets of the TSF timer at the start of the SP. If APSD and Schedule subfields areis set to 0, this field is also set to 0 (unspecified).

7.3.2.34 Schedule element

Change the first paragraph of 7.3.2.34 as follows:

The Schedule element is transmitted by the HC to a non-AP STA to announce the schedule that the HC/AP follows for admitted streams originating from or destined to that non-AP STA, or GCR-SP streams destined to that non-AP STA in the future. The information in this element may be used by the non-AP STA for power management, internal scheduling, or any other purpose. The element information format is shown in Figure 7-93.

Change the third paragraph of 7.3.2.34 as follows:

The Aggregation subfield is set to 1 if the schedule is an aggregate schedule for all TSIDs associated with the non AP STA to which the frame is directed. It is set to 0 otherwise. The TSID subfield is as defined in 7.1.3.5.1 and indicates the TSID for which this schedule applies, except when a Schedule element is sent within a GCR Response element, when the TSID field is reserved. The Direction subfield is as defined in 7.3.2.30 and defines the direction of the TSPEC associated with the schedule.For a Schedule element sent within a GCR Response element, the Direction subfield is set to Downlink. The TSID and Direction subfields are valid only when the Aggregation subfield is set to 0. If the Aggregation subfield is set to 1, the TSID and Direction subfields are reserved.

Change the fifth paragraph of 7.3.2.34 as follows:

The Service Interval field is 4 octets and indicates the time, expressed in microseconds, between two successive SPs and represents the measured time from the start of one SP to the start of the next SP. If the Schedule element is included within a GCR Response element that has the GCR delivery method  set to GCR-SP, a value of 0 in the Service Interval field indicates the delivery method is Active GCR-SP.

Change the seventh paragraph of 7.3.2.34 as follow

In cases other than a Schedule element included within a GCR Response element that has the GCR delivery method set to GCR-SP, Tthe HC may set both the Service Start Time field and the Service Interval field to 0 (unspecified) for non-powersaving STAs.

7.3.2.88 DMS Request element

Change paragraphs 8, 9, and 10 of 7.3.2.88 as follows:

When the Request Type field is set to "Add", the TCLAS elements field contains one or more TCLAS information elements to specify group addressed frames as defined in 7.3.2.31. When a GCR Request subelement is included in the DMS Descriptor and the Request Type field is set to ”Add”, the TCLAS Elements field contains at least a TCLAS information element  with Frame classifier type equal to 0(Ethernet parameters) to specify a destination group address as defined in 7.3.2.31. When the Request Type field is set to any value other than "Add", the TCLAS Elements field contains zero TCLAS elements.

When the Request Type field is set to  “Add” and when there are two or more TCLAS information elements present, the TCLAS Processing Element field optionally contains one TCLAS Processing information element to define how these TCLAS information elements are to be processed, as defined in 7.3.2.33. Otherwise, the TCLAS Processing Element field contains zero TCLAS Processing information elements.

When the Request Type  field is set to ”Add” or  “Change”, the TSPEC Element field optionally contains one TSPEC information element to specify the characteristics and QoS expectations of the corresponding traffic flow as defined in 7.3.2.30. When a GCR Request subelement is included in the DMS Descriptor and the Request Type field value is set to  ”Add” or  ”Change”,  the TSPEC Element field contains one TSPEC  information element.  Otherwise, the  TSPEC Element  field  contains  zero  TSPEC information elements.

Change the Reserved row in Table 7-43bd as follows:

Table 7-43bd—Optional Subelement IDs for DMS Descriptor

| Subelement ID | Name | Length field (octets) | Extensible |
| --- | --- | --- | --- |
| 0-220 | Reserved |  |  |
| 1 | GCR Request | 2 | Yes |
| 2-220 | Reserved |  |  |
| 221 | Vendor Specific | 3 to 248 |  |
| 222-255 | Reserved |  |  |

Insert the following paragraphs, figures and tables after Table 7-43bd and before paragraph 13.

Each DMS Descriptor contains zero or one GCR Request subelements. If present and the Request Type field is set to  “Add” or  “Change”, the GCR Request subelement indicates a request by a STA to respectively add or change the GCR service for a group address stream identified by the TCLAS information element or DMSID in the DMS Descriptor, respectively. The format of the GCR Request subelement is shown in Figure 7-aa3.

| Subelement ID | Length | GCR Retransmission Policy | GCR Delivery Method |
| --- | --- | --- | --- |

Octets:	1	1	1	1

Figure 7-aa3—GCR Request subelement field

The value of the GCR Request subelement Length field is 2.

The GCR Retransmission Policy field is set to indicate the STA‘s preferred retransmission policy or the group address for which the GCR service is requested. The values are shown in Table 7 aa1.

Table 7-aa1— GCR Retransmission Policy field values

| Value | GCR Retransmission Policy | Notes |
| --- | --- | --- |
| 0 | No Preference |  |
| 1 | DMS | See 11.22.15.1 |
| 2 | GCR-Unsolicited-Retry | See 11.22.15.aa2.6 |
| 3 | GCR-Block-Ack | See 11.22.15.aa2.7 |
| 4-255 | Reserved |  |

The GCR Delivery Method field is set to indicate the STA‘s preferred delivery method for the group address for which the GCR service is requested. The values are shown in Table 7-aa2.

Table 7-aa2— GCR Delivery Method field values

| Value | GCR Delivery Method | Notes |
| --- | --- | --- |
| 0 | No Preference |  |
| 1 | Active -PS or FMS |  |
| 2 | GCR-SP | See 11.22.15.aa2.8 |
| 3-255 | Reserved |  |

7.3.2.89 DMS Response element

Change the fourth paragraph of 7.3.2.89 as follows:

The Response Type field indicates the response type returned by the AP responding to the non-AP STA's request or by a mesh STA responding to its peer mesh STA’s request or indicates the DMS Status is an advertisement of an existing GCR service, as indicated in Table 7-43be.

Change the fifth paragraph of 7.3.2.89 as follows:

In an infrastructure BSS, the DMSID field is assigned by the AP and provides a unique identifier within the BSS for the DMS traffic flow identified by the TCLAS Elements, TCLAS Processing Element and TSPEC Element fields. The uniqueness of this identifier is independent of the ordering of the TCLAS elements. In a mesh BSS, the DMSID field is assigned by a mesh STA that responds to a GCR Request and is unique among all existing DMSIDs used by the mesh STA for its current GCR agreements.

Change Table 7-43be as follows:

Table 7-43be—Response Type field values

| Field value | Description | Notes |
| --- | --- | --- |
| 0 | Accept | STA accepts the DMS or GCR request |
| 1 | Denied | STA rejects the DMS or GCR request |
| 2 | Terminate | STA terminates the previously accepted DMS or GCR request |
| 3 | GCR Advertise | STA advertises a group addressed stream subject to an existing GCR agreement |
| 34-255 | Reserved |  |

Change paragraphs10, 11, and 12 of 7.3.2.89 as follows:

When the Status field is set to ”Accept” or ”Denied” and a GCR Response subelement is not included in the DMS Status field, the TCLAS Elements field contains one or more TCLAS information elements to specify group addressed frames as defined in 7.3.2.31. When the Status field is set to ”Accept”, ”Denied or “GCR  Advertise” and a GCR Response subelement is included in the DMS Status field, the TCLAS Elements field contains at least one TCLAS information element with Frame classifier type equal to 0 (Ethernet parameters) to specify a destination group address as defined in 7.3.2.31. Otherwise, the TCLAS Elements field contains zero TCLAS information elements.

When the Status field is set to “Accept” or “Denied”, the TCLAS Processing Element field optionally contains one TCLAS Processing information element to define how these TCLAS information elements are to be processed,  as defined in 7.3.2.33. When the Status field is set to “Terminate” or when there is only one TCLAS information element, the TCLAS Processing Element field contains zero TCLAS Processing elements.

When the Status field is set to “Accept” or “Denied”, the TSPEC Element field optionally contains one TSPEC information element to specify the characteristics and QoS expectations of the corresponding traffic flow as defined in 7.3.2.30. When a GCR Response subelement is included in the DMS Status field and the Type field value is set to “Accept”, “Denied or “GCR Advertise”, the TSPEC Element field contains one TSPEC information element. Otherwise, the TSPEC Element field contains zero TSPEC elements.

Change the reserved rows of Table 7-43bf as follows:

Table 7-43bf—Optional Subelement IDs for DMS Status

| Subelement ID | Name | Length field (octets) | Extensible |
| --- | --- | --- | --- |
| 0-220 | Reserved |  |  |
| 1 | GCR Response | 1 to 249 | Subelements |
| 2-220 | Reserved |  |  |
| 221 | Vendor Specific | 3 to 248 |  |
| 222-255 | Reserved |  |  |

Insert the following paragraphs after Table 7-43bf and before paragraph 15.

The GCR Response subelement contains a response:

- by an AP to a GCR request by a non-AP STA

- by a mesh STA to a GCR request by a peer mesh STA for GCR service for a group address

- an unsolicited advertisement for the parameters of a group addressed stream subject to the GCR service.

The format of the GCR Response subelement is shown in Figure 7-aa4.

| Subelement ID | Length | GCR Retransmission Policy | GCR Delivery Method | GCR Concealment Address | Schedule element |
| --- | --- | --- | --- | --- | --- |

Octets:	1	1	0 or 1	0 or 1	0 or 6	0 or 14

Figure 7-aa4—GCR Response subelement field

The  GCR  Retransmission  Policy,  GCR  Delivery  Method,  GCR  Concealment  Address  and  Schedule element fields are present when the Status field is not equal to Denied; otherwise they are omitted.

The GCR Retransmission Policy field is set to indicate the current GCR retransmission policy for the group address for which the GCR service is requested. The values are shown in Table 7-aa1.

The GCR Delivery method field is set to indicate the current GCR Delivery method for the group address for which the GCR service is requested. The values are shown in Table 7-aa2.

The GCR Concealment Address, when present, indicates the GCR concealment address.

The Schedule Element field is present if the GCR Delivery method field is equal to GCR-SP. It indicates the current SP schedule for the group addressed stream (see 7.3.2.34).

7.4 Action frame format details

7.4.12 WNM Action details

7.4.12.26 DMS Response frame format

Change the first paragraph of 7.4.12.26 as follows:

The DMS Response frame is sent by an AP or a mesh STA in response to a DMS Request frame, or autonomously to terminate a requested DMS stream, or to advertise the current parameters for one or more GCR streams. The format of the DMS Response frame is shown in Figure 7-101aw.

Insert the following new subclause (7.4.13) after 7.4.12:

7.4.aa13.3 Group Membership Request frame format

The   Group   Membership   Request   frame   is   sent   to   a   STA   to   request   the   contents   of   its

dot11GroupAddressesTable.  The  Action  field  of  a  Group  Membership  Request  frame  contains  the

information shown in Figure 7-aa23.

Category               Robust Action          Dialog Token

Octets:	1	1	1

Figure 7-aa23—Group Membership Request frame Action field format

The Category field is set to <ANA> (representing Robust AV Streaming).

The Robust Action field is set to the value specified in Table 7-aa12 for a Group Membership Request frame.

The Dialog Token field is set to a nonzero value that is unique among the Group Membership Request frames sent by the  STA for which a corresponding Group Membership Response frame has not been received.

Usage of the Group Membership Request frame is described in 11.22.15.aa2.2.

9. MAC sublayer functional description

9.9 HCF

9.9.1 HCF contention-based channel access (EDCA)

9.9.1.6 Retransmit procedures

Insert the following subclause (9.9.1.6.aa1) after 9.9.1.6:

9.9.1.6.aa1 Unsolicited retry procedure

When using the GCR-Unsolicited-Retry delivery method for a group address, the AP or a mesh STAmay retransmit an MPDU to increase the probability of correct reception of group addressed frame by STAs that are listening to this group address (i.e. the group address is in their dot11GroupAddressTable). How an AP or a mesh STA chooses which MPDUs to retransmit is an implementation decision and beyond the scope of this standard.

A protective mechanism (such as transmitting using HCCA, MCCA, RTS/CTS, setting the Duration fields in the first  frame  and  response  frames  to  update  the  NAVs  of  STAs  in  the  BSS  and  OBSS(s)  or  another mechanism described in 9.13) should be used to reduce the probability of other STAs transmitting during the GCR TXOP. In an infrastructure BSS, if there is more than one STA in a GCR group, an AP may use the OBSS information reported by STAs to select the responding STA.

The TXOP initiation rules defined in 9.9.1.2 (EDCA TXOPs) and 9.9.2.2 (TXOP structure and timing) shall be used for initiating a GCR TXOP.

When transmitting MPDUs using the GCR service with retransmission policy equal to GCR-Unsolicited- Retry:

   Following a MAC protection exchange that includes a response frame, for all retransmissions the STA shall either transmit the frames within a TXOP separated by an interframe space (subject to TXOP limits) or invoke its backoff procedure at the PHY-TXEND.confirm with a CW equal to CWmin[AC]. The STA shall not transmit an MPDU and a retransmission of the same MPDU within the same TXOP. The final frame transmitted within a GCR TXOP shall follow the backoff procedure defined in 9.9.1.5

   Without  MAC  protection  or  with  MAC  protection  that  lacks  a  response  frame,  for  all transmissions  the  STA  shall  invoke  the  backoff  procedure  defined  in  9.9.1.5  at  the  PHY- TXEND.confirm.

9.10 Block Acknowledgment (Block Ack)

Insert the following subclauses (9.10.aa10) after 9.10.9:

EDITORIAL NOTE: Clause 9.10.9 is clause 9.20.9 in REVmb D6.0

9.10.aa10 GCR Block Ack

This subclause extends the Block Ack mechanism to group addressed frames that are subject to the GCR-Block Ack retransmission policy.

A protective mechanism (such as transmitting an HCCA CAP, MCCA,  RTS/CTS, setting the Duration field in the first  frame  and  response  frames  to  update  the  NAVs  of  STAs  in  the  BSS  and  OBSS(s)  or  another mechanism described in 9.13) should be used to reduce the probability of other STAs transmitting during the GCR TXOP. The protective mechanism of NAV update can be achieved by setting the Duration field in the first and response frames appropriately to cover the entire duration of the TXOP and thereby update the NAVs to of STAs in the BSS and OBSS(s) according to the rules of 9.2.5.4.In an infrastructure BSS,  if there is more than one STA in a GCR group, an AP may use the OBSS information reported by STAs to select the STA used to initiate the protection mechanism.

After an originator transmits between one and GCR Buffer Size MSDUs or A-MSDUs with RA set to a GCR group address when the retransmission Policy for that group address is GCR-Block-Ack, the originatorshall send a BlockAckReq to one of the STAs that has a GCR-Block-Ack agreement for this group address. Upon reception of the BlockAck, an originator may send a BlockAckRequest to another STA that has a Block-Ack agreement for this group address, and this process may be repeated multiple times. Theoriginator shall not send a BlockAckReq to a STA with a MAC address that matches the SA in any of the MSDUs or A-MSDUs transmitted during the GCR TXOP.

NOTEAs an example of how the above procedure might be implemented, the originatorsends a BlockAckReq to one group member after several MSDUs have been delivered using the GCR-Block-Ack retransmission policy. Theoriginator begins with the first member of the GCR group and cycles through the members as the originatortransmits subsequent GCR–Block-Ack MSDUs.

When a recipient receives a BlockAckReq with the GCR Group Address subfield equal to a GCR group address, the recipient shall transmit a BlockAck frame at a delay of SIFS after the BlockAckReq. The BlockAck acknowledges the STA‘s reception status of the block of group addressed frames requested by the BlockAckReq frame. The receive buffer operation, the selection of BlockAck and BlockAckReq variants, and the BlockAck generation shall follow the rules in 9.10.4, 9.10.6, and 9.10.7.

Figure 9-aa1 shows an example of a frame exchange when the GCR Block-Ack retransmission policy isused. The originator sends several MPDUs using the GCR-Block-Ack retransmission policy. The originator then sends a BlockAckRequest frame to group member 1 of the GCR group, waits for the BlockAck frame and then sends a BlockAckRequest to group member 2. After receiving the BlockAck frame from GCR group member 2, the originatordetermines if any MPDUs need to be retransmitted and sends some more MPDUs (some of which might be retransmissions of previous MPDUs) using the GCR-Block-Ack retransmission policy .

BlockAckReq and BlockAck frames might be lost or incorrectly received by the intended recipients. If an originatortransmits a GCR BlockAckReq to a GCR group member and does not successfully receive a BlockAck frame from the STA, then the originator may retransmit, in a new TXOP, a BlockAckReq. The process may be restarted by the originatortransmitting an updated BlockAckReq with a new Block Ack Starting Sequence Control field if the data MSDUs requested for acknowledgement in the BlockAckReq have reached their lifetime limit.

After  completing  the  BlockAckReq  and  BlockAck  frame  exchanges,  the  originator  determines  from  the information provided in the BlockAck bitmap and from the missing BlockAcks which, if any,  MSDUs or A-MSDUs that need to be retransmitted.

An originatoradopting the GCR-Block-Ack retransmission policy for a GCR group address chooses a lifetime limit for the group address. The originatormay vary the lifetime limit for the group address at any time, and may use different lifetime limits for different GCR group addresses. The originatortransmits and retries each MSDU or A-MSDU until the appropriate lifetime limit is reached, or until each one has been received by all group members to which a BlockAckReq has been sent, whichever occurs first.

An originator may regularly send a BlockAckReq with the GCR Group Address subfield set to the GCR group address and the Block Ack Starting Sequence Control set to the Sequence Number field of the earliest MSDU or A-MSDU of the GCR stream that has not expired due to lifetime limits, for GCR streams with retransmission policy equal to GCR-Block-Ack, in order to minimize buffering latency at receivers in the GCR group.

NOTEThis is because an originatormight transmit management frames, QoS data frames with a group address in the Address 1 field (including different GCR streams), and non-QoS data frames intermingled. Since these are transmitted using a single sequence counter, missing frames or frames sent to group addresses absent  from  a receiving  STA‘s  dot11GroupAddresses  table complicates receiver processing   for  GCR streams with a GCR-Block-Ack retransmission policy since the cause of a hole in a receiver‘s  Block Ack bitmap is ambiguous: it is due either to an MPDU being lost from the GCR stream or to transmissions of MPDUs not related to the GCR service using the same sequence number counter.

The  beginning  of  reception  of  a  BlockAck  response  is  detected  by  the  occurrence  of  PHYCCA indication(BUSY,channel-list) primitive at the STA that is expecting the response where:

   The channel-list parameter is absent, or

   The channel-list is equal to {primary} and the HT STA expected to transmit the expected response supports 20 MHz operation only, or

   The channel-list is equal to either {primary} or {primary, secondary} and the HT STA expected to transmit the expected response supports both 20 MHz and 40 MHz operation (see 10.15.2 (Basic 20/40 MHz BSS functionality)).

If the beginning of such reception does not occur during the first slot time following a SIFS, then the originator may perform error recovery by retransmitting a BlockAckReq frame PIFS after the previous BlockAckReq frame when both of the following conditions are met:

   The carrier sense mechanism (see 9.2.1) indicates that the medium is idle at the TxPIFS slot boundary (defined in 9.2.10) after the expected start of a BlockAck, and

   The Duration of the failed BlockAck is longer than the total time of the retransmitted GCR BlockAckReq plus one slot time.

NOTEIf an originator fails to receive a BlockAck frame in response to BlockAckReq frame and there is insufficient time to transmit a recovery frame, the  originator retransmits the BlockAckReq frame in a new TXOP.

10. Layer management

10.3 MLME SAP Interface

10.3.70 DMS or GCR request and response procedure

Change the first paragraph of 10.3.70 as follows:

The following MLME primitives support the signaling of DMS or GCR request and response procedure. The informative diagram shown in Figure 10-6s depicts the DMS or GCR request and response process and is not meant to be exhaustive of all possible protocol uses.

Change Figure 10-6s as follows:

SME

STA 	 AP  or mesh STA

MLME	MLME

SME

MLME-

DMS.request 	DMS Request Frame

MLME- DMS.indication

Process DMS or GCR request

MLME-

DMS.confirm 	DMS Response Frame

MLME- DMS.response

AP or  mesh STA 
may decide to terminate a   
 granted DMS or GCR service

at any time

MLME-DMS-

TERM.indication	 DMS Response Frame

Figure 10-6s—DMS or GCR Setup Protocol   Exchange

MLME-DMS- TERM.request

10.3.70.5 MLME-DMS-TERM.request

10.3.70.5.1 Function

Change the first paragraph of 10.3.70.5.1 as follows:

This primitive requests the transmission of a DMS Response frame to STAs to terminate a granted DMS or GCR service.

11. MLME

11.22 Wireless network management procedures

Change the title of 11.22.15 as follows

11.22.15 Directed Multicast Service and Groupcast with Retries DMS Procedure

Insert the subclause title of 11.22.15.1 after the title of 11.22.15 as follows

11.22.15.1 DMS Procedures

EDITORIAL NOTE—the following change is based on IEEE P802.11v D16.0.

Change the second paragraph of 11.22.15.1 as follows

Implementation of DMS is optional for a WNM STA and mandatory for a Robust AV Streaming STA. A STA that implements DMS has the MIB attribute dot11MgmtOptionDMSImplemented set to true. When dot11MgmtOptionDMSImplemented is true, at least one of dot11WirelessManagementImplemented and dot11RobustAVStreamingImplemented shall be true, and dot11HighThroughputOptionImplemented shall be true. A STA that has a value of true for the MIB attribute dot11MgmtOptionDMSActivated is defined as a	STA that	supports	Directed	Multicast.	A	STA	for	which	the	MIB	attribute dot11MgmtOptionDMSActivated is true shall set the DMS field of the Extended Capabilities information element to 1.

Insert the following subclauses at the end of 11.22.15

11.22.15.aa2 GCR Procedures

11.22.15.aa2.1 Overview

Advanced GCR is optional for a RobustAVStreaming STA. In an infrastructure BSS, a STA that implements advanced GCR has the MIBattribute	dot11GCRImplemented	set	to	true. When	dot11GCRImplemented	is true, dot11MgmtOptionDMSImplemented and dot11HighThroughputOptionImplemented shall be true. In a mesh BSS, a STA that implements advanced GCR has the MIB attribute	 dot11MeshGCRImplemented	set	to	true. When	dot11MeshGCRImplemented	is true, dot11HighThroughputOptionImplemented shall be true.

Groupcast with Retries (GCR) is a flexible service to improve the delivery of group addressed frames while optimizing for a range of criteria. GCR service may be provided by the AP to associated STAs in an infrastructure BSS or by a mesh STA to its peer mesh STAs in a mesh BSS. GCR is an extension of DMS (11.22.15.1). In particular:

- A GCR agreement applies to a single group address whereas a DMS flow is defined by TCLAS

information element(s) and an optional TCLAS Processing information element, and

- DMS offers multicast-to-unicast conversion only whereas GCR includes several retransmission policies and delivery methods.

DMS allows the transmission of group addressed MSDUs as individually addressed A-MSDUs and is particularly suited to low numbers of group members. It provides a high level of reliability but has low scalability as the efficiency decreases and delay increases proportionally to the number of group members.

GCR employs the DMS Request and DMS Response elements with the addition of GCR Request and Response subelements respectively for administering the set up and tear down of GCR services between an AP and non-AP STAs or between peer mesh STAs.  The DMS procedures and state machine of 11.22.15.1 shall apply to GCR with the extensions and constraints specific to GCR described below in 11.22.15.aa2.3 to 11.22.15.aa2.8.

GCR  defines  two  additional  retransmission  policies  for  group  addressed  frames,  in  addition  to  the

mechanisms defined in 9.2.7 (labeled “No-Ack/No-Retry” or “non-GCR”), and 11.22.15.1 (labeled DMS):

   GCR-Unsolicited-Retry

   GCR-Block-Ack

When using the GCR-Unsolicited-Retry delivery method for a group address, the STA providing GCR service retransmits an MSDU one or more times to increase the probability of correct reception at STAs that are listening to this group address. The decision to retransmit these MSDUs is implementation dependant. GCR-Unsolicited-Retry is particularly suited to use with large numbers of group members as it has moderate delay, efficiency and reliability, but high scalability.

The GCR-Block-Ack delivery method extends the block acknowledgement mechanism to group addressed frames. The STA providing GCR service initiates block Ack agreements with each  STA receiving GCR frames that supports GCR-Block-Ack for a particular group address. Once this block Ack agreement is in place, the STA providing GCR service regularly sends Block Ack Request frames to the STAs receiving the frames to ascertain the reception status of MSDUs related to this group address, as described in 9.10.10. This allows the STA providing GCR service to discover MSDUs that have failed to be received and to schedule their retransmission. GCR-Block-Ack is particularly suited to use with moderate numbers of group members as it has moderate delay, high efficiency, moderate scalability and reliability.

The GCR service has two delivery methods for group addressed frames:

   As per 11.2.1 (labeled “Active-PS”) or FMS (see 11.2.1.4a) in an infrastructure BSS, or as per   			   11C.13 in a mesh BSS (collectively labeled “non 	GCR-SP”)

   GCR-SP (see 11.22.15.aa2.8)

GCR-SP transmits GCR group addressed frames at regular intervals. Compared to non-GCR-SP, GCR-SP has lower delay and jitter and moderate power savings.

11.22.15.aa2.2 GCR Group Membership Procedures

The procedures described in clauses 11.22.15.aa2.3 to 11.22.15.aa2.8 depend upon the AP or a mesh STAknowing the membership of the multicast groups of STAs that support GCR.

One method for an AP to discover the multicast groups to which its associated STAs are receiving  or for a mesh STA to discover the multicast groups to which its peer mesh STAs are receiving is to use the  Group  Membership  Request  frame  (as  defined  in  7.4.7.aa22)  to  request  the  contents  of  the dot11GroupAddressesTable of its associated STAs or peer mesh STAs.

Other methods of group membership detection are also possible, using information that is outside the scope of this standard. For example group membership detection could be achieved via RFC 3376 (Internet Group Management Protocol (IGMP)) snooping.

A STA for which dot11GCRActivated or dot11MeshGCRActivated is true shall reply to a Group Membership Request frame by sending a Group Membership Response frame with the dialog token field set to the value from the  Group   Membership   Request  frame,   the   Address  Count   field   set   to   the   number   of  entries  in dot11GroupAddressesTable and the Group Address List field set to the group MAC addresses in the dot11GroupAddressesTable.  A STA  for   which  dot11GCRActivated or dot11MeshGCRActivated   is  true  shall   set dot11GCRGroupMembershipAnnouncementActivated  to  true  upon  reception  of  a  Group  Membership Request frame.

A		STA	for	which			dot11GCRGroupMembershipAnnouncementActivated and at least one of dot11MeshGCRActivated or dot11GCRActivated are   true   shall   send   an   unsolicited   Group Membership Response frame with the dialog token field set to 0, the Address Count field set to the number of entries in dot11GroupAddressesTable and the Group Address List field set to the group MAC addresses in the dot11GroupAddressesTable, every time the contents of the dot11GroupAddressesTable is modified. If an unsolicited Group Membership Response frame is sent by a mesh station in a mesh BSS, the frame shall be a broadcast frame.

11.22.15.aa2.3 GCR Setup Procedures

If an AP for which dot11GCRActivated is true or or mesh STA for which  dot11MeshGCRActivated is true detects that an associated STA with Robust AV Streaming set to 1 in the Extended Capabilities element in the STA‘s most recent (Re)Association  Request or a peer mesh STA with Mesh Robust AV Streaming set to 1 in the Extended Capabilities element  in the most recent mesh beacon is receiving one or more group addresses for which there is an active GCR service and it does not have a GCR agreement  for  the  group(s),  then  the  AP or mesh STA may  alert  the  associated  STA or peer mesh STAby  sending  an  unsolicited individually addressed DMS Response frame that contains one DMS Status field with a GCR Response subelement per group address. Each DMS Status field includes a TCLAS element to identify the GCR group address, the DMSID corresponding to this GCR traffic flow, and other associated parameters. The Status field of this DMS Status field shall be set to “GCR Advertise”. The associated STA may ignore the DMS Response frame, or initiate a GCR agreement for one or more of the group addresses.

A STA may request use of the GCR service for a group address by sending a DMS Descriptor as described in 11.22.15.1 with the following modifications:

   The DMS Descriptor shall contain one TCLAS element with Frame classifier type equal to 0 (Ethernet parameters), one TSPEC element and one GCR Request subelement.

   The DMS Descriptor may contain other TCLAS elements in addition to the mandatory TCLAS

element (that has a Frame classifier type equal to 0).

   When there are multiple TCLAS elements, a TCLAS processing element shall be present.

Otherwise no TCLAS processing elements shall be present in the DMS Descriptor.

   The TSID subfield within the TS Info field of the TSPEC element shall be reserved. Since the

AP might choose a delivery method of GCR-SP, the non-AP STA should   set the Minimum Service Interval, Maximum Service Interval and Service Start Time fields in the TSPEC to indicate the STA‘s preferred wake-up  schedule.In a mesh BSS, the Delivery  Method field shall  not be  set to  “GCR-SP”.

   The  GCR  Request  subelement  specifies  the  retransmission  policy  and  delivery  method

requested by the STA for the group addressed stream. The Retransmission Policy field shall  not  be  set to  “No  Preference”.  The Delivery  Method field shall  not be  set to  “No Preference”.

A STA shall not request transmission of a GCR group address via the GCR service while it has an active DMS service for this group address. A STA shall not request transmission of a group address via DMS while it has an active GCR service for this group address.

An AP or mesh STA accepts a GCR request by sending a DMS Status field with  the Status field set to “Accept” as described in 11.22.15.1 with the following modifications:

   The DMS Status field shall include a GCR Response subelement indicating the retransmission policy and delivery method and GCR Concealment Address for the group addressed stream. The Retransmission  Policy field shall not be set to “No Preference”. The Delivery Method field shall not be set to “No Preference”. The GCR Concealment Address field of the GCR Response subelement shall be set to dot11GCRConcealmentAddress. In a mesh BSS, the Delivery Method field shall not be  set to  “GCR-SP”.

   If the GCR group address stream is subject to the GCR-SP delivery method, then the AP shall also include a Schedule element in the DMS Status field indicating the wake-up schedule for the group address stream.

For each GCR Request subelement, the AP or a mesh STA may adopt the requested retransmission policy and delivery method, maintain its existing retransmission policy and delivery method, select an alternate retransmission policy and delivery method or deny GCR service for the group addressed stream.

In an infrastructure BSS, the retransmission policy shall not be GCR-Block-Ack for a GCR group address while the AP has a GCR agreement for the group address with a non-AP STA that had the Advanced GCR field set to 0 in the Extended Capabilities element in the (Re)Association Request most recently received by the AP.

In a mesh BSS, the retransmission policy shall not be GCR-Block-Ack for a GCR group address while the mesh STA has a GCR agreement for the group address with a peer mesh STA that had the Mesh Advanced GCR field set to 0 in the Extended Capabilities element.

An AP or a mesh STA denies a GCR request by sending a DMS Status field  with  the Status  field  set to “Deny” as described in 11.22.15.1 with the following modifications:

   The DMS Status field shall include an empty GCR Response subelement

The AP shall not reject a Reassociation Request for the reason that one or more GCR Service requests are denied.

If  a STA requesting GCR service determines that one or more GCR Response subelements are unacceptable, then the STA shall discard any received ADDBA request frames for the unacceptable GCR streams and the  STA shall send a new DMS Request frame containing a DMS Request element with one DMS Descriptor for each unacceptable GCR stream. The DMSID fields shall be set to the DMSIDs of the unacceptable  streams and the Request Type field shall be set to “Remove”.

In an infrastructure BSS, if the non-AP STA accepts the GCR Response, it shall set dot11GCRConcealmentAddress to the value contained in the GCR Concealment Address field of the GCR Response subelement.

In a mesh BSS, if a STA requesting GCR service accepts the GCR Response, it shall  add to dot11GroupAddressesTable the value contained in the GCR Concealment Address field of the GCR Response subelement.

In a mesh BSS, a GCR agreement instance is identified by a GCR agreement instance identifier. The mesh GCR agreement instance consists of the DMSID, localMAC, peerMAC, and Concealment address.

For each group addressed stream requested by the non-AP STA, the AP shall immediately initiate a Block Ack negotiation if all the following conditions are true:

   The AP advertised an Advanced GCR field set to 1 in its Extended Capabilities element

   The non-AP STA advertised an Advanced GCR field set to 1 in the Extended Capabilities element in the Reassociation Request most recently received by the AP.

For each group addressed stream requested by the a mesh STA, the peer  mesh STAshall immediately initiate a Block Ack negotiation if both the mesh STAs advertised a Mesh Advanced GCR field set to 1 in their Extended Capabilities element in their most recently received mesh Beacon.

If all the above conditions are true the AP or a mesh STA shall immediately initiate a Block Ack negotiation by sending an ADDBA Request frame to the STA that originated the GCR request. The Block Ack Policy field in the Block Ack Parameter field within the ADDBA frames shall not be set to 0 (for delayed Block Ack).  STAs shall maintain this Block Agreement for the duration of their GCR agreement, irrespective of whether the GCR-Block-Ack is the current retransmission policy or not. While the retransmission policy of the GCR group address stream is DMS, theSTA receiving GCR frames shall suspend its Block Ack processing for the group addressed stream.

NOTE: Having a Block Ack agreement with all members of a GCR group address allows the AP or mesh STA to change the GCR retransmission policy dynamically irrespective of the current GCR retransmission policy.

A GCR agreement between a non-AP STA and an AP or between peer mesh STAs shall begin when the  STA providing GCR service successfully transmits an individually addressed DMS Response frame with a DMS Response element containing a DMS Status field that has the Status field set to “Accept” as described in 11.22.15.1 with the following  modification:

-   The DMS Status field shall include a GCR Response subelement

11.22.15.aa2.4 GCR Frame Exchange Procedures

In an infrastructure BSS, a GCR-Block-Ack agreement exists between a non-AP STA and an AP for a group addressed stream from when the non-AP STA successfully transmits an ADDBA Response frame until either the AP or non-AP STA successfully transmits a DELBA frame to the other party, or this GCR-Block-Ack agreement expires (see 9.10.5), or the GCR agreement no longer exists.

In a mesh BSS, a GCR-Block-Ack agreement exists between a mesh STA and its peer mesh STA for a group addressed stream from when the mesh STA successfully transmits an ADDBA Response frame to the peer mesh STA until either the mesh STA or the peer mesh STA successfully transmits a DELBA frame to the other party, or this GCR-Block-Ack agreement expires (see 9.10.5), or the GCR agreement is terminated.

An AP or a mesh STA may transmit a group address stream via the No-Ack/No-Retry (non-GCR; see 9.2.7) service and GCR service simultaneously. Each frame shall be transmitted via the No-Ack/No-Retry retransmission policy before it  is transmitted via the GCR service. A STA providing GCR service may switch dynamically between the GCR-Unsolicited-Retry, GCR-Block-Ack or GCR-Unsolicited-Retry delivery modes, but only one delivery mode may be active at any given time for each GCR group address.

An AP or mesh STA shall transmit a frame belonging to a group address via the GCR service if an associated non-AP STA or a peer mesh STA has a GCR agreement for the group address, and otherwise does not transmit the frame via the GCR service.

In an infrastructure BSS, an AP shall transmit a frame belonging to a group address via the No-Ack/No-Retry service if:

 There is at least one non-AP STA within the BSS with dot11RobustAVStreamingImplemented equal to false or without a GCR agreement for the group address, and

   Either

o	The group address is the broadcast address or

o	The group address is not the broadcast address and at least one of these non-AP STAs has been determined by the AP to be a member of the group address. How this determination is made is out of scope of this standard.

In a mesh BSS, a mesh STA providing GCR service shall transmit a frame belonging to a group address via the No-Ack/No-Retry service if:

- The group address is the broadcast address, or

- The group address is not the broadcast address and at least one peer mesh STA has the Mesh Robust AV Streaming bit set to 0 in the Extended Capabilities element of the STA’s most recent mesh Beacon and has been determined to be a member of the group address (how this determination is made is out of scope of this standard), or

- The group address is not the broadcast address and at least one peer mesh STA has a Block-Ack agreement for the group address and the frame precedes the start of the Block Ack agreement (the sequence number of the frame is less than the starting sequence number of the block Ack agreement, as described in 9.10.2).

To avoid undetected retries being passed up at a receiver‘s  MAC-SAP, duplicate detection and removal for group addressed frames is required in STAs with dot11RobustAVStreamingImplemented or  dot11MeshRobustAVStreamingImplemented set to true (see 9.2.9).

GCR frames shall be QoS data frames (with QoS subfield of the Subtype field set to 1).

If the Block Ack agreement is successfully established for the group addressed stream and the delivery method  for  the  group  addressed  stream  is  GCR-SP,  then  the  non-AP  STAensures  it  is  awake  for subsequent SPs (see 11.22.15.aa2.8).

A STA may request a change of GCR service for a grouped addressed stream by sending a DMS Descriptor  with the DMSID identifying   the group address and the  Request  Type  set to  “Change”  as described in 11.22.15.1 with the following modifications:

   The DMS Descriptor shall contain  zero TCLAS elements, zero TCLAS Processing elements,

one TSPEC element and one GCR Request subelement.

   The TSPEC element and GCR Request subelement of this DMS Descriptor shall together

contain at least one field that is different from the original TSPEC element and GCR Request

subelement identified by the DMSID

The AP or a mesh STA may update the retransmission policy, delivery method, and schedule as the size of the group changes, the capabilities of the members of the group change, GCR Request subelements for the group are received, Multicast Diagnostics or for any other reason. The AP or a mesh STA advertises the current settings upon a change and periodically by:

   Transmitting an unsolicited DMS Response frame with the current settings addressed to the

broadcast address. This DMS Response frame shall be scheduled for delivery at the appropriate

DTIM interval or SP where all STAs within the group are awake to receive the frame. One TCLAS element, one TSPEC element and one GCR Subselement shall be included per DMS Descriptor in the DMS Response element of the DMS Response frame to identify each GCR stream. The DMSID that identifies the GCR stream shall be included the DMS Descriptor. ach Status field in the DMS Status fields included in the frame shall be set to GCR Advertise.

   Transmitting an unsolicited DMS Response frame with the current settings addressed to the

GCR  group  address.  This  DMS  Response  frame  shall  be  scheduled  for  delivery  at  the appropriate DTIM interval or SP where allSTAs within the group are awake to receive the frame. One TCLAS element, one TSPEC element and one GCR Subselement shall be included per DMS Descriptor in the DMS Response element of the DMS Response frame to identify each GCR stream. The DMSID that identifies the GCR stream shall be included the DMS Descriptor. Each Status field in the DMS Status fields included in the frame shall be set to GCR Advertise.

   Transmitting unsolicited DMS Response frames with the current settings individually addressed

to each GCR group member. The DMSID shall be included in per DMS Descriptor in the DMS

Response element of the DMS Response frame to identify each GCR stream.   No TCLAS

element,  no  TSPEC  element  and  no  GCR  Subselement  shall  be  included  in  these  DMS

Descriptors. Each Status field in the DMS Status fields included in the frame shall be set to

GCR Advertise.

STAs receiving GCR frames shall recover from missing group addressed GCR Response frames that advertise a changed retransmission policy or delivery method according to Table 11-aa1 or Table 11-aa2, respectively.

Table 11-aa1:STA recovery procedures for a changed retransmission policy

| Current retransmission policy state at STA receiving GCR frames | Actual retransmission policy being used by the AP or a mesh STA providing GCR service | Recovery procedure |
| --- | --- | --- |
| GCR-Unsolicited-Retry or GCR-Block-Ack | No-Ack/No-Retry | A STA receiving GCR frames cancels the GCR service for the group address when no frames for the group address are received via the GCR service after a period of dot11GCRPolicyChangeTimeout |
| DMS | GCR-Unsolicited-Retry or GCR- Block-Ack | A STA receiving GCR frames shall update its current retransmission policy of the GCR stream to GCR-Unsolicited- Retry upon receiving an MSDU for the DMS group address concealed via the GCR Concealment address. |
| GCR-Unsolicited-Retry or GCR-Block-Ack | DMS | A STA receiving GCR frames shall update its current retransmission policy of the GCR stream to DMS upon receiving an A-MSDU with the RA field set to the STA‘s individual address and the DA field of the A-MSDU subframe set to the GCR group address. |
| GCR-Unsolicited-Retry | GCR-Block-Ack | A STA receiving GCR frames shall update its current retransmission policy of the GCR stream to GCR-Block-Ack upon receiving a BlockAckReq frame with a GCR Group Address subfield set to the GCR group address |
| GCR-Block-Ack | GCR-Unsolicited-Retry | A STA receiving GCR frames shall update its current retransmission policy of the GCR stream to GCR-Unsolicited- Retry if MSDUs for the GCR group address concealed via the GCR Concealment address are being received yet no BlockAckReq frames for the GCR group address are received when the block ack agreement timeout occurs. |

Table 11-aa2: Non-AP STA recovery procedures for a changed delivery method

| Current delivery method state at non-AP STA | Actual delivery method being used by the AP | Recovery procedure |
| --- | --- | --- |
| Non-GCR-SP | GCR-SP | A non-AP STA shall update the current delivery method state of the |
|  |  | GCR stream to GCR-SP if a) no frames with the More field set to 1 for the GCR stream are received for a period of dot11GCRPolicyChangeTi meout, and b) at least one frame for the GCR stream with the More field set to 0 is received. Note that upon detecting condition a), the STA should enter the Awake state in order to assist with detecting condition b). |
| GCR-SP | Non-GCR-SP | A non-AP STA shall update the current delivery method of the GCR stream to Non-GCR-SP if a) no frames with the More field set to 0 for the GCR stream are received for a period of dot11GCRPolicyChangeTi meout, and b) at least one frame for the GCR stream with the More field set to 1 is received. |

A GCR agreement between a non-AP STA and an AP or between peer mesh STAsshall end as described in 11.22.15.1 when:

   In an infrastructure BSS, the AP deauthenticates or disassociates the non-AP STA, or

- . In a mesh BSS, the mesh STA providing GCR service tears down the peer link to the mesh STA receiving GCR frames, or

   The non-AP STA or the mesh STA receiving GCR frames successfully transmits a DMS Request frame to the AP or mesh STA providing GCR service containing a DMS Request element that has a DMS Descriptor with the DMSID identifying the group addressed stream and the Request Type field set to “Remove”, or

   The AP or a mesh STA providing GCR service successfully transmits an individually addressed DMS Response frame with a DMS Response  element  containing  a  DMS  Status  field  with  the  DMSID  identifying  the  group addressed stream that has the Status field set to “Terminate”

A GCR agreement between a non-AP STA and an AP or between peer mesh STAs shall end as described in 11.22.15.1 with the following modifications:

   The DMS Status field shall include a GCR Response subelement

   The DMS response frame may instead by transmitted to the broadcast or GCR group addresses

A cancellation of a GCR agreement shall also cause the Block Ack agreement to be cancelled for the GCR stream.

11.22.15.aa2.5 Concealment of GCR transmissions

Concealment prevents group addressed frames transmitted via the GCR-Unsolicited-Retry or GCR-Block- Ack retransmission policies from being passed up the MAC-SAP of GCR-incapable STAs.

GCR   group   addressed   MSDUs   transmitted   via   the   GCR-Unsolicited-Retry   or   GCR-Block-Ack retransmission policies shall be sent in an A-MSDU frame format with the address 1 field set to the GCR Concealment address dot11GCRConcealmentAddress. The DA field in the A-MSDU subframe shall contain the group address of the GCR group address that is being concealed (i.e. the same value as the DA field for non-GCR group addressed delivery).

A STA with dot11RobustAVStreamingImplemented or dot11MeshRobustAVStreamingImplemented set to true shall not use its GCR Concealment address for any purpose other than the transmission of GCR streams.

A STA with dot11RobustAVStreamingImplemented or dot11MeshRobustAVStreamingImplemented set to true and a GCR agreement shall add the GCR Concealment address from the GCR response subelement to the STA‘s dot11GroupAddressesTable.

The Individual/Group (I/G) address bit (LSB of octet 0) and the Universally or Locally administered (U/L) bit (the bit of octet 0 adjacent to the I/G address bit.) of dot11GCRConcealmentAddress shall both be set.

11.22.15.aa2.6 GCR-Unsolicited-Retry

A	STA	supports	the	GCR-Unsolicited-Retry	retransmission	policy	if dot11RobustAVStreamingImplemented or dot11MeshRobustAVStreamingImplemented is true; otherwise the STA does not support the GCR service with retransmission policy equal to GCR-Unsolicited-Retry.

An AP or a mesh STA adopting the GCR-Unsolicited Retry retransmission policy for a GCR group address chooses a lifetime limit for the group address. The AP or a mesh STA may vary the lifetime limit for the group address at any time, and may use lifetime limits for different GCR group addresses. An AP or a mesh STA adopting the GCR-Unsolicited- Retry  retransmission  policy  for  a  GCR  group  address  shall  transmit  each  MSDU  according  to 11.22.15.aa2.5,  subject  to  the  lifetime  limit.  Transmission uses the backoff procedure described in 9.9.1.6.aa1.

If a Block Ack agreement has successfully been established for a group addressed stream that is delivered using the  GCR-Unsolicited-Retry retransmission  policy,  the  STA shall follow the  duplicate detection procedures defined in 9.2.9 and 9.10.4.

11.22.15.aa2.7 GCR-Block-Ack

A STA supports the GCR-Block-Ack retransmission policy if both dot11RobustAVStreamingImplemented and  dot11GCRImplemented	are  true or both dot11MeshRobustAVStreamingImplemented and  dot11MeshGCRImplemented	are  true;  otherwise  the  STA  does  not  support  the  GCR  service  with retransmission policy equal to GCR-Block-Ack.

GCR Buffer Size for a group address is defined to equal to the minimum Buffer Size field in the Block Ack Parameter Set field in the last received ADDBA.response for that group address across members of the GCR group (see 9.10.aa10).

Annex D

(normative)

ASN.1 encoding of the MAC and PHY MIB

Change the end of the “Dot11StationConfigEntry” of the “dotStationConfig TABLE” as follows:

dot11DependentSTAType 					INTEGER,

dot11RobustAVStreamingImplemented 			TruthValue,

dot11GCRImplemented 					TruthValue,

dot11GCRActivated 					TruthValue,

dot11SCSImplemented 					TruthValue,

dot11SCSActivated 					TruthValue,

dot11QLoadReportActivated 				TruthValue,

dot11QLoadReportIntervalDTIM 				INTEGER,

dot11AlternateEDCAActivated 				TruthValue,

dot11HCCATXOPNegotiationActivated 			TruthValue,

dot11HCCATXOPBeaconTimeout 				INTEGER,

dot11GCRConcealmentAddress 				MacAddress,

dot11GCRPolicyChangeTimeout 				INTEGER,

dot11GCRGroupMembershipAnnouncementActivated 	TruthValue,

dot11MeshGCRImplemented					Truthalue,

dot11MeshRobustAVStreamingImplemented 		TruthValue,

dot11MeshGCRActivated					TruthValue.

}

dot11MeshGCRImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This is a capability variable.

Its value is determined by device capabilities.

This attribute, when TRUE, indicates that the mesh station

implementation supports the Advanced GCR features”

DEFVAL { false }

::= { dot11StationConfigEntry aa14 }

dot11MeshGCRActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This is a control variable.

It is written by the SME or external management entity.

Changes take effect for the next MLME-START.request primitive

or MLME-JOIN.request primitive

This attribute, when TRUE, indicates that the mesh station

implementation supports the GCR procedures as defined in 11.22.15.aa2 	and

that this has been activated.”

DEFVAL { false }

::= { dot11StationConfigEntry aa15 }

dot11MeshRobustAVStreamingImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This is a capability variable.

Its value is determined by device capabilities.

This attribute, when TRUE, indicates that the mesh station

implementation supports Mesh Robust AV Streaming”

DEFVAL { false }

::= { dot11StationConfigEntry aa16 }

Annex A

(normative)

Protocol Implementation Conformance Statement (PICS)

proforma

A.2 Abbreviations and special symbols

A.2.2 General abbreviations for Item and Support columns

Insert one new list item at the end of A.2.2 as indicated below:

AVT audio video transport

A.4 PICS proforma–IEEE Std. 802.11, 2007 Edition

A.4.3 IUT configuration

Insert this entry to the end of the IUT configuration table:

| Item | IUT configuration | References | References | Support |
| --- | --- | --- | --- | --- |
| *Cfaa | Is RobustAVT supported? | 5.2.aa12 | (CF12):O | Yes, No, N/A |

Insert this new clause at the end of A.4:

A.4.aa1 RobustAVT extensions

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| AVT1 | Extended Capabilities information element | 7.3.2.27 | CFaa:M | Yes, No, N/A |
| AVT2 AVT2.1 | Groupcast with Retries (GCR) Advanced GCR | 9.2.7.3 7.3.2.27, 11.22.15.aa2.6 11.22.15.aa2.7 11.22.15.aa2.8, 9.10.aa10 | (CF16 and CFaa): M (CFaa and QB5): O | Yes, No, N/A Yes, No, N/A |
| AVT3 | Alternate EDCA transmit queues | 9.1.3.1 | CFaa:O | Yes, No, N/A |
| AVT4 AVT4.1 AVT4.2 AVT4.3 | Stream Classification Service SCS Request frame SCS Request frame SCS Response frame DEI | 11.aa23.2 7.4.aa13.1 7.4.aa13.211.aa2 3.2 | CFaa:O AVT4:M AVT4:M (CF16 and AVT4):M | Yes, No, N/A Yes, No, N/A Yes, No, N/A Yes, No, N/A |
| ATV5 | OBSS Management | 11.aa247.3.2.27 | (CF12):M | Yes, No, N/A |
| ATV6 AVT6.1 AVT6.2 AVT6.3 | QLoad Report QLoad Report element QLoad Request frame QLoad Report frame | 11.aa24 7.3.2.aa93 7.4.7.aa18 7.4.7.aa19 | (AVT5 and CF1):M AVT6:M AVT6:M AVT6:M | Yes, No, N/A Yes, No, N/A Yes, No, N/A Yes, No, N/A |
| AVT7 AVT7.1 | HCCA TXOP Advertisement element HCCA TXOP Negotiation | 7.3.2.aa94, 11.aa24.311.aa24 | (AVT5:M and QP1):M AVT7:O | Yes, No, N/A Yes, No, N/A |
| AVT8 | Groupcast with Retries (GCR) for mesh | 7.3.2.27, 11.22.15.aa2.6 11.22.15.aa2.7 9.10.aa10 | (CF16 and CFaa and CF2a): O | Yes, No, N/A |