IEEE P802.11
Wireless LANs

| Destination Address in Congestion Notification |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-12-08 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Michael Bahr | Siemens AG, Corporate Technology | Otto-Hahn-Ring 6 80200 München, Germany |  | bahr et siemens dod com |
| Barbara Staehle | University of Würzburg Institute of Computer Science Chair of Computer Networks | Am Hubland 97074 Würzburg |  | bstaehle et informatik dod uni-wuerzburg dod de |
| Dirk Staehle | University of Würzburg Institute of Computer Science Chair of Computer Networks | Am Hubland 97074 Würzburg |  | dstaehle et informatik dod uni-wuerzburg dod de |
| Dan Harkins | Aruba Networks | 1322 Crossman ave, Sunnyvale, California |  | dharkins et arubanetworks dod com |

Abstract

This submission provides text for a simple extension in the format of the Congestion Notification element that allows a more flexible use of the Congestion Notification element. Especially, it provides a simple and easy remedy to a possible situation in intra-mesh congestion control where data flows are unnecessarily blocked (cf. 11-10/1429).

# Resolved Comments

CID 28: Counter: Simple extensions for more flexibility provided in submission 11-10/1428r0

CID 85: Counter: Simple extensions for more flexibility provided in submission 11-10/1428r0

CID 204: Counter: Editorial proposed resolutions of commenter are accepted. Refined text is contained in submission 11-10/1428r0, which also contains the editorial changes.

CID 205: Accept: Also contained in submission 11-10/1428r0

# Normative Text of Submission

Instruction to Editor: Change the given clauses as indicated by the WinWord change marks. All changes based on D7.03

7.3.2.0a General

| Table 7-26—Element IDs |  |  |  |
| --- | --- | --- | --- |
| Element | Element ID | Total length of element in octets including the Type and Length octets | Extensible |
| Congestion Notification (see 7.3.2.99) | 116 | 16 | Yes |

7.3.2.99 Congestion Notification element

The Congestion Notification element is used to indicate the congestion status of the mesh STA per mesh destination and AC, and the duration for which the STA expects the congestion to last. The format of the Congestion Notification element is shown in Figure s7-95o135 (Congestion Notification element format). The Congestion Notification element is included in Congestion Control Notification frames as described in 7.4.15.6 (Congestion Control Notification frame format).

| Element ID | Length | Destination Mesh STA Address | Congestion Notification Duration (AC_BK) | Congestion Notification Duration (AC_BE) | Congestion Notification Duration (AC_VI) | Congestion Notification Duration (AC_VO) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: 1 | 1 | 6 | 2 | 2 | 2 | 2 |  |
| Figure 7-95o135—Congestion Notification element format |  |  |  |  |  |  |  |

The Element ID is set to the value given in Table 7-26 (Element IDs) for this element.

The Length field is set to 14.

The Destination Mesh STA Address field is represented as a 48-bit MAC address and is set to the address of the mesh destination for which the intra-mesh congestion control shall be applied. It is set to the broadcast address if the intra-mesh congestion control shall be applied to all destinations.

The element contains four Congestion Notification Duration fields for the four EDCA access categories to indicate the estimated congestion duration per AC at the mesh STA transmitting the congestion notification. The congestion notification duration values are encoded as unsigned integers in units of 100 µs.

7.4.15.6 Congestion Control Notification frame format

A mesh STA uses the Congestion Control Notification frame to indicate its congestion status to its neighbor peer mesh STA(s). This frame is transmitted using individual addresses or group addresses. The format of the Congestion Control Notification frame Action field is shown in Table s7-57v35 (Congestion Control Notification frame Action field).

| Table 7-57v35—Congestion Control Notification frame Action field |  |  |
| --- | --- | --- |
| Order | Information | Notes |
|  | Category |  |
|  | Mesh Action |  |
|  | Congestion Notification element | One or more Congestion Notification elements (7.3.2.99 (Congestion Notification element)) |

The Category field is set to the value in Table 7-24 (Category values) for category Mesh Action.

The Mesh Action field is set to the value in Table s7-57v30 (Mesh Action field values) representing Congestion Control Notification.

The Congestion Notification element is set as described in 7.3.2.99 (Congestion Notification element).

10.3.79.1.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME- MBSSCongestionControl.request(

peerMAC,

CongestionNotification,

VendorSpecificInfo

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| peerMAC | MAC Address | Any valid individual MAC address | Specifies the address of the peer MAC entity to which the Congestion Control Notification is sent. |
| CongestionNotification | A set of Congestion Notification elements | As defined in 7.3.2.99 (Congestion Notification element) | Congestion notification information generated by the mesh STA. |
| VendorSpecificInfo | A set of elements | As defined in 7.3.2.26 | Zero or more elements. |

10.3.79.3.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME- MBSSCongestionControl.indication(

peerMAC,

CongestionNotification,

VendorSpecificInfo

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| peerMAC | MAC Address | Any valid individual MAC address | Specifies the address of the peer MAC entity from which the Congestion Control Notification was received. |
| CongestionNotification | A set of Congestion Notification elements | As defined in 7.3.2.99 (Congestion Notification element) | Congestion notification information contained in the received Congestion Control Notification frame. |
| VendorSpecificInfo | A set of elements | As defined in 7.3.2.26 | Zero or more elements. |

11C.11 Intra-mesh congestion control

11C.11.1 General

Intra-mesh congestion control is based on the following three main mechanisms:

- Local congestion monitoring and congestion detection

- Congestion control signaling

- Local rate control

At any given time, there is only one congestion control protocol active in a particular MBSS, signalled in the Congestion Control Mode Identifier field of the Mesh Configuration element. This standard specifies the Congestion Control Signaling Protocol that shall be available in any MBSS with an activated congestion control.

NOTE—This standard allows for inclusion of more advanced or alternative congestion control schemes through the Congestion Control Mode Identifier in the Mesh Configuration element.

11C.11.2 Congestion Control Signaling Protocol

The Congestion Control Signaling Protocol specifies the signaling messages used with intra-mesh congestion control. Specific algorithms for local congestion monitoring and congestion detection are beyond the scope of the Congestion Control Signaling Protocol.

The Congestion Control Signaling Protocol  is triggered after congestion is detected at a mesh STA. A mesh STA that detects congestion, and the  traffic destination  causing this congestion,  may transmit a Congestion Control Notification frame to the mesh STAs of the traffic source and other neighboring mesh STAs.. The frame contains one or more Congestion Notification elements, each of which specifies the traffic destination causing the congestion and the expected duration of the congestion per AC per mesh destination as estimated by the congested mesh STA.

Upon receipt of a Congestion Notification frame a mesh STA may stop forwarding, or reduce the rate of forwarding,  traffic to the destinations listed in the Congestion Control Notification elements via the mesh STA reporting congestion for the duration specified in the Congestion Control Notification element. It may also send its own Congestion Control Notification frame to mesh STAs that are the source of the reported congestion, and other neighboring mesh STAs. Any time difference between receipt of the original Congestion Control Notification frame and the transmission of this new Congestion Control frame should be reflected in the duration indicated in the new Congestion Control Notification in such a way that any timers set by mesh STAs in response to the first report of congestion for a given destination will all expire at the same time.

If the Destination MAC Address field in a received Congestion Notification element is the group address, it should be interpreted to mean that communication with the transmitter of this frame should be stopped, or reduced, for the duration specified in the Congestion Notification Element. This event should not result in the transmission of a Congestion Notification element with a Destination MAC Address field set to the group address to any neighbor mesh STAs.

When the duration of a traffic congestion report has expired a mesh STA should resume forwarding of traffic destined to the indicated STA via the mesh STA that reported congestion.

NOTE1—Local policies/mechanisms implemented in a mesh STA might be required to ensure timely transmission of the congestion control signaling messages and to avoid transmission of stale messages that can reduce network efficiency.

NOTE2—A mesh STA that receives a Congestion Control Notification frame might choose to adjust its frame rate, defined by the number of transmitted frames per a unit of time, to the sender of the Congestion Control Notification frame in the identified congested AC(s) for the duration specified in the Congestion Notification element. The reduction of the frame rate to a congested mesh STA avoids waste of the mesh resources for transmission of packets that with high probability will not be handled/forwarded by the congested mesh STA.

# References:

11-10/1429r0 Barbara Staehle, Dirk Staehle, Michael Bahr: “Proposed Change to 802.11 Intra-Mesh Congestion Notification Frame”, Dec 2010-12-08

IEEE 802.11s WLAN Mesh Networking, Draft Standard D7.03