IEEE P802.11
Wireless LANs

| Resolution for CID 279 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-05-20 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Dee Denteneer | Philips | HTC 37; 5656 AE Eindhoven; The Netherlands | +31-40-27-49743 | dee.denteneer@philips.com |

Abstract

We provide normative changes to 802.11s/D7.03 to resolve CIDs 279 and 236.

Replace  in Clause 11.1.3.2.1 the paragraph “

In an infrastructure BSS or in an IBSS, STAs receiving Probe Request frames shall respond with a probe response when the SSID in the probe request is the wildcard SSID, matches the specific SSID of the STA, or the specific SSID of the STA is included in the SSID List element. In an MBSS, STAs receiving Probe Request frames shall respond with a probe response when the SSID in the probe request is the wildcard SSID and the Mesh ID in the probe request is the wildcard Mesh ID or matches the specific Mesh ID of the STA. Furthermore, a STA with dot11RadioMeasurementActivated true receiving a probe request with a DS Parameter Set element containing a Current Channel field value that is not the same as the value of dot11CurrentChannelNumber shall not respond with a probe response. Probe Response frames shall be sent as directed frames to the address of the STA that generated the probe request. An AP shall respond to all probe requests meeting the above criteria. In an IBSS, a STA that transmitted a Beacon frame since the last TBTT shall respond to probe requests. The SSID List element shall not be included in a Probe Request frame in an IBSS. In an MBSS, all STAs shall respond to all received probe requests meeting the above criteria.”

With the following text

“In an infrastructure BSS or in an IBSS, STAs receiving Probe Request frames shall respond with a probe response when the SSID in the probe request is the wildcard SSID, matches the specific SSID of the STA, or the specific SSID of the STA is included in the SSID List element. Furthermore, a STA with dot11RadioMeasurementActivated true receiving a probe request with a DS Parameter Set element containing a Current Channel field value that is not the same as the value of dot11CurrentChannelNumber shall not respond with a probe responseAn AP shall respond to all probe requests meeting the above criteria. In an IBSS, a STA that transmitted a Beacon frame since the last TBTT shall respond to probe requests.

In an MBSS, mesh STAs receiving Probe Request frames shall respond with a probe response when the SSID in the probe request is the wildcard SSID and the Mesh ID in the probe request is the wildcard Mesh ID or matches the specific Mesh ID of the mesh STA, except when  dot11RadioMeasurementActivated is true and the  probe request contains a DS Parameter Set element with a Current Channel field value that is not the same as the value of dot11CurrentChannelNumber.

Probe Response frames shall be sent as directed frames to the address of the STA that generated the probe request. The SSID List element shall not be included in a Probe Request frame in an IBSS.”