IEEE P802.11
Wireless LANs

| Some Clarifications related on CID 125 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-12-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | Email |
| Haeyoung Jun | Samsung | 416, Maetan-3Dong, Yeongtong-Gu, Suwon-Si, Gyeonggi-Do, 443-742, Korea |  | haeyoung.jun@samsung.com |
| Hykchoon Kwon | Samsung | 416, Maetan-3Dong, Yeongtong-Gu, Suwon-Si, Gyeonggi-Do, 443-742, Korea |  | Hyukchoon.kwon@samsung.com |
| Hodong Kim | Samsung | 416, Maetan-3Dong, Yeongtong-Gu, Suwon-Si, Gyeonggi-Do, 443-742, Korea |  | hodong0803.kim@samsung.com |
| Sooyeon Jung | Samsung | 416, Maetan-3Dong, Yeongtong-Gu, Suwon-Si, Gyeonggi-Do, 443-742, Korea |  | sooyn.jung@samsung.com |

Abstract

Some clarifications supporting CID 125.

| Comment ID | Commenter Name | Commenter Co | Subclause | Comment Type | Comment | SuggestedRemedy |
| --- | --- | --- | --- | --- | --- | --- |
| 125 | Cordeiro, Carlos | Intel Corporation | General | T | The spec needs some clarifications in different sections | Contribution(s) will be submitted |

-------------------------------------------------------------------------------------------------------------------------------

Clarification 1: Comments on P190L2, 9.24, the equation & sentence on PCP/AP clustering rule has some errors. Clarify the equation.

Proposed Resolution: In P190L2, modify the equation on PCP/AP clustering as follows.

Please change the following clause 9.24 as follows. (Page 190, lines 2 to page 190 lines 12.)

For each cluster, there exists a set of ClusterMaxMem Beacon SPs. The nth Beacon SP, Beacon SPn, begins at ClusterTimeOffsetn-1 usec following the start of each BTT of the S-PCP/S-AP, where:

ClusterTimeOffsetn-1 = ((dot11BeaconPeriod TUs)/ClusterMaxMem)

*(n-1)

and (n=2, 3…, ClusterMaxMem)

The ClusterTimeOffsetn-1 and Beacon SPn where n equals one is reserved for the S-PCP/S-AP.

-------------------------------------------------------------------------------------------------------------------------------

Clarification 2: Comment on P239L1, 9.28, to support independent DTP configuration for separate beamforming links within EL-cluster, some clarification is required.

Proposed Resolution: After P239L3, insert the following sentence, "If Beam link cluster is set to 0, then the links within EL-Cluster which are supporting DTP may maintain independent DTP configuration."

Please change the following clause 9.28 as follows. (Page 239, lines 1 to 3.)

The transmitting STA may send another DTP Request frame to a receiving STA even if it decided not to switch to the DTP configuration indicated by the receiving STA‘s last transmitted, if any, DTP Report frame.

If Beam link cluster is set to 0, then the links within EL-Cluster which are supporting DTP may maintain independent DTP configuration.