IEEE P802.11
Wireless LANs

| Comment resolution part1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 04 December 2010 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Leila Issa, Lai Xu and Ismail Lakkis | Tensorcom |  |  | ilakkis@tensorcom.com |

Abstract

This document provides solution to CID 177 in realtion to the Draft P802.11ad_D1.0 and the data base 11-10-1220-07-00ad-lb168-comment-database

Editor’s instructions:  replace sections 21.7.1.3.2 with the following text:

21.7.1.3.2 Encoding

Data is encoded using an outer RS(224,208) block code and a short inner code. The inner code is a (16,8) block code for MCSs 25 and 28, a (12,8) block code for MCSs 26 and 29, and a (9,8) single parity check block code for MCSs 27 and 30 and the identity block-code for MCS 31. The encoding is further detailed in the following steps:

- First, the number of block padding bits is calculated:

  - The total number of Reed Solomon codewords is  and the total number of RS encoded symbols is given by: ;

  - After Reed Solomon encoding, the data is encoded with a short block code (N,8) where N can take one of the following values 9, 12 or 16 depending on the MCS as shown in Table 72. Therefore:

    - The total number of encoded bits is: ;

    - The total number of 512 (in length) blocks (each containing 392 data symbols) is. The total number of zero block padding bits is: ;

- The data is broken into blocks of length 208×8 bits (except for possibly the last block);

- Each block is encoded using the RS(224,208) block code as described in 21.7.1.3.2.3, except perhaps the last block which is encoded by a shortened version of the code (i.e., if the last block length is K, the shortened block code is RS(16+K,K);

- The Reed Solomon encoded data stream is further encoded using the block code(N,8) as described in 21.7.1.3.2.4, 21.7.1.3.2.5 and 21.7.1.3.2.6 depending on the MCS.

- The coded bit stream is concatenated with zeros. They are scrambled with the continuation of the scrambler sequence that scrambled the PSDU input bits.

- Each set of 7 octets is interleaved using a uniform interleaver of 7 rows and 8 columns. The 7 octets are written row wise and read column wise, i.e., the index  of a bit after interleaving is related to the index  of a bit before interleaving by the following rule: where the function floor(.) denotes the largest integer not exceeding the function parameter.

- The resulting bit stream is modulated then blocked as described in 21.7.1.3.l.3 and 21.7.1.3.4.

21.7.1.3.2.5 (N,8) Block-Coding

Every octet is encoded into an N-bit codeword such that where the generator matrix is given by the following expressions for  respectively:

Replace table 72 with the following:

| MCS | Modulation | Effective Code Rate | Coding Scheme | NCPB | Rate (Mbps) |
| --- | --- | --- | --- | --- | --- |
| 25 | /2-BPSK | 13/28 | RS(224,208)+Block-Code(16,8) | 392 | 626 |
| 26 | /2-BPSK | 13/21 | RS(224,208)+Block-Code(12,8) | 392 | 834 |
| 27 | /2-BPSK | 52/63 | RS(224,208)+SPC(9,8) | 392 | 1112 |
| 28 | /2-QPSK | 13/28 | RS(224,208)+Block-Code(16,8) | 392 | 1251 |
| 29 | /2-QPSK | 13/21 | RS(224,208)+Block-Code(12,8) | 392 | 1668 |
| 30 | /2-QPSK | 52/63 | RS(224,208)+SPC(9,8) | 392 | 2224 |
| 31 | /2-QPSK | 13/14 | RS(224,208) | 392 | 2503 |