IEEE P802.11
Wireless LANs

| Scheduled Change in Channel Availability for TVBDs |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2011-1-4 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Zhou Lan | NICT | 3-4, Hikarino-oka, Yokosuka, Kanagawa, Japan, 239-0847 | +81 45 847 5097 | lan@nict.go.jp |

Abstract

According to the new FCC ruling (FCC-10-174), database must provide information on schedule changes that occur during the 48 hour period following the recheck.  Fixed and mode II devices after obtaining this information shall adjust to these scheduled changes. There is no such mechanism specified in the draft text D0.08. This submission proposes Channel Schedule Management mechanism to meet the new FCC rules for database to provide channel schedule information to TVBDs.

The country specific information is separated from the normative text and moved to Annex E for the purpose of creating a standard that applies not only to FCC but also other regulatory domains.

This submission aligns with D0.08.

Discussion

The new FCC ruling (FCC-10-174 713 (i)) says “A database must provide fixed and Mode II personal portable TVBDs with channel availability information that includes scheduled changes in channel availability over the course of the 48 hour period beginning at the time the TVBDs make a re-check contact.” To meet this requirement, we define the Channel Schedule Management element of RLQP to convey the channel schedule information using RLQP. In addition, we define a general Channel Schedule Management element to share channel schedule information among mode II or fixed devices because the new FCC ruling allows a fixed or mode II device obtain the available channel list from another fixed or mode II device according to FCC-10-174 section 703(f). The Channel Schedule Management element can be transmitted over any media using a secured container. Further, a public action frame and its protected version namely Channel Schedule Management frame is proposed as a third means to enable channel schedule information to be exchanged between mode II and fixed devices that may not be in the same BSS.

The channel schedule information can be delivered in two forms based on TV channel or WLAN channel. A Channel Schedule Management Mode field is introduced to distinguish in which form the information is delivered. If the channel schedule information is based on the WLAN channels, the optional Operating Class information shall be provided.

It is assumed that before using the Channel Schedule Management function, the certification procedure to check the FCC ID for both mode I and mode II device should have been completed. It is also assumed that WSM has already been obtained by the device that wants to use this mechanism to receive channel schedule information.

6. Layer Management

6.3 MLME SAP Interface

TGaf Editor: Insert following new clause after the last sentence of subclause 6.3.af4:

6.3.af5 Channel schedule management

The following MLME primitives support the signaling of channel schedule management.

6.3.af5.1 MLME-CHANNELSCHEDULEMANAGEMENT.request

6.3.af5.1.1 Function

This primitive requests that a (Protected) Channel Schedule Management frame be sent by an STA.

6.3.af5.1.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-CHANNELSCHEDULEManagement.request (

ChannelScheduleManagement,

Protected,

VendorSpecificInfo,

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ChannelScheduleManagement | A set of information subfields | As described in 8.4.2.af4 | As described in 8.4.2.af4 |
| Protected | Boolean | true, false | Specifies whether the request is sent using a Robust Management frame. If true, the request is sent using the Protected Channel Schedule Management frame. If false, the request is sent using the Channel Schedule Management Announcement frame. |
| VendorSpecificInfo | A set of elements | As defined in 8.4.2.28 (Vendor Specific element) | Zero or more elements. |

6.3.af5.1.3 When generated

This primitive is generated by the STA management entity (SME) to request that a (Protected) Channel Schedule Management frame be sent by a STA.

6.3.af5.1.4 Effect of receipt

On receipt of this primitive, the MLME constructs and transmits a (Protected) Channel Schedule Management frame.

6.3.af5.2 MLME-CHANNELSCHEDULEMANAGEMENT.confirm

6.3.af5.2.1 Function

This primitive reports the result of a request to channel schedule management.

6.3.af5.2.1 Semantics of the service primitive

The primitive parameters as follows:

MLME-CHANNELSCHEULEMANAGEMENT.confirm (

ResultCode,

VendorSpecificInfo,

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, INVALID_PARAMETERS, UNSPECIFIED_FAILURE | Reports the result of an channel Schedule management request |
| VendorSpecificInfo | A set of elements | As defined in 8.4.2.28 (Vendor Specific element) | Zero or more elements. |

6.3.af5.2.2 When generated

This primitive is generated by the MLME when a channel schedule request completes.  Possible unspecified failure causes include an inability to provide the channel schedule information.

6.3.af5.2.3 Effect of receipt

The SME is notified of the results of the channel schedule management procedure.

6.3.af4.3 MLME-CHANNELSCHEDULEMANAGEMENT.indication

6.3.af5.3.1  Function

This primitive indicates that a (Protected) Channel Schedule Management frame was received.

6.3.af5.3.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-CHANNELSCHEDULEMANAGEMENT.indication (

PeerMACAddress,

ChannelScheduleManagement,

Protected,

VendorSpecificInfo,

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerMACAddress | MACAddress | Any valid individual MAC address | The address of the peer MAC entity from which the Channel Schedule Management frame was received. |
| ChannelScheduleManagement | A set of information subfields | As described in 8.4.2.af4 | As described in 8.4.2.af4 |
| Protected | Boolean | true, false | Specifies whether the request was received using a Robust Management frame. If true, the request was received using the Protected Channel Schedule Management frame. If false, the request was received using the Channel Schedule Management frame. |
| VendorSpecificInfo | A set of elements | As defined in 8.4.2.28 (Vendor Specific element) | Zero or more elements. |

6.3.af5.3.3 When generated

This primitive is generated by the MLME when a valid (Protected) Channel Schedule Management frame is received.

6.3.af5.3.4 Effect of receipt

On receipt of this primitive, the SME decides whether to provide the channel schedule information.

6.3.af5.4 MLME-CHANNELSCHEDULEMANAGEMENT.response

6.3.af5.4.1  Function

This primitive is used to provide channel schedule information on channel availability.

6.3.af5.4.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-CHANNELSCHEDULEMANAGEMENT.response (

ChannelScheduleManagement,

VendorSpecificInfo,

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ChannelScheduleManagement | A set of information subfields | As described in 8.4.2.af4 | As described in 8.4.2.af4 |
| VendorSpecificInfo | A set of elements | As defined in 8.4.2.28 (Vendor Specific element) | Zero or more elements. |

6.3.af5.4.3 When generated

This primitive is generated by the SME to provide the channel schedule information.

6.3.af5.4.4 Effect of receipt

On receipt of this primitive, the MLME provides channel schedule information on channel availability.

8.4 Management frame body components

8.4.2 Information elements

8.4.2.1 General

TGaf Editor: Insert following rows in Table 8-50 Element Ids in the correct position to preserver ordering by the “Element ID” column:

Table 8-50 – Element IDs

| Information Element | Element ID | Length (in octets) | Extensible |
| --- | --- | --- | --- |
| Channel Schedule Management(see 8.4.2.af5) | <ANA> | 3 to 255 |  |

8.4.2.29 Extended capabilities element

TGaf Editor: Add the following rows to Table 8-88 and renumber the reserved values accordingly:

Table 8-88 – Capabilities field

| Bit | Information | Notes |
| --- | --- | --- |
| <ANA> | Channel Schedule Management | The STA sets the Channel Schedule Management field to 1 when the MIB attribute dot11ChannelScheduleManagementActivated is true, and set it to zero otherwise, see 8.4.2.af4. |

TGaf Editor: Insert following new clause after the last sentence of subclause 8.4.2.af3:

8.4.2.af4 Channel Schedule Management element

The Channel Schedule Management element is transmitted in the public action frame or any secured container to indicate the channel schedule change.

|  |  |  |  |  | These fields are repeated, as determined by the Length field. |
| --- | --- | --- | --- | --- | --- |
|  | Element ID | Length | Reason Result Code | Channel Schedule Management Mode | Channel Schedule Descriptor |
| Octets: | 1 | 1 | 1 | 1 | Variable |

Figure 8-42af4 – Channel Schedule Management element format

The Length field indicates the length of the remaining fields in octets, and the value is variable.

The Reason Result Code field indicates the reason of generating the Channel Schedule Management for a query request. It also indicates the result of a response whether the query is successful or not, and the reason if it is not successful. The value of this field is defined in Table 8-42af41.

Table 8-42af41 – Reason Result Code filed values

| Reason Result Code field values | Description |
| --- | --- |
| 0 | Requesting for a new channel schedule information from TVWS database. It happens when the first time STAs try to obtain channel schedule information from TVWS database. |
| 1 | Requesting for an updated channel schedule information from TVWS database. It happens when the STAs change locations and have to re-contact the TVWS database for updated channel schedule information. |
| 2 | Successful with channel schedule information obtained |
| 3 | Successful with no channel schedule changes from last time querying the TVWS database |
| 4 | Request declined by STAs |
| 5 | Request declined by TVWS database |
| 6 | Timeout |
| 7-256 | Reserved |

The Channel Schedule Management Mode field indicates if the schedule information of the channel availability is based on TV channel or WLAN channel. If the schedule information is based on WLAN channel, the optional Operating Class shall be present in the Channel Schedule Descriptor as defined in Annex E. This field also indicates whether the optional Channel Availability Starting Time is presented in the Channel Schedule Descriptor or not. The value of this field is defined in Table 8-42af42

Table 8-42af42 – Channel Schedule Management Mode values

| Channel Schedule Management Mode value | Description |
| --- | --- |
| 0 | The Channel schedule information is based on the TV channel. Only Channel number fields shall be presented in the Channel Schedule Descriptor. The Reason and Result Code field shall be set to either zero or one. |
| 1 | The Channel Schedule information is based on the WLAN channel. Only Operating Class and Channel number fields shall be presented in the Channel Schedule Descriptor. The Reason and Result Code field shall be set to either zero or one. |
| 2 | The Channel Schedule information provided is based on the TV channel. The Operating Class shall not be present in the Channel Schedule Descriptor. Channel Availability Starting Time shall be presented in the Channel Schedule Descriptor and the time that the channel becomes available shall be provided. |
| 3 | The channel Schedule information provided is based on the WLAN channel. The optional Operating Class field shall be present in the Channel Schedule Descriptor. The Channel Availability Starting Time shall be presented in the Channel Schedule Descriptor and the time that the channel becomes available shall be provided. |
| 4 | The Channel Schedule information provided is based on the TV channel. The optional Operating Class field shall not be present in the Channel Schedule Descriptor. The Channel Availability Starting Time shall NOT be presented in the Channel Schedule Descriptor. |
| 5 | The channel Schedule information provided is based on the WLAN channel. The optional Operating Class field shall be present in the Channel Schedule Descriptor. The Channel Availability Starting Time filed shall NOT be presented in the Channel Schedule Descriptor. |
| 6 – 255 | Reserved. |

8.4.5 Registered Location Query Protocol elements

TGaf Editor: Modify and add the following rows to Table 8-45af1 and renumber the reserved values accordingly:

Table 8-45af1 – Registered Location Query Protocol info ID definitions

| Info Name | Info ID | RLQP Info element (clause) |
| --- | --- | --- |
| Channel Schedule Management | 5 | 8.4.5.5 |
| Reserved | 56 – 220 | N/A |
| Vendor Specific | 221 | 8.4.2.28 |
| Reserved | 222 – 255 | N/A |

TGaf Editor: Insert the following new subclause after 8.4.5.2, as shown:

8.4.5.5 Channel Schedule Management element

The Channel Schedule Management element of RLQP is used by an enabling STA to query the database for channel schedule information.

|  |  |  |  |  |  |  | These fields are repeated, as determined by the Length field. |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Info ID | Length | Requestor STA address | Requestor STA address | Reason Result Code | Channel Schedule Management Mode | Channel Schedule Descriptor |
| Octets: | 1 | 1 | 1 | 1 | 1 | 1 | Variable |

Figure 8-455 – Channel Schedule Management element format

The Info ID field is set to the value for Channel Schedule Management defined in Table 8-45af1.

The Length field indicates the length of the remaining element fields in octets, and the value is variable.

The Requester STA Address field is the MAC address of the requesting STA that initiates the Channel Schedule Management process.

The Responder STA Address field is the MAC address of the responding STA that grants Channel Schedule Management process.

The remaining fields are as defined in the Channel Schedule Management element (see section 8.4.2.af4).

8.5 Action frame format details

8.5.8 Public Action frame details

8.5.8.1 Public Action frames

TGaf Editor: Insert the following rows in Table 8-131:

Table 8-131 – Public Action field values

| Action field value | Description |
| --- | --- |
| <ANA> | Channel Schedule Management |

8.5.11 Protected Dual of Public Action frames

8.5.8.1 Protected Dual of Public Action details

TGaf Editor: Insert the following rows in Table 8-143:

Table 8-143 – Protected Dual of Public Action field values

| Action field value | Description |
| --- | --- |
| <ANA> | Protected Channel Schedule Management |

TGaf Editor: Before clause 9, insert text as shown:

Insert the following new clause after the last subclause of 8.5.8.af5:

8.5.8.af6 Channel Schedule Management frame format

The Channel Schedule Management frame is a Public Action frame. It is transmitted by a STA to exchange the channel schedule information as part of Channel Schedule Management procedure. The format of the Channel Schedule Management frame body is shown in Figure 8-58af6.

|  |  |  |  |  |  |  |  | These fields are repeated, as determined by the Length field. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Category | Action Value | Requestor STA address | Requestor STA address | Length | Reason Result Code | Channel Schedule Management Mode | Channel Schedule Descriptor |
| Octets: | 1 | 1 | 1 | 1 | 1 | 1 | 1 | Variable |

Figure 8-58af6 – Channel Schedule Management frame format

The Category field is set to the value for public action defined in Table 8-36 (Category values).

The Action Value field is set to indicate a Channel Schedule Management frame, as defined in Table 8-131 (Public Action field values) in section 8.5.8.1.

The remaining fields are as described in the Channel Schedule Management element (see 8.3.2.af4 (Channel Schedule Management element).

Insert the following new clause after the last subclause of 8.5.11.7af4:

8.5.11.7af5 Protected Channel Schedule Management frame format

The Protected Channel Schedule Management frame format is the same as the Channel Schedule Management frame format (see 8.5.8.af6).  It is used instead of the Channel Schedule Management frame when Management Frame Protection is negotiated.

10. MLME

TGaf Editor: Insert the following new subclause as 10.af1, as shown:

10.af2 Channel Schedule Management

An enabling STA with dot11ChannelScheduleManagementActivated set true at each required time for checking the database shall use Channel Schedule Management RLQP element to query the database whether there is schedule change on the channel availability.

Each time when an enabling STA with dot11ChannelScheduleManagementActivated set true powers up, it firstly utilizes the Channel Availability Query procedure as defined in 10.12.6.2 to obtain the available channel list with WSM as defined in 8.4.2.af1. Among the listed available channels within WSM, the STA chooses channels and sends query request for these selected channels that it wants to obtain channel schedule information.  When a STA queries Channel Schedule Management frame, it transmits the interested channel numbers within Channel Schedule Descriptor set to Channel Schedule Management mode zero or one. If a STA queries for channel schedule information for each TV Channel, the TV Channel Numbers shall be included in the Channel Schedule Management frame.

When the location of an enabling STA changed, the enabling STA shall use this mechanism to update the channel schedule information. The Channel Availability Starting Time as described in Annex E may not be provided to reduce the overhead of messaging procedure, the time when the response message is received is taken as the starting time of channel availability.

The schedule information exchange may also happen between an enabling STA and a RLS. The RLS may map the channel schedule information from TV channel to WLAN channel and provide this information, as described in Annex E, to facilitate the WLAN operation.

When there is no schedule change of the channel, the Channel Schedule Management element in the query response shall be constructed to indicate there is no schedule change as described in section 8.4.5.5. The query response only contains the schedule change from the last checking of the database.

An enabling STA with dot11ChannelScheduleManagementActivated set true may send Channel Schedule Management element as described in 8.4.2.af4 in Public Action frame to update the channel schedule information of another enabling STA that receive available channel list from it. It may also use Channel Schedule Management frame for the same purpose.

The details of the procedures for Channel Schedule Management requesting STA and Channel Schedule Management Responding STA are provided in subclauses 10.af2.1 and 10.af2.2.

10.af2.1 Channel Schedule Management Requesting STA

A STA employing RLQP may use GAS protocol for its Channel Schedule Management process with a RLS. Upon receipt of the MLME-GAS.request primitive for sending a GAS query request to a peer station for the purpose of Channel Schedule Management, the channel schedule management requesting STA transmits a RLQP element with Info ID for Channel Schedule Management in the Query Request field in a GAS Initial Request frame. The specific information items in the Query Request field of the GAS Initial Request frame are generated as follows.

  - Reason result code= 0 or 1 as described in 8.4.2.af4.

  - Channel schedule management mode=0 or 1 as described in 8.4.2.af4.

  - Channel Schedule Descriptor, as defined in Annex E, contains only the channel that the STA wants to query.

A STA may use Channel Schedule Management frame as defined in 8.5.8.af6 or its protected frame as defined in 8.5.11.7af5 to query another STA for channel schedule information. Upon receipt of the MLME-CHANNELSCHEDULEMANAGEMENT.request primitive for sending a query request to a peer station for the purpose of Channel Schedule Management, the channel schedule management requesting STA transmits a Channel Schedule Management frame. The Request frame is generated as follows.

  - Reason result code= 0 or 1 as described in 8.4.2.af4.

  - Channel schedule management mode=0 or 1 as described in 8.4.2.af4.

  - Channel Schedule Descriptor, as defined in Annex E, contains only the channel that the STA wants to query.

10.af2.2 Channel Schedule Management Responding STA

A STA employing RLQP may use GAS protocol for its Channel Schedule Management process with a RLS. Upon receipt of the MLME-GAS.response primitive for sending a GAS query response to a peer station for the purpose of Channel Schedule Management, the Channel Schedule Management responding STA transmits a RLQP element with Info ID for Channel Schedule Management in the Query Response field in a GAS Initial Response frame or one or more GAS Comeback Response frames. The specific information items in the Query Response field of the GAS Initial Response frame is generated as follows.

  - Reason result code= values other than 0 and 1 as described in 8.4.2.af4.

  - Channel schedule management mode= values other than 0 and 1 as described in 8.4.2.af4.

  - Channel Schedule Descriptor, as defined in Annex E, contains the channel and related channel schedule information

A STA may use Channel Schedule Management frame as defined in 8.5.8.af6 or its protected frame as defined in 8.5.11.7af5 to respond to another STA for channel schedule information. Upon receipt of the MLME-CHANNELSCHEDULEMANAGEMENT.response primitive for sending a query response to a peer station for the purpose of Channel Schedule Management, the channel schedule management responding STA transmits a Channel Schedule Management frame. The Response frame is generated as follows.

  - Reason result code= values other than 0 and 1 as described in 8.4.2.af4.

  - Channel schedule management mode= values other than 0 and 1 as described in 8.4.2.af4.

  - Channel Schedule Descriptor, as defined in Annex E, contains the channel and related channel schedule information

Annex B (normative) Protocol Implementation Conformance Statement (PICS) proforma

B.4 PICS proforma – IEEE Std 802.11-<year>

B.4.af TVWS functions

TGaf Editor: Insert the following row (ignoring the header row) at the end of B.4.af, as shown:

| Item | IUT configuration | References | Status | Support |
| --- | --- | --- | --- | --- |
| ANA | Channel Schedule Management | 8.4.2.af4, 8.4.5.5, 8.5.8.af6, 10.af2 | CFaf:M | Yes ℃ No ℃ N/A ℃ |

Annex C (normative) ASN.1 encoding of the MAC and PHY MIB

Insert following entries at the end of the dot11StationConfigEntry sequence list, as shown:

Dot11StationConfigEntry::=

SEQUENCE {

dot11ChannelScheduleManagementActivated	TruthValue,

}

Insert the following element at the end of dot11StationConfigEntry element definitions, as shown:

dot11ChannelScheduleManagementActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a control variable.

It is written by the SME when the device is initialized for operation in a band defined by an Operating Class.

This attribute, when true, indicates that the station implementation is capable of supporting Channel Schedule Management Announcement.  The capability is disabled, otherwise.”

DEFVAL { false }

::= { dot11StationConfigEntry <ANA> }

Insert the following entry at the end of dot11SMTbase10 object group, as shown:

dot11SMTbase10 OBJECT-GROUP

OBJECTS {

dot11ChannelScheduleManagementActivated }

Annex E

(normative)

Country elements and operating classes

E.af1 TVWS TLVS (Type Length Value Scope)

E.af1.1 Channel Schedule Descriptor

This parameter contains the channel number associated with channel schedule information used for channel schedule management. The Channel Schedule Descriptor Tuple attribute format is shown in Table E-af1 and Table E-af2.

Table E-af1 – Channel Schedule Descriptor tuple definition

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| <ANA> | variable | Compound TLVs in Table E-af2 | AF_CSM, FCC |

Table E-af2 – Channel Schedule Descriptor value field

| Name | Type | Length | Value | Scope |
| --- | --- | --- | --- | --- |
| Operating Class | <ANA>.1 | 1 | The Operating Class field is set to the number of the operating class of the channel, which is defined in Annex E.1. | AF_CSM, FCC |
| Channel Number | <ANA>.2 | 1 | The Channel Number field is set to the number of the channel, which is the subject to the value of Channel Schedule Management Mode field. If the Channel Schedule Management Mode field indicates the schedule information is based on WLAN channel, the channel number is a channel from the STA’s operating class as defined in Annex E.1; otherwise, the Operating Class compound TLV will not be present and the Channel Number is a positive integer value as defined in Annex D.1 to indicate the available TV channel for WLAN operation. | AF_CSM, FCC |
| Channel Availability Starting Time | <ANA>.3 | 8 | The Channel Availability Starting Time field indicates the starting time in Coordinate Universal Time (UTC) from when the channel indicated in the Channel Number field is available for operation. When this field is not presented, the station will take the time that the response element is received as the starting time of the channel availability. | AF_CSM, FCC |
| Channel Availability Duration | <ANA>.4 | 2 | The Channel Availability Duration field indicates the duration in minute of the availability of the channel that indicated in the Channel Number field. | AF_CSM, FCC |