IEEE P802.11
Wireless LANs

LB170 CID #1004 Resolution

Date: 2011-01-05

| Name | Company | Address | Phone | email |
| --- | --- | --- | --- | --- |
| Ganesh Venkatesan | Intel Corp | 2111 NE 25th Ave, Hillsboro, OR | +1-503-334-6720 | Ganesh.Venkatesan@intel.com |

Abstract

This submission contains the normative text related to Resolving LB170 CID #1004. This document is based on 802.11aa Draft 2.01. Editorial instructions are relative to the contents of 802.11aa Draft 2.01.

Insert the following primitive after ADDTSReserve.confirm and [re]number the clauses as appropriate

10.3.4.24.aa MLME-ADDTSReserve.inidication

10.3.4.24.aa.1 Function

This primitive reports to the non-AP STA’s SME the initiation of AP Initiated TS Setup procedure.

10.3.24.aa.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-ADDTSReserve.indication (

APAddress,

TSPEC,

Schedule,

Higher Layer Stream ID,

VendorSpecificInfo)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| AP Address | MAC Address |  | Contains the MAC address of the AP initiating the TS Setup procedure |
| TSPEC | TSPEC element | As defined in 7.3.2.30 (8.4.2.32 in D6.02 Rev mb) | Specifies the QoS parameters of the TS. |
| Schedule | Schedule element | As defined in 7.3.2.34 (8.4.2.36 in D6.02 Rev mb) | Specifies the schedule information, service start time, SI, and the specification interval. |
| Higher Layer Stream ID | Higher Layer Stream ID element | As defined in 7.3.2.aa95 | Identifies the higher layer stream corresponding to which this primitive is invoked. |
| VendorSpecificInfo | A set of information elements | As defined in 7.3.2.26 (VendorSpecific information element) (8.4.2.28 in D6.02 Rev mb) | Zero or more information elements. |

10.3.24.aa.3 When generated

This primitive is generated by the MLME at the non-AP STA as a result of the receipt of a ADDTS Reserve action frame from the AP to which the non-AP STA is associated.

10.3.24.aa.4 Effect of receipt

The SME is notified of the receipt of the ADDTS Reserve action frame from the AP. This primitive solicits the SME to participate in the AP initiated TS Setup procedure.

The SME should operate according to the procedures defined in 11.4.4.2 (AP initiated TS Setup).