IEEE P802.11
Wireless LANs

| MLME MFQ Primitives Update |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2011-01-17 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Stephen McCann | Research in Motion | 200 Bath Road, Slough, Berkshire, SL1 3XE, UK | +44 1753 667099 | smccann@rim.com |

Abstract

This document contains proposed text for an MLME MFQ primitives update based on TGae draft 1.01.

It addresses comments 13, 160, 162, 163, 230, 233, 234, 235, 269, 273, 274, 275, 305, 321, 345, 350, 351 and 446.

Modify the following sub-clauses:

### 6.3.ae1  MFQ Policy

The following MLME primitives support the signaling of MFQ Policy.

#### 6.3.ae1.1 MLME-MFQPOLICYQUERY.request

##### 6.3.ae1.1.1 Function

This primitive supports the signalling of MFQ Policy Queries between peer SMEs. An SME queries the MFQ Policy within a peer SME.

##### 6.3.ae1.1.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-MFQPOLICYQUERY.request (

Peer MAC Address,

Dialog Token,

[SM: typo, not removed from D1.01]

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MAC Address | Any valid individual MAC Address | The address of the peer MAC entity to which the MFQ Policy Query request is sent. |
| Dialog Token | Integer | 1-255 | The dialog token to identify the MFQ Policy Query transaction. |

##### 6.3.ae1.1.3 When generated

This primitive is generated by the SME when a STA wishes to query the MFQ Policy within another STA..

##### 6.3.ae1.1.4 Effect of receipt

On receipt of this primitive, the MLME constructs a MFQ Policy Query Request frame containing the set of MFQ Policy parameters. This frame is then scheduled for transmission.

#### 6.3.ae1.2 MLME-MFQPOLICYQUERY.confirm

##### 6.3.ae1.2.1 Function

This primitive reports the result of a MFQ Policy Query.

##### 6.3.ae1.2.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-MFQPOLICYQUERY.confirm (

Dialog Token,

Result Code,

Peer MAC Address,

MFQ Policy parameters

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Dialog Token | Integer | 1-255 | The dialog token to identify the MFQ Policy Query transaction. |
| Result Code | Enumeration | SUCCESS, INVALID_PARAMETERS, TIMEOUT, TRANSMISSION_FAILURE, UNSPECIFIED_FAILURE | Reports the outcome of an MFQ Policy Query. |
| Peer MAC Address | MAC Address | Any valid individual MAC Address | The address of the peer MAC entity to which the MFQ Policy Query request is sent. |
| MFQ Policy parameters | MFQ Policy element | As defined in 8.4.2.ae1 (MFQ Policy element) | These parameters indicate the MFQ Policy parameters received from the peer in MFQ Policy Query Response frame. |

##### 6.3.ae1.2.3 When generated

This primitive is generated by the MLME when the request to MFQ Policy Query transmission completes.

##### 6.3.ae1.2.4 Effect of receipt

On receipt of this primitive, the SME evaluates the result code.

#### 6.3.ae1.5 MLME-MFQPOLICYCONFIG.request

##### 6.3.ae1.5.1 Function

This primitive supports the signaling of MFQ Policy Configuration between peer SMEs. An SME configures the MFQ Policy within a peer SME.

##### 6.3.ae1.5.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-MFQPOLICYCONFIG.request (

Peer MAC Address,

Dialog Token,

MFQ Policy parameters

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MAC Address | Any valid individual MAC Address | The address of the peer MAC entity to which the MFQ Policy Config request is sent. |
| Dialog Token | Integer | 1-255 | The dialog token to identify the MFQ Policy Config transaction. |
| MFQ Policy parameters | MFQ Policy element | As defined in 8.4.2.ae1 (MFQ Policy element) | These parameters indicate the MFQ Policy the peer is requested to use. |

##### 6.3.ae1.5.3 When generated

This primitive is generated by the SME when a STA wishes to request a change of the MFQ Policy within another STA..

##### 6.3.ae1.5.4 Effect of receipt

On receipt of this primitive, the MLME constructs a MFQ Policy Config Request frame containing the set of MFQ Policy parameters. This frame is then scheduled for transmission.

#### 6.3.ae1.6 MLME-MFQPOLICYCONFIG.confirm

##### 6.3.ae1.6.1 Function

This primitive reports the result of a MFQ Policy Config.

##### 6.3.ae1.6.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-MFQPOLICYCONFIG.confirm (

Dialog Token,

Result Code,

Peer MAC Address

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Dialog Token | Integer | 1-255 | The dialog token to identify the MFQ Policy Config transaction. |
| Result Code | Enumeration | SUCCESS, REJECT, INVALID_PARAMETERS, TIMEOUT, TRANSMISSION_FAILURE, UNSPECIFIED_FAILURE | Reports the outcome of a request to send a MFQ Policy Config Request frame. |
| Peer MAC Address | MAC Address | Any valid individual MAC Address | The address of the peer MAC entity to which the MFQ Policy Config request is sent. |

[SM: Deletion of MFQ Policy Parameters from this table is not shown, as a marked change]

##### 6.3.ae1.6.3 When generated

This primitive is generated by the MLME when the request to MFQ Policy Config transmission completes.

##### 6.3.ae1.6.4 Effect of receipt

On receipt of this primitive, the SME evaluates the result code.

#### 6.3.ae1.7 MLME-MFQPOLICYCONFIG.indication

##### 6.3.ae1.7.1 Function

This primitive indicates that a MFQ Policy Config Request frame has been received from a peer entity.

##### 6.3.ae1.7.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-MFQPOLICYCONFIG.indication (

Peer MAC Address,

Dialog Token,

MFQ Policy parameters

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MAC Address | Any valid individual MAC Address | The address of the peer MAC from which the MFQ Policy Config request was received |
| Dialog Token | Integer | 1-255 | The dialog token to identify the MFQ Policy Config transaction. |
| MFQ Policy parameters | MFQ Policy element | As defined in 8.4.2.ae1 (MFQ Policy element) | This element is optionally present. |

#### 6.3.ae1.7.3 When generated

This primitive is generated by the MLME when a valid MFQ Policy Config Request frame is received from a peer entity.

##### 6.3.ae1.7.4 Effect of receipt

On receipt of this primitive, the MFQ Policy Config Request frame elements are provided to the SME to be processed.

#### 6.3.ae1.8 MLME-MFQPOLICYCONFIG.response

##### 6.3.ae1.8.1 Function

This primitive requests the transmission of MFQ Policy Config Response frame to a peer entity, in response to a received MFQ Policy Config Request frame.

##### 6.3.ae1.8.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-MFQPOLICYCONFIG.response (

Peer MAC Address,

Dialog Token,

Result Code

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MAC Address | Any valid individual MAC Address | The address of the peer MAC entity to which the MFQ Policy Config request is sent. |
| Dialog Token | Integer | 1-255 | The dialog token to identify the MFQ Policy Config transaction. |
| Result Code | Enumeration | SUCCESS, REJECT, INVALID_PARAMETERS, TIMEOUT, TRANSMISSION_FAILURE, UNSPECIFIED_FAILURE | Reports the outcome of a MFQ Policy Config Request. |

##### 6.3.ae1.8.3 When generated

This primitive is generated by the SME to request that a MFQ Policy Config Response frame be sent to a

peer entity to convey MFQ Policy information, following the configuration request from that peer entity.

##### 6.3.ae1.8.4 Effect of receipt

On receipt of this primitive, the MLME constructs a MFQ Policy Config Response frame containing the

set of MFQ Policy elements specified. This frame is then scheduled for transmission.

#### 6.3.ae1.9 MLME-MFQPOLICYSET.request

##### 6.3.ae1.9.1 Function

This primitive supports the signaling of MFQ Policy from the SME to the MLME.

#### 6.3.ae1.9.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-MFQPOLICYSET.request (

Dialog Token,

MFQ Policy parameters

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Dialog Token | Integer | 1-255 | The dialog token to identify the MFQ Policy Set transaction. |
| MFQ Policy parameters | MFQ Policy element | As defined in 8.4.2.ae1 (MFQ Policy element) | This element is optionally present when the MIB attribute dot11MgmtFrameQoSPolicyActivated is set to true, and is not present otherwise. |

##### 6.3.ae1.9.3 When generated

This primitive reports the outcome of an MFQ Policy Set Request.

##### 6.3.ae1.9.4 Effect of receipt

On receipt of this primitive, the MLME uses the supplied set of MFQ Policy parameters for the BSS.

#### 6.3.ae1.10 MLME-MFQPOLICYSET.confirm

##### 6.3.ae1.10.1 Function

This primitive reports the result of a MFQ Policy Set request.

##### 6.3.ae1.10.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-MFQPOLICYSET.confirm (

Dialog Token,

Result Code,

MFQ Policy parameters

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Dialog Token | Integer | 1-255 | The dialog token to identify the MFQ Policy Set transaction. |
| Result Code | Enumeration | SUCCESS, REJECT, INVALID_PARAMETERS, TIMEOUT, TRANSMISSION_FAILURE, UNSPECIFIED_FAILURE | Reports the outcome of a [SM: typo not picked up in D1.01] MFQ Policy Set Request. |
| MFQ Policy parameters | MFQ Policy element | As defined in 8.4.2.ae1 (MFQ Policy element) | This element is optionally present when the MIB attribute dot11MgmtFrameQoSPolicyActivated is set to true, and is not present otherwise. |

##### 6.3.ae1.10.3 When generated

This primitive is generated by the MLME when the request to MFQ Policy Set frame transmission completes.

##### 6.3.ae1.10.4 Effect of receipt

On receipt of this primitive, the SME evaluates the result code.