IEEE P802.11
Wireless LANs

| BRP Comment Resolution for CID 125 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2011-01-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Christopher Hansen | Broadcom Corporation | 190 Mathilda Place Sunnyvale, CA 94086 USA | +1 408 543 3378 | chansen@broadcom.com |

Abstract

Draft text for changes to address renaming of a sub-field in the DBand beacon, small changes to the DBand Link Margin element, small changes to the EVM definition, and clarifications for zero padding BRP packets as part of CID 125.

Editor’s instructions:  Add a reference to 802.11r in Section 2.

Editor’s instructions:  modify section 7.2.4.1 as shown in the following text:

#### DBand Beacon

The format of the DBand Beacon is shown in Figure 1.

In addition to supporting functions such as network synchronization (Error: Reference source not found), the DBand Beacon frame can also be used as training frame for beamforming (Error: Reference source not found).

| Octets: | 2 | 2 | 6 | Variable | 4 |
| --- | --- | --- | --- | --- | --- |
|  | Frame Control | Duration | BSSID | Body | FCS |

Figure 1 – DBand Beacon frame format

The Duration field is set to the time remaining until the end of the BTI.

The BSSID field contains the BSSID.

Editor’s instructions:  replace section 7.3.2.105 with the following text:

7.3.2.105 DBand Link Margin Element, page 81, line 6-7:

The MCS field is reserved if the Activity field is not set to ‘Decrease MCS’,

‘Increase MCS’ or ‘Power conserve mode’.

7.3.2.105.1 Activity field

Table 20 – Activity field values

| Preferred Action Value | Meaning |
| --- | --- |
| 0 | No PreferenceNo change preferred |
| 1 | Decrease MCS |
| 2 | Increase MCS |
| 3 | Decrease transmit power |
| 4 | Increase transmit power |
| 5 | Fast session transfer (FST) |
| 6 | Power conserve mode |
| 7-255 | Reserved |

Editor’s instructions:  Modify section 9.7d.1 as shown:

9.7d.1 A-MPDU contents

A mSTADBand STA shall transmit an A-MPDU in either the Data Enabled Immediate Response

context or the Control Response context specified in Table 7-57wTable 7-57y and Table 7-57ab, respectively.

The contents of an A-MPDU transmitted by a mSTA DBand STA in the data enabled immediate

response context shall be the ACK MPDU, or the HT-immediate BlockAck, or the Action No Ack as specified in Table 7-57xTable 7-57y.

Editor’s instructions:  Modify section 9.27 as shown:

9. 27 DBand Link adaptation

If the Dialog Token field in the Link Measurement Request frame is set to a non-zero value, the

requesting STA shall send a frame to the same responding STA SIFS after receiving the ACK or BAframe corresponding to the Link Measurement Request frame. the responding STA shall perform the measurement on the next frame received from the requesting STA and shall send back a Link Measurement Report frame corresponding to the received frame..

Editor’s instructions:  Modify section 21.4.1.1.1 to incorporate the new text as shown:

##### 21.4.1.1.1 Transmit EVM

The transmit EVM accuracy test shall be performed by instrumentation capable of converting the transmitted signal into a stream of complex samples, with sufficient accuracy in terms of I/Q arm amplitude and phase balance, dc offsets, phase noise, etc.  The instrumentation shall perform carrier lock, symbol timing recovery and amplitude adjustment while making the measurements. The instrumentation shall incorporate a rake receiver or equalizer to minimize error resulting from multipath. If used, the equalizer shall be trained using information in the  Preamble (STF and/or CEF).  For the CPHY EVM, the signal is first de-spread using . The EVM will then be calculated on the resulting symbols according to the formula below:

Editor’s instructions:  Modify section 21.5.3.2.2.2 as shown:

##### 21.5.3.2.2.2 LDPC encoding process

The LDPC encoding process is composed of several steps including determining the number of padding bits, padding with zeros and the coding of every word.

- First the total number of padding bits NPAD is calculated, using the number of LDPC codeword NCW and the number of OFDM symbols NSYM:

Recalculate NCW = NSYM⋅NCBPSLCW

Where LCW=672 is the LDPC code word length, Length is the length of the PSDU defined in the header field, R is the code rate and NCBPS is the number of code bits per symbol as defined in the MCS table.

- The PSDU is concatenated with NPAD zeros. They are scrambled using the continuation of the scrambler sequence that scrambled the PSDU.

- The output stream of the scrambler is broken into blocks of LCWD= R×LCW bits such as the m’th data word is .

- To each data word, n-k=LCW-R×LCW parity bits are added to create the code word  such that.

- The code words are the concatenated one after the other to create the coded bits stream .

Editor’s instructions:  Modify section 21.6.3.2.2.2 as shown:

##### 21.6.3.2.2.2 LDPC encoding process

The LDPC encoding process is composed of several steps that includes deciding the number of shortening/repetition bits in every code word, the shortening itself, the coding of each word and then any repetition of bits.

- First the total number of data pad bits NDATA_PAD is calculated, using the number of LDPC codewords NCW:

where LCW=672 is the LDPC code word length, Length is the length of the PSDU defined in the header field (in octets), ρ is the repetition factor (1 or 2), and R is the code rate.

NCW min is defined for BRP packets in Table X.

The scrambled PSDU is concatenated with NDATA_PAD zeros. They are scrambled using the continuation of the scrambler sequence that scrambled the PSDU input bits.

- The procedure for converting the scrambled PSDU data to LDPC codewords depends on the repetition factor.

  - If  = 1,

    - The output stream of the scrambler is broken into blocks of LCWD = LCW ×R bits such that the mth data word is .

    - To each data word, n-k=LCW-R×LCW parity bits are added to create the code word c(m)=b1(m),b2(m),...,bLCWD(m),p1(m),...pn−k(m) such that

  - If ρ = 2,

    - The data bits in each codeword whereare concatenated with zeros to produce a sequence in length of .

    - The LDPC code word  is created by generating the parity bits  such that . Where H is the parity matrix for rate 1/2 LDPC coding specified in Error: Reference source not found.

    - Replace bits though 336 of the codeword c with bits from the sequence  XOR’ed by a PN sequence which is generated from the LFSR used for data scrambling as defined in Error: Reference source not found. The LFSR is initialized to the all ones vector and reinitialized to the same vector after every codeword.

- The code words are then concatenated one after the other to create the coded bits stream c1,c2,...,cLCWD⋅NCW.

- The number of symbol blocks, NBLKS, and the number of symbol block padding bits, NBLK_PAD, are calculated:

NBLKS=NCW⋅LCWNCBPBNBLKPAD=NBLKS⋅NCBPB−NCW⋅LCW,

where NCBPB is the number of coded bits per symbol block, per Error: Reference source not found in Error: Reference source not found.

- The coded bit stream is concatenated with NBLK_PAD zeros. They are scrambled using the continuation of the scrambler sequence that scrambled the NDATA_PAD bits in step 1.

Editor’s instructions:  Replace section 21.10.2.2.3 as shown:

##### 21.10.2.2.3 Beam Refinement packet duration

The minimum duration of the data field of a beam refinement packet when sent in an SC PHY is 18 SC blocks (see Error: Reference source not found) and, if needed, the data field of the packet shall be extended by extra zero padding to generate the required number of SC blocks.

Table X – Zero Filling for SC Beam Refinement Packets

| MCS Index | Modulation | NCBPS | Repetition | Code Rate | Data Rate (Mbps) | NCWmin |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | π/2-BPSK | 1 | 2 | 1/2 | 385 | 12 |
| 2 | π/2-BPSK | 1 | 1 | 1/2 | 770 | 12 |
| 3 | π/2-BPSK | 1 | 1 | 5/8 | 962.5 | 12 |
| 4 | π/2-BPSK | 1 | 1 | 3/4 | 1155 | 12 |
| 5 | π/2-BPSK | 1 | 1 | 13/16 | 1251.25 | 12 |
| 6 | π/2-QPSK | 2 | 1 | 1/2 | 1540 | 23 |
| 7 | π/2-QPSK | 2 | 1 | 5/8 | 1925 | 23 |
| 8 | π/2-QPSK | 2 | 1 | 3/4 | 2310 | 23 |
| 9 | π/2-QPSK | 2 | 1 | 13/16 | 2502.5 | 23 |
| 10 | π/2-16QAM | 4 | 1 | 1/2 | 3080 | 46 |
| 11 | π/2-16QAM | 4 | 1 | 5/8 | 3850 | 46 |
| 12 | π/2-16QAM | 4 | 1 | 3/4 | 4620 | 46 |

The minimum duration of the data field of a beam refinement packet when sent in an OFDM PHY is 20 OFDM blocks and, if needed, the data field of the packet shall be extended by extra zero padding to generate the required number of OFDM symbols.