IEEE P802.11
Wireless LANs

| Type Length Value Scope |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2011-01-17 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Peter Ecclesine | Cisco Systems |  | +1-408-527-0815 | pecclesi@cisco.com |

Abstract

Definition of domain-specific Type Length Value Scope terms

## Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGaf Draft.  This introduction is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGaf Draft (i.e. they are instructions to the 802.11 editor on how to merge the text with the baseline documents).

TGaf Editor: Editing instructions preceded by “TGaf Editor” are instructions to the TGaf editor to modify existing material in the TGaf draft.  As a result of adopting the changes, the TGaf editor will execute the instructions rather than copy them to the TGaf Draft.

Discussion: The publishing of FCC 10-174 in December 2010 makes it important to distinguish between 802.11af terms and FCC terms. For this reason, we propose to add domain- and country-specific information elements in Annex E.3.

## Editing instructions:

TGaf Editor:Insert the following text after the end of E.2.4”

### E.3 TLV Values

The following TLV encodings shall be used for parameters in MAC Management messages (8.3). TLV tuples with Type values not specified in this standard or specified as “reserved” shall be silently discarded. The STA shall silently discard any TLV with an unknown type number. The length of the Type field shall be one byte.

The format of the Length field shall be an unsigned number whose length is indicated by each TLV entry.

NOTE—Uniqueness of TLV Type values is assured by identifying the groups of IEEE 802.11 entities (MAC management messages) that share references to specific TLV encodings. Disjoint collections of TLVs are formed that correspond to each such functional grouping. Each set of TLVs that are explicitly defined to be members of a compound TLV structure form additional collections. Unique type values are assigned to the member TLV encodings of each collection.

An additional collection, the Common encodings, is defined that consists of TLV encodings that are referenced by more than one of the functional groups. The Type values of the TLV members of this collection are assigned to assure uniqueness across all collections. This is the only collection for which global uniqueness is guaranteed.

In cases where a collection contains TLV encodings that are PHY-specification-specific, subcollections are formed that contain these TLV encodings. Type values assigned to members of each subcollection are assigned so that the values are unique within the subcollection and with non-PHY-specification-specific members of the collection. Type values are not unique across PHY-specific subcollections.

TLV Type values are assigned in accordance with the following rules:

— Common encoding types start at 149, subsequent values are assigned in descending order.

— For individual collections, non-PHY-specification-specific encodings start at 1, subsequent values are assigned

in ascending order.

— For individual collections, PHY-specification-specific encodings start at 150, subsequent values are assigned in

ascending order.

Unless otherwise indicated, bit 0 is the LSB of the least significant byte for all TLVs with length of multiple bytes.

### E.3.1 Common TLV Values

The general form of table is shown in Table E-7af1.

Table E-7af1 – Common TLV values

| Name | Type | Length | Value | Scope |
| --- | --- | --- | --- | --- |
|  | <ANA> | variable | Compound TLVs in Table E-af2 | XXX, US |

Where there is a two character Scope value, it identifies the country or non-country entity to which the station’s operation is bound. If the two character value is a country entity, the two characters are the two character country code as described in document ISO/IEC 3166-1.

References:

IEEE Std 802.16-2009 Clause 11 TLV encodings