IEEE P802.11
Wireless LANs

| Indication of Group Address in PLCP Header |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2011-01-17 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Jarkko Kneckt | Nokia Corporation | Itämerenkatu 11 – 13 00180 Helsinki, Finland |  | Jarkko.Kneckt@nokia.com |
| Mika Kasslin | Nokia Corporation | Itämerenkatu 11 – 13 00180 Helsinki, Finland |  | Mika.Kasslin@nokia.com |

Abstract

The document describes means to carry the group address information in PLCP header. The group address information is compatible with Flexible Multicast Service (FMS) as defined in 802.11v [1].

The presentation for this normative text is 11-11-0096-00-00ac-Indication-of-Groupaddress.doc

##### 3.2.3.2.2 VHT-SIG-A definition

Instructions to editor: Incorporate the changes as shown with track changes.

R3.2.1.H: VHT-SIG-A includes the fields listed in Table 1.

Table 1 - VHT-SIG-A fields

| Bit Index | Field | Bit allocation | Description |
| --- | --- | --- | --- |
| VHT-SIG-A1 |  |  |  |
| 0-1 | BW | 2 | Set to 0 for 20 MHz, 1 for 40 MHz, 2 for 80 MHz, 3 for 160 MHz and 80+80 MHz mode |
| 2 | Reserved | 1 | Reserved for possible expansion of BW field. Set to 1. |
| 3 | STBC | 1 | Set to 1 if all streams use STBC, otherwise set to 0. When STBC bit is 1, an odd number of space time streams per user is not allowed. |
| 4-9 | Group ID [10/0582r1] | 6 | A value of [1,1,1,1,1,0] indicates a transmission to FMSID stream and needs to bypass a group (e.g. broadcast) A value of [1,1,1,1,1,1] indicates [10/0382r2]: - A single user transmission - A transmission where the group membership has not yet been established - A transmission that is not assigned to FMSID stream and needs to bypass a group (e.g. broadcast) |
| 10-21 | NSTS [10/0582r1] | 12 | For MU: 3 bits/user with maximum of 4 users - Set to 0 for 0 space time streams - Set to 1 for 1 space time stream - Set to 2 for 2 space time streams - Set to 3 for 3 space time streams - Set to 4 for 4 space time streams When Group ID is set to [1,1,1,1,1,0]: first 3 bits contain stream allocation, set to 0 for 1 space time stream, set to 1 for 2 space time streams, etcetera up to 8 streams. The next 8 bits contain FMSID. The last bit is reserved. When Group ID is set to [1,1,1,1,1,1]: first 3 bits contain stream allocation, set to 0 for 1 space time stream, set to 1 for 2 space time streams, etcetera up to 8 streams. Remaining 9 bits contain partial AID: being the 9 LSB bits of AID. For Broadcast and multicast that are not assigned to FMSID stream, these 9 bits are set to 0. For STA-to-AP, these 9 bits are set to a special value (TBD). |
| 22-23 | Reserved | 2 | All ones |
|  | Total | 24 |  |
| VHT-SIG-A2 |  |  |  |
| 0-1 | Short GI | 2 | Set B0 to 0 for Long GI, set to 1 for Short GI Set B1 to 1 when Short GI and Nsym%10 == 9 |
| 2-3 | Coding | 2 | For SU: Set B2 to 0 for BCC, set to 1 for LDPC For MU: [10/1277r0] Set B2 to 0 for BCC, set to 1 for LDPC for 1st user If user 1 has 0 Nsts value, then B2 is reserved and set to 1 B3 is defined as Nldpc-ext (see 3.2.4.2.2) |
| 4-7 | MCS | 4 | For SU/Broadcast/Multicast: MCS index For MU: [10/1277r0] B4: Set to 0 for BCC, 1 for LDPC for the 2nd user B5: Set to 0 for BCC, 1 for LDPC for the 3rd user B6: Set to 0 for BCC, 1 for LDPC for the 4th user If user 2, 3, or 4 has 0 Nsts value, then corresponding bit is reserved and set to 1 B7: Reserved and set to 1 |
| 8 | SU-Beamformed | 1 | Set to 1 when packet is a SU-beamformed packet Set to 0 otherwise For MU: Reserved, set to 1 |
| 9 | Reserved | 1 | All ones |
| 10-17 | CRC | 8 | CRC calculated as in 11n Section 20.3.9.4.4 with C7 in B10 |
| 18-23 | Tail | 6 | All zeros |
|  | Total | 24 |  |

Note that the fields are transmitted in the order shown in and LSB first.

[10/1052r0]

References:

[1] 	Draft 802.11v_D 15.0, September 2010