IEEE P802.11
Wireless LANs

| Indication of Group Address in PLCP Header |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2011-01-19 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Jarkko Kneckt | Nokia Corporation | Itämerenkatu 11 – 13 00180 Helsinki, Finland |  | Jarkko.Kneckt@nokia.com |
| Mika Kasslin | Nokia Corporation | Itämerenkatu 11 – 13 00180 Helsinki, Finland |  | Mika.Kasslin@nokia.com |

Abstract

The document describes means to carry the group address information in PLCP header.

The partial group address enables group addressed receivers to skip the reception of unnecessary group addressed frames and improve the non-AP STA power efficiency especially in the stand-by mode and in passive scanning.

##### 3.2.3.2.2 VHT-SIG-A definition

Instructions to editor: Incorporate the changes as shown with track changes.

R3.2.1.H: VHT-SIG-A includes the fields listed in Table 1.

Table 1 - VHT-SIG-A fields

| Bit Index | Field | Bit allocation | Description |
| --- | --- | --- | --- |
| VHT-SIG-A1 |  |  |  |
| 0-1 | BW | 2 | Set to 0 for 20 MHz, 1 for 40 MHz, 2 for 80 MHz, 3 for 160 MHz and 80+80 MHz mode |
| 2 | Reserved | 1 | Reserved for possible expansion of BW field. Set to 1. |
| 3 | STBC | 1 | Set to 1 if all streams use STBC, otherwise set to 0. When STBC bit is 1, an odd number of space time streams per user is not allowed. |
| 4-9 | Group ID [10/0582r1] | 6 | A value of [1,1,1,1,1,0] indicates a transmission that needs to bypass a group (e.g. broadcast) A value of [1,1,1,1,1,1] indicates [10/0382r2]: - A single user transmission - A transmission where the group membership has not yet been established |
| 10-21 | NSTS [10/0582r1] | 12 | For MU: 3 bits/user with maximum of 4 users - Set to 0 for 0 space time streams - Set to 1 for 1 space time stream - Set to 2 for 2 space time streams - Set to 3 for 3 space time streams - Set to 4 for 4 space time streams When Group ID is set to [1,1,1,1,1,0]: first 3 bits contain stream allocation, set to 0 for 1 space time stream, set to 1 for 2 space time streams, etcetera up to 8 streams. Remaining 9 bits contain the 9 LSB of the group address. When Group ID is set to [1,1,1,1,1,1]: first 3 bits contain stream allocation, set to 0 for 1 space time stream, set to 1 for 2 space time streams, etcetera up to 8 streams. Remaining 9 bits contain partial AID: being the 9 LSB bits of AID. For STA-to-AP, these 9 bits are set to a special value (TBD). |
| 22-23 | Reserved | 2 | All ones |
|  | Total | 24 |  |