IEEE P802.11
Wireless LANs

| GCR Sequence numbers |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2011-01-19 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Alex Ashley | NDS Ltd | One London Road, Staines, Middlesex, TW18 4EX, UK |  | aashley at nds dot com |

Abstract

Draft 2 of P802.11aa has an issue with sequence numbering when switching between DMS and GCR, as the non-AP STA does not know how the sequence numbers for the group addressed delivery match with the unicast delivery.

7.3.2.89 DMS Response element

Change paragraphs eight and nine of 7.3.2.89 as indicated:

When the Last Sequence Control field is not supported the Last Sequence Control field is set to 65535. When the Last Sequence Control field is supported and the Response Type field is set to a value that is not equal to “Terminate” or “Advertise”, the Last Sequence Control field is reserved.

When the Response Type field is “Terminate” and the Last Sequence Control field is supported, Bit 0 to Bit 3 of the Last Sequence Control field is 0, and Bit 4 to Bit 15 of the Last Sequence Control field contains the sequence number of the last group addressed frame that the AP converted to an individually addressed frame and sent successfully delivered to the non-AP STA that is the recipient of the DMS Response Frame. If the Response Type field is “Terminate” and the this last frame received by the non-AP STA prior to DMS termination has not also been sent using a group addressed frame, the Last Sequence Control field is set to 65534.

##### 11.22.15.aa2.4 GCR Frame Exchange Procedures

Change nineth paragraph of 11.22.15.aa2.4 as indicated:

The AP may update the retransmission policy, delivery method, and schedule as the size of the group changes, the capabilities of the members of the group change, GCR Request subelements for the group are received, Multicast Diagnostics or for any other reason. The AP advertises the current settings upon a change and periodically by:

- Transmitting an unsolicited DMS Response frame with the current settings addressed to the broadcast address. This DMS Response frame shall be scheduled for delivery at the appropriate DTIM interval or SP where all non-AP STAs within the group are awake to receive the frame. One TCLAS element, one TSPEC element and one GCR Subselement shall be included per DMS Descriptor in the DMS Response element of the DMS Response frame to identify each GCR stream. The DMSID that identifies the GCR stream shall be included the DMS Descriptor. Each Status field in the DMS Status fields included in the frame shall be set to GCR Advertise.

- Transmitting an unsolicited DMS Response frame with the current settings addressed to the GCR group address. This DMS Response frame shall be scheduled for delivery at the appropriate DTIM interval or SP where all non-AP STAs within the group are awake to receive the frame. One TCLAS element, one TSPEC element and one GCR Subselement shall be included per DMS Descriptor in the DMS Response element of the DMS Response frame to identify each GCR stream. The DMSID that identifies the GCR stream shall be included the DMS Descriptor. Each Status field in the DMS Status fields included in the frame shall be set to GCR Advertise.

- Transmitting unsolicited DMS Response frames with the current settings individually addressed to each GCR group member. The DMSID shall be included in per DMS Descriptor in the DMS Response element of the DMS Response frame to identify each GCR stream.  No TCLAS element, no TSPEC element and no GCR Subselement shall be included in these DMS Descriptors. Each Status field in the DMS Status fields included in the frame shall be set to GCR Advertise.

Insert the following paragraph after paragraph nine:

When the AP updates the retransmission policy, the AP should set the Last Sequence Control field in the GCR response frame to the sequence number of the last A-MSDU delivered before the change in retransmission policy.

References: