IEEE P802.11
Wireless LANs

| MIB Contribution |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: January 19, 2011 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Daniel R. Borges | Apple, Inc. |  | 415.425.7347 | drborges@apple.com |
| Carlos Cordeiro | Intel Corporation |  | +1 (503) 705-1286 | carlos.cordeiro@intel.com |
| Saishankar Nandagopalan | Tensorcom |  | (858) 337-6652 | nsai@tensercom.com |

Abstract

This document is a contribution to address the open CIDs related to the TGad Management Information Base (MIB).  The following CIDs are addressed with this contribution:

- 1102

- 920

- 176

- 164 (duplicate of 176)

- 257

Editor: In the Major Section of Annex D, insert the following comment to the end of SMT attributes after dot11HTStationConfigTable:

-- Added for 802.11ad

-- dot11DBandSTAConfigTable			::= { dot11smt 18 }(11ad)

Editor: In the Major Section of Annex D, insert the following comment to the end of dot11mac section after dot11ResourceInfoTable:

-- Added for 802.11ad

-- dot11DBandOperationTable		::= { dot11mac 8 }(11ad)

-- dot11DBandCountersTable		::= { dot11mac 9 }(11ad)

Editor: In the Major Section of Annex D, insert the following comment to the end of dot11phy section after dot11TransmitBeamformingConfigTable:

-- Added for 802.11ad

-- dot11DBandPHYConfigTable		::= { dot11phy 19 }(11ad)

-- dot11DBandPHYTxRateTable		::= { dot11phy 20 }(11ad)

-- dot11DBandPHYRxRateTable		::= { dot11phy 21 }(11ad)

-- dot11DBandBeamformingConfigTable ::= { dot11phy 22 }(11ad)

Editor: In the Major Section of Annex D, modify the dot11PhyType object with the following definition.

-- **********************************************************************

-- *    dot11PhyOperation  TABLE

-- **********************************************************************

dot11PHYType OBJECT-TYPE

SYNTAX INTEGER {

fhss(1),

dsss(2),

irbaseband(3),

ofdm(4),

hrdsss(5),

erp(6),

ht(7),

dband(8)

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is an 8-bit integer value that identifies the PHY type

supported by the attached PLCP and PMD. Currently defined

values and their corresponding PHY types are:

FHSS 2.4 GHz = 01 , DSSS 2.4 GHz = 02, IR Baseband = 03,

OFDM 5GHz = 04, HRDSSS = 05, ERP = 06, HT = 07, DBand = 08."

::= { dot11PhyOperationEntry 1 }

Editor: After the definition of the dot11HTStationConfigTable, insert the dot11DBandSTAConfigTable as defined below:

-- ********************************************************************

-- * dot11DBandSTAConfigTable TABLE

-- ********************************************************************

dot11DBandSTAConfigTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11DBandSTAConfigEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The dot11DBandSTAConfig Table"

::= { dot11smt 18 }

dot11DBandSTAConfigEntry OBJECT-TYPE

SYNTAX Dot11DBandSTAConfigEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11DBandSTAConfig Table.

ifIndex - Each IEEE 802.11 interface is represented by an

ifEntry.  Interface tables in this MIB module are indexed

by ifIndex."

INDEX { ifIndex }

::= { dot11DBandSTAConfigTable 1 }

Dot11DBandSTAConfigEntry ::=

SEQUENCE {

dot11MultibandSupportActivated			TruthValue,

dot11DBandCapable						TruthValue,

dot11RelayRUSSupportActivated				TruthValue,

dot11RSNAProtectedManagementFramesActivated	TruthValue

}

dot11MultibandSupportActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Multiband Support Activated

This attribute, when true, indicates that the capability

of the station to operate in multiple bands is enabled.

The capability is disabled, otherwise. The default value

of this attribute is false."

DEFVAL { false }

::= {  dot11DBandSTAConfigEntry 1 }

dot11DBandCapable OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"DBand Capable Object

This attribute, when true, indicates the STA is DBand

capable.  This attribute, when false, indicates the STA is not

DBand capable.  The default value of htis attribute is false."

DEFVAL { false }

::= { dot11DBandSTAConfigEntry 2 }

dot11RelayRUSSupportActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"A STA is a RUS if the value of its local MIB variable

dot11RelayRUSSupportEnabled is true."

DEFVAL { false }

::= { dot11DBandSTAConfigEntry 3 }

dot11RSNAProtectedManagementFramesActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"If dot11RSNAProtectedManagementFramesEnabled is TRUE, the recipient

shall maintain a single replay counter for received unicast Robust

Management frames and shall use the PN from the received frame to

detect replays."

DEFVAL { false }

::= { dot11DBandSTAConfigEntry 4 }

Editor: After the definition of the dot11DBandPHYConfigTable, insert the dot11DBandBeamformingConfigTable as defined below:

-- ********************************************************************

-- * dot11DBandBeamformingConfig TABLE

-- ********************************************************************

dot11DBandBeamformingConfigTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11DBandBeamformingConfigEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The dot11DBandBeamformingConfig Table"

::= { dot11phy 22 }

dot11DBandBeamformingConfigEntry OBJECT-TYPE

SYNTAX Dot11DBandBeamformingConfigEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11DBandBeamformingConfig Table.

ifIndex - Each IEEE 802.11 interface is represented by an

ifEntry.  Interface tables in this MIB module are indexed

by ifIndex."

INDEX { ifIndex }

::= { dot11DBandBeamformingConfigTable 1 }

Dot11DBandBeamformingConfigEntry ::=

SEQUENCE {

dot11MaxBFTime				Unsigned32,

dot11BFTXSSTime				Unsigned32,

dot11MaximalSectorScan			Unsigned32,

dot11ABFTRSSSwitch			Unsigned32,

dot11RSSRetryLimit			Unsigned32,

dot11RSSBackoff				Unsigned32,

dot11BFRetryLimit				Unsigned32,

dot11BFTXSSTime				Unsigned32,

dot11BeamLinkMaintenanceTime		Unsigned32

}

dot11MaxBFTime OBJECT-TYPE

SYNTAX Unsigned32 (1..16)

UNITS "milliseconds"

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Maximum Beamforming Time (in units of Beacon Interval)."

DEFVAL { 4 }

::= { dot11DBandBeamformingConfigEntry 1 }

dot11BFTXSSTime OBJECT-TYPE

SYNTAX Unsigned32 (1..256)

UNITS "microseconds"

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Beamforming Transmission ScS Time (in Milliseconds)."

DEFVAL { 4 }

::= { dot11DBandBeamformingConfigEntry 2 }

dot11MaximalSectorScan OBJECT-TYPE

SYNTAX Unsigned32 (1..32)

UNITS "microseconds"

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Maximal Sector Scan (in units of Beacon Interval)."

DEFVAL { 8 }

::= { dot11DBandBeamformingConfigEntry 3 }

dot11ABFTRSSSwitch OBJECT-TYPE

SYNTAX Unsigned32 (1..32)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"A-BFT Responder Sector Sweep Switch (in units of Beacon Interval)."

DEFVAL { 4 }

::= { dot11DBandBeamformingConfigEntry 4 }

dot11RSSRetryLimit OBJECT-TYPE

SYNTAX Unsigned32 (1..32)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Responder Sector Sweep Retry Limit"

DEFVAL { 8 }

::= { dot11DBandBeamformingConfigEntry 5 }

dot11RSSBackoff OBJECT-TYPE

SYNTAX Unsigned32 (1..32)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Responder Sector Sweep Backoff"

DEFVAL { 8 }

::= { dot11DBandBeamformingConfigEntry 6 }

dot11BFRetryLimit OBJECT-TYPE

SYNTAX Unsigned32 (1..32)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Beamforming Retry Limit"

DEFVAL { 8 }

::= { dot11DBandBeamformingConfigEntry 7 }

dot11BFTXSSTime OBJECT-TYPE

SYNTAX Unsigned32 (1..1024)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Timeout until the initiator restarts ISS (in Milliseconds)"

DEFVAL { 100 }

::= { dot11DBandBeamformingConfigEntry 8 }

dot11BeamLinkMaintenanceTime OBJECT-TYPE

SYNTAX Unsigned32 (0..64000000)

UNITS "microseconds"

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Beam Link Maintenance Time

dot11BeamLinkMaintenanceTime = BeamLink_Maintenance_Unit * BeamLink_Maintenance_Value.

Otherwise, the dot11BeamLinkMaintenanceTime is left undefined. An

undefined value of the dot11BeamLinkMaintenanceTime indicates

that the STA does not participate in beamformed link maintenance."

DEFVAL { 0 }

::= { dot11DBandBeamformingConfigEntry 9 }

Editor: After the definition of the dot11ResourceInfoTable, insert the dot11DBandOperationTable as defined below:

-- ********************************************************************

-- * dot11DBandOperation TABLE

-- ********************************************************************

dot11DBandOperationTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11DBandOperationEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The dot11DBandOperation Table"

::= { dot11mac 8 }

dot11DBandOperationEntry OBJECT-TYPE

SYNTAX Dot11DBandOperationEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11DBandBeamformingConfig Table.

ifIndex - Each IEEE 802.11 interface is represented by an

ifEntry.  Interface tables in this MIB module are indexed

by ifIndex."

INDEX { ifIndex }

::= { dot11DBandOperationTable 1 }

Dot11DBandOperationEntry ::=

SEQUENCE {

dot11MaxLostBeacons			Unsigned32,

dot11MinBIHeaderDuration		Unsigned32,

dot11PSRequestSuspensionInterval	Unsigned32,

dot11AssocRespConfirmTime		Unsigned32,

dot11BroadcastSTAInfoDuration		Unsigned32,

dot11NbrOfChangeBeacons			Unsigned32,

dot11ImplicitHandoverLostBeacons	Unsigned32,

dot11MinPPDuration				Unsigned32,

dot11SPIdleTimeout				Unsigned32,

dot11QABTimeout					Unsigned32,

dot11ClusterEnableTime				Unsigned32,

dot11PNWarningThreshold				Unsigned32,

dot11BeaconSPDuration				Unsigned32,

dot11PNExhaustionThreshold			Unsigned32,

dot11MaxNumberOfClusteringMonitoringPeriods	Unsigned32

}

dot11MaxLostBeacons OBJECT-TYPE

SYNTAX Unsigned32 (1..32)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Maximum Number of Lost Beacons"

DEFVAL { 4 }

::= { dot11DBandOperationEntry 1 }

dot11MinBIHeaderDuration OBJECT-TYPE

SYNTAX Unsigned32 (1..100000)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Minimum Beacon Interval Header Duration (in microseconds)"

DEFVAL { 5000 }

::= { dot11DBandOperationEntry 2 }

dot11PSRequestSuspensionInterval OBJECT-TYPE

SYNTAX Unsigned32 (1..64)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"PS Request Suspension Interval (in units of Beacon Interval)"

DEFVAL { 8 }

::= { dot11DBandOperationEntry 3 }

dot11AssocRespConfirmTime OBJECT-TYPE

SYNTAX Unsigned32 (1..8000)

UNITS "microseconds"

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Association Response Confirmation Time (in Milliseconds)."

DEFVAL { 1000 }

::= { dot11DBandOperationEntry 4 }

dot11BroadcastSTAInfoDuration OBJECT-TYPE

SYNTAX Unsigned32 (1..512)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Broadcast STA Information Duration (in units of Beacon Interval)"

DEFVAL { 32 }

::= { dot11DBandOperationEntry 5 }

dot11NbrOfChangeBeacons OBJECT-TYPE

SYNTAX Unsigned32 (1..32)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Number of Change Beacons (in units of Beacon Interval)"

DEFVAL { 4 }

::= { dot11DBandOperationEntry 6 }

dot11ImplicitHandoverLostBeacons OBJECT-TYPE

SYNTAX Unsigned32 (1..32)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Implicit Handover Lost Beacons (in units of Beacon Interval)"

DEFVAL { 8 }

::= { dot11DBandOperationEntry 7 }

dot11MinPPDuration OBJECT-TYPE

SYNTAX Unsigned32 (1..100000)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The MinPPDuration subfield indicates the minimum duration of the

PP and GP as part of the dynamic allocation of service period

mechanism (in microseconds)."

DEFVAL { 200 }

::= { dot11DBandOperationEntry 9 }

dot11SPIdleTimeout OBJECT-TYPE

SYNTAX Unsigned32 (1..100000)

UNITS "microseconds"

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The SPIdleTimeout subfield indicates time during which a STA

expects to receive a frame from its partner STA (

in microseconds)."

DEFVAL { 200 }

::= { dot11DBandOperationEntry 10 }

dot11QABTimeout OBJECT-TYPE

SYNTAX Unsigned32 (1..64000)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Quiet Adjacent BSS Operation Timeout (in milliseconds)"

DEFVAL { 1000 }

::= { dot11DBandOperationEntry 11 }

dot11ClusterEnableTime OBJECT-TYPE

SYNTAX Unsigned32 (1..32)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Frequency with which a non-PCP/non-AP is allowed to retransmit a cluster request element to its PCP/AP (in units of Beacon Interval)"

DEFVAL { 8 }

::= { dot11DBandOperationEntry 12 }

dot11PNWarningThreshold OBJECT-TYPE

SYNTAX Unsigned32 (1..2e48)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Threshold used for generating a warning to the MLME about a potential PN exhaustion (in units of packet number)"

DEFVAL { 2e36 }

::= { dot11DBandOperationEntry 13 }

dot11BeaconSPDuration OBJECT-TYPE

SYNTAX Unsigned32 (1..100000)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The size of a Beacon SP used for PCP/AP clustering (in microseconds)"

DEFVAL { 600 }

::= { dot11DBandOperationEntry 14 }

dot11PNExhaustionThreshold OBJECT-TYPE

SYNTAX Unsigned32 (1..2e48)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Threshold used for indicating an imminent PN exhaustion (in units of packet number)"

DEFVAL { 2e47 }

::= { dot11DBandOperationEntry 15 }

dot11MaxNumberOfClusteringMonitoringPeriods OBJECT-TYPE

SYNTAX Unsigned32 (1..16)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Number of clustering monitoring periods until determination of the loss of an S-PCP/S-AP."

DEFVAL { 3 }

::= { dot11DBandOperationEntry 16 }

-- Statistics and Counters

-- Need to define group where these stats belong

dot11RSNAStatsGCMPReplays OBJECT-TYPE

SYNTAX Unsigned32 (1..4294967295)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Counter for the number of times an MPDU of type Data is discarded due to PN violation"

DEFVAL { 0 }

::= { TBD }

dot11RSNAStatsRobustMgmtGCMPReplays OBJECT-TYPE

SYNTAX Unsigned32 (1..4294967295)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

" Counter for the number of times an MPDU of type Management is

discarded due to PN violation"

DEFVAL { 0 }

::= { TBD }

Editor: After the definition of the dot11PhyOperationComplianceGroup2, insert the dot11PhyDBandComplianceGroup as defined below:

-- **********************************************************************

-- *   Groups - units of conformance

-- **********************************************************************

dot11PhyDBandComplianceGroup OBJECT-GROUP

OBJECTS { }

STATUS current

DESCRIPTION

"Attributes that configure the DBand PHY for IEEE 802.11"

::== { dot11Groups 54 }

dot11MacDBandComplianceGroup OBJECT-GROUP

OBJECTS { }

STATUS current

DESCRIPTION

"Attributes that configure the DBand MAC for IEEE 802.11"

::== { dot11Groups 55 }

dot11BeamformingDBandComplianceGroup OBJECT-GROUP

OBJECTS { }

STATUS current

DESCRIPTION

"Attributes that configure the DBand Beaforming functionality

for IEEE 802.11"

::== { dot11Groups 56 }

Editor: Modify the dot11Compliance module-compliance object as defined below.

-- **********************************************************************

-- *   Compliance Statements

-- **********************************************************************

dot11Compliance MODULE-COMPLIANCE

STATUS current

DESCRIPTION

"The compliance statement for SNMPv2 entities that implement the

IEEE 802.11 MIB."

MODULE -- this module

MANDATORY-GROUPS {

dot11SMTbase10,

dot11MACbase3, dot11CountersGroup3,

dot11SmtAuthenticationAlgorithms,

dot11ResourceTypeID, dot11PhyOperationComplianceGroup2,

dot11PhyDBandComplianceGroup, dot11MacDBandComplianceGroup, dot11BeamformingDBandComplianceGroup }

Editor: After the definition of the dot11TransmitBeamformingGroup, insert the dot11PhyDBandComplianceGroup as defined below:

GROUP dot11PhyDBandComplianceGroup

DESCRIPTION

"Implementation of this group is required when the object

dot11PHYType has the value of DBand.

This group is mutually exclusive to the following groups:

- "

GROUP dot11MacDBandComplianceGroup

DESCRIPTION

"DBand Compliance Group"

GROUP dot11BeamformingDBandComplianceGroup

DESCRIPTION

"DBand Beamforming Compliance Group"

::= { dot11Compliances 1 }

Editor: After the definition of dot11RMCompliance insert the following text:

-- ********************************************************************

-- * Compliance Statements - DBand (11ad)

-- ********************************************************************

dot11DbandCompliance MODULE-COMPLIANCE (11ad)

STATUS current

DESCRIPTION

"The compliance statement for SNMPv2 entities that implement the

IEEE 802.11 MIB for DBand PHY."

MODULE -- this module

MANDATORY-GROUPS

{

dot11PhyDBandComplianceGroup,

dot11MacDBandComplianceGroup,

dot11BeamformingDBandComplianceGroup

}

::= { dot11Compliances 4 }

-- **********************************************************************

-- * End of 802.11 MIB

-- **********************************************************************

END