IEEE P802.11
Wireless LANs

| D1.0 comment resolution |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2011-02-03 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Carlos Cordeiro | Intel |  |  | carlos.cordeiro@intel.com |

Abstract

This document proposes resolution to TGad D1.0 CIDs.

CID: 937

## 5.3 Logical service interfaces

Editor Note: replace the contents of subclause 5.3.0a with the following

### 5.3.0a General

.11 Editor Note: Change the second paragraph as follows:

IEEE Std 802.11 explicitly does not specify the details of DS implementations. Instead, IEEE Std 802.11 specifies services. The services are associated with different components of the architecture. There are three two categories of IEEE 802.11 service—the station service (SS), the PBSS service and the distribution system service (DSS). These Both categories of service are used by the IEEE 802.11 MAC sublayer.

.11 Editor Note: Change the last paragraph as follows:

This set of services is divided into three two groups: the SS, the PBSS service and the DSS. The SS are part of every STA. The PBSS service is provided by the PCP of a PBSS. The DSS are provided by the DS.

Editor Note: insert the following new subclause

### 5.3.1a PBSS service

The service provided by the PCP of a PBSS is known as the PBSS services. Since all STAs within a PBSS can operate as a PCP, all STAs within the PBSS are capable of providing PBSS services should they become the PCP of the PBSS. Non-PCP STAs do not provide PBSS services.

The services that comprise the PBSS services are as follows:

- Association

- Disassociation

- Reassociation

- QoS traffic scheduling (QoS facility only)

PBSS services are specified for use by MAC sublayer entities.

Editor Note: replace the contents of subclause 5.3.2 with the following

### 5.3.2 DSS

.11 Editor Note: Change the last paragraph as follows:

Figure 5-7 (Complete IEEE 802.11 architecture) combines the components from previous figures with both the three types of services to show the complete IEEE 802.11 architecture.

.11 Editor Note: replace the current (Figure 5-7 Complete IEEE 802.11 architecture) with the following figure:

## 5.4 Overview of the services

### 5.4.0a General

.11 Editor Note: change the second paragraph as follows

This subclause presents the services, an overview of how each service is used, and a description of how each service relates to other services and the IEEE 802.11 architecture. The services are presented in an order designed to help build an understanding of the operation of an IEEE 802.11 ESS network. As a result, the services that comprise the SS and DSS are intermixed in order (rather than being grouped by category). The descriptions of the services that comprise the PBSS service are described as part of the SS and DSS.

.11 Editor Note: change the title of 5.4.2 as follows

### 5.4.2 Services that support the distribution service and the PBSS service

CID: 1105

.11 Editor Note: insert a new subclause as follows

### 5.7.2 Reference model for multi-band operation

The reference model of a STA that is multi-band capable (see 11.34) is shown in Figure XXX.

Figure XXX – Reference model for a multi-band capable STA

A multi-band capable STA is characterized by having a single SME that can manage operation over more than one frequency band. The operation across the different frequency bands can be simultaneous or non-simultaneous.

The SME of a multi-band capable STA is identified by any of the MAC addresses the STA supports, and as such receives notification on MLME frames received by any of the MLME entities within the multi-band capable STA.

The SME of a multi-band capable STA contains a multi-band management entity which is responsible to coordinate the setup, configuration, tear down and transfer of FST sessions from a band/channel to another band/channel supported by the multi-band capable STA. If using non-transparent FST, the multi-band management entity can employ a combination of the source and destination MAC addresses in both the old and new band/channel to configure the routing of MSDUs and MLME primitives within the STA. If using transparent FST, in addition to the MAC addresses the multi-band management entity can employ the TID of a FST session for this routing.

A MAC SAP together with its corresponding MLME SAP is identified by a MAC address. This MAC address does not have to be unique within the multi-band capable STA, particularly when transparent FST is intended to be used.

Each MAC SAP is controlled by a separate and independent RSNA key management entity, unless if transparent FST is used in which case the RSNA key management entity can be shared among the MAC SAPs.

NOTE – For simplicity, Figure XXX depicts the reference model when there is a one-to-one mapping between each PHY and MAC layers. However, the reference model for a multi-band capable STA can be applied in conjunction with the reference model for STAs that support multiple MAC addresses (5.7.1).

CID: 7, 8, 1106

Editor Note: replace the contents of 5.2.6 with the following

### 5.2.6 QoS BSS: The QoS network

The IEEE 802.11 QoS facility provides MAC enhancements to support LAN applications with QoS requirements. The QoS enhancements are available to QoS STAs associated with a QoS access point or PCP in a QoS BSS. A subset of the QoS enhancements is available for use between STAs that are members of the same QoS IBSS. In the OBand, Bbecause a QoS STA implements a superset of STA functionality, as defined in this standard, the STA may associate with a non-QoS access point in a non-QoS BSS, to provide non-QoS MAC data service when there is no QoS BSS with which to associate.

Every STA within a BSS operating in the DBand is a QoS STA, and hence every BSS established in the DBand is a QoS BSS.

The enhancements that distinguish QoS STAs from non-QoS STAs and QoS APs from non-QoS APs are collectively termed the QoS facility. The quantity of certain, QoS-specific, mechanisms may vary among QoS implementations, as well as between QoS STAs and QoS PCPs/APs, over ranges specified in subsequent clauses. All service primitives, frame formats, coordination function and frame exchange rules, and management interface functions except for the Block Acknowledgment (Block Ack) function, direct-link setup (DLS), and automatic power-save delivery (APSD) are part of the core QoS facilities. A QoS STA or QoS AP or PCP must implement those core QoS facilities necessary for its QoS functions to interoperate with other STAs in the BSS. Functions such as the Block Ack, DLS, and APSD are separate from the core QoS facilities; and the presence of these functions is indicated by STAs separately from the core QoS facilities.

This standard provides four two mechanisms for the support of applications with QoS requirements.

The first mechanism, designated the enhanced distributed channel access (EDCA), is applicable to the OBand and DBand and delivers traffic based on differentiating user priorities (UPs). This differentiation is achieved by varying the following for different UP values:

— Amount of time a STA senses the channel to be idle before backoff or transmission, or

— The length of the contention window to be used for the backoff, or

— The duration a STA may transmit after it acquires the channel.

These transmissions may also be subject to certain channel access restrictions in the form of admission control. In the DBand, EDCA is used only within CBAPs. Details of this mechanism are provided in 9.9.1 (HCF contention-based channel access (EDCA)) and, for the DBand, additional details are provided in 9.23.5 and 9.23.6.2.

The second mechanism, designated the hybrid coordination function (HCF) controlled channel access (HCCA), is applicable only to the OBand and allows for the reservation of transmission opportunities (TXOPs) with the hybrid coordinator (HC). A STA based on its requirements requests the HC for TXOPs, both for its own transmissions as well as for transmissions from the AP to itself.13 The request is initiated by the station management entity (SME) of the STA. The HC, which is collocated at the AP, either accepts or rejects the request based on an admission control policy. If the request is accepted, the HC schedules TXOPs for both STAs (both the AP and the non-AP STA). For transmissions from the non-AP STA, the HC polls the STA based on the parameters supplied by the STA at the time of its request. For transmissions to the STA, the AP directly obtains TXOPs from the collocated HC and delivers the queued frames to the STA, again based on the parameters supplied by the STA. Details of the mechanism are provided in 9.9.2 (HCCA) and 11.4 (TS operation). This mechanism may be used for applications such as voice and video, which may need periodic service from the HC. If the application constraints dictate the use of this mechanism, the application initiates this mechanism by using the management service primitives.

The third mechanism, designated the service period (SP) access, is applicable only to the DBand and allows for the reservation of channel time with the PCP/AP. A STA based on its requirements requests the PCP/AP for SPs, which can be used for transmission to any other STA in the BSS. The request is initiated by the SME of the STA. The PCP/AP either accepts or rejects the request based on an admission control policy. If the request is accepted, the PCP/AP uses the Extended DBand Schedule element to schedule SPs for communication between the source and destination STAs indicated within the request. Details of this mechanism are provided in 9.23.6.1, 9.23.6.3, 9.23.6.5, and 11.4.

The fourth mechanism, designated as dynamic allocation, is applicable only to the DBand and allows for near real-time reservation of channel time with the PCP/AP. This type of access is used on top of the service period access and contention-based access period. A PCP/AP can poll a STA and receive requests for channel time allocation. Based on the received requests, the PCP/AP can accept a request and immediately allocate channel time for the STA to communicate with another STA by using a Grant frame. Details of this mechanism are provided in 9.23.7, 9.23.8, and 9.23.9.

In the OBand, Nnon-QoS STAs may associate in a QoS BSS, if allowed to associate by the AP. All directed frames that are sent to non-QoS STAs by an AP do not use the frame formats associated with the QoS facility.

In the OBand, Aa QoS STA associated in a non-QoS BSS acts as a non-QoS STA.

CID: 588

7.1.3.1.3 To DS and From DS fields

Editor Note: make the following changes to Table 7-2 (To/From DS combinations in data frames)

| To DS and From DS values | Meaning |
| --- | --- |
| To DS = 0 From DS = 0 | A data frame direct from one STA to another STA within the same IBSS or the same PBSS, or a data frame direct from one non-AP STA to another non-AP STA within the same infrastructure BSS, as well as all management, extension and control frames. |
| To DS = 1 From DS = 0 | A data frame destined for the DS or being sent by a STA associated with an AP to the Port Access Entity in that AP. |
| To DS = 0 From DS = 1 | A data frame exiting the DS or being sent by the Port Access Entity in an AP. |
| To DS = 1 From DS = 1 | A data frame using the four-address format. This standard does not define procedures for using this combination of field values. |

CID: 954

7.3.1.9 Status Code field

Editor Note: change the second paragraph as follows

If an operation is successful, then the status code is set to 0. In the DBand, a status code of 85 also indicates a successful operation. If an operation results in failure, the status code indicates a failure cause. The failure cause codes are defined in Table 7-23 (Status codes). Table 7-23 also specifies whether a given status code is applicable to the OBand, DBand or both.

Editor Note: change Table 7-23 as follows

| Status Code | Meaning | OBand | DBand |
| --- | --- | --- | --- |
| 0 | Successful | Yes | Yes |
| 1 | Unspecified failure | Yes | Yes |
| 2 | TDLS wakeup schedule rejected but alternative schedule provided | Yes | No |
| 3 | TDLS wakeup schedule rejected | Yes | No |
| 4 | Reserved | Yes | Yes |
| 5 | Security disabled | Yes | No |
| 6 | Unacceptable lifetime | Yes | No |
| 7 | Not in same BSS | Yes | No |
| 8-9 | Reserved | Yes | Yes |
| 10 | Cannot support all requested capabilities in the Capability Information field | Yes | No |
| 11 | Reassociation denied due to inability to confirm that association exists | Yes | Yes |
| 12 | Association denied due to reason outside the scope of this standard | Yes | Yes |
| 13 | Responding STA does not support the specified authentication algorithm | Yes | Yes |
| 14 | Received an Authentication frame with authentication transaction sequence number out of expected sequence | Yes | Yes |
| 15 | Authentication rejected because of challenge failure | Yes | Yes |
| 16 | Authentication rejected due to timeout waiting for next frame in sequence | Yes | Yes |
| 17 | Association denied because AP is unable to handle additional associated STAs | Yes | Yes |
| 18 | Association denied due to requesting STA not supporting all of the data rates in the BSSBasicRateSet parameter | Yes | Yes |
| 19 | Association denied due to requesting STA not supporting the short preamble option | Yes | No |
| 20 | Association denied due to requesting STA not supporting the PBCC modulation option | Yes | No |
| 21 | Association denied due to requesting STA not supporting the Channel Agility option | Yes | No |
| 22 | Association request rejected because Spectrum Management capability is required | Yes | Yes |
| 23 | Association request rejected because the information in the Power Capability element is unacceptable | Yes | Yes |
| 24 | Association request rejected because the information in the Supported Channels element is unacceptable | Yes | Yes |
| 25 | Association denied due to requesting STA not supporting the Short Slot Time option | Yes | No |
| 26 | Association denied due to requesting STA not supporting the DSSS-OFDM option | Yes | No |
| 27 | Association denied because the requesting STA does not support HT features | Yes | No |
| 28 | R0KH unreachable | Yes | Yes |
| 29 | Association denied because the requesting STA does not support the phased coexistence operation (PCO) transition time required by the AP | Yes | No |
| 30 | Association request rejected temporarily; try again later | Yes | Yes |
| 31 | Robust Management frame policy violation | Yes | Yes |
| 32 | Unspecified, QoS-related failure | Yes | Yes |
| 33 | Association denied because QoS AP or PCP has insufficient bandwidth to handle another QoS STA | Yes | Yes |
| 34 | Association denied due to excessive frame loss rates and/or poor conditions on current operating channel | Yes | Yes |
| 35 | Association (with QoS BSS) denied because the requesting STA does not support the QoS facility | Yes | No |
| 36 | Reserved | Yes | Yes |
| 37 | The request has been declined | Yes | Yes |
| 38 | The request has not been successful as one or more parameters have invalid values | Yes | Yes |
| 39 | The TS has not been created because the request cannot be honored; however, a suggested TSPEC/Extended DBand TSPEC is provided so that the initiating STA may attempt to set another TS with the suggested changes to the TSPEC/Extended DBand TSPEC | Yes | Yes |
| 40 | Invalid element, i.e., an element defined in this standard for which the content does not meet the specifications in Clause 7 (Frame formats) | Yes | Yes |
| 41 | Invalid group cipher | Yes | Yes |
| 42 | Invalid pairwise cipher | Yes | Yes |
| 43 | Invalid AKMP | Yes | Yes |
| 44 | Unsupported RSN element version | Yes | Yes |
| 45 | Invalid RSN element capabilities | Yes | Yes |
| 46 | Cipher suite rejected because of security policy | Yes | Yes |
| 47 | The TS has not been created; however, the PCP/HC may be capable of creating a TS, in response to a request, after the time indicated in the TS Delay element | Yes | Yes |
| 48 | Direct link is not allowed in the BSS by policy | Yes | No |
| 49 | The Destination STA is not present within this BSS | Yes | Yes |
| 50 | The Destination STA is not a QoS STA | Yes | No |
| 51 | Association denied because the ListenInterval is too large | Yes | Yes |
| 52 | Invalid FT Action frame count | Yes | Yes |
| 53 | Invalid pairwise master key identifier (PMKID) | Yes | Yes |
| 54 | Invalid MDE | Yes | Yes |
| 55 | Invalid FTE | Yes | Yes |
| 56 | Requested TCLAS processing is not supported by the PCP/AP | Yes | Yes |
| 57 | The PCP/AP has insufficient TCLAS processing resources to satisfy the request | Yes | Yes |
| 58 | The TS has not been created because the request cannot be honored; however, the PCP/HC suggests the STA to transition to other BSSs to setup the TS. | Yes | Yes |
| 59 | Disassociated due to external service requirements | Yes | Yes |
| 60 | No outstanding GAS request | Yes | Yes |
| 61 | GAS Response not received from the Advertisement server | Yes | Yes |
| 62 | STA timed out waiting for GAS Query Response | Yes | Yes |
| 63 | GAS Response larger than query response length limit | Yes | Yes |
| 64 | Request refused because home network does not support request | Yes | Yes |
| 65 | Advertisement server in the network is not currently reachable | Yes | Yes |
| 66 | Reserved | Yes | Yes |
| 67 | Request refused due to permissions received via SSPN interface | Yes | Yes |
| 68 | Request refused because PCP/AP does not support unauthenticated access | Yes | Yes |
| 69-71 | Reserved | Yes | Yes |
| 72 | Invalid contents of RSNE | Yes | Yes |
| 73 | U-APSD Coexistence is not supported | Yes | No |
| 74 | Requested U-APSD Coexistence mode is not supported | Yes | No |
| 75 | Requested Interval/Duration value cannot be supported | Yes | Yes |
| 76 | Authentication is rejected because an Anti-Clogging Token is required | Yes | Yes |
| 77 | Authentication is rejected because the offered finite cyclic group is not supported | Yes | Yes |
| 78 | The request has not been successful as the STA could not find the appropriate TBTT | Yes | Yes |
| 79 | Transmission Failure | Yes | Yes |
| 80 | Requested TCLAS Not Supported | Yes | Yes |
| 81 | TCLAS Resources Exhausted | Yes | Yes |
| 82 | Rejected with suggested BSS Transition | Yes | Yes |
| 83 | Reject with recommended schedule | No | Yes |
| 84 | Reject because no wakeup schedule specified | No | Yes |
| 85 | Success, the destination DBand STA is in power save mode | No | Yes |
| 86 | FST pending, in process of acquiring resources from PCP/AP | Yes | Yes |
| 87 | Performing FST now | Yes | Yes |
| 88 | FST pending, gap(s) in Block Ack window | Yes | Yes |
| 89 | Reject because of U-PID setting | No | Yes |
| 8390-65 535 | Reserved | Yes | Yes |

CID: 954

7.1.4 Duration/ID field (QoS STA)

Editor instruction: delete the instruction “.11 Editor’s instructions: replace “TXOP” with “TXOP and SP” in subclause 7.1.4” and insert the text below

Editor instructions: change the title of the subclause as follows

7.1.4.3 Setting for QoS CF-Poll frames (in the OBand)

Editor instructions: change the title of the subclause as follows

7.1.4.4 Setting for frames sent by a TXOP holder under HCCA (in the OBand)

Editor instructions: change the title of the subclause as follows

7.1.4.5 Settings within a PSMP sequence (in the OBand)

Editor instructions: change the title of the subclause as follows

7.1.4.6 Settings within a dual CTS sequence (in the OBand)

7.1.4.8 Setting for other response frames

Editor instructions: change the first paragraph as follows

For any frame transmitted by a STA that is not the TXOP holder, is not specified by 7.1.4.1, and is not specified by 7.1.4.1 (General) through 7.1.4.7 (Setting for control response frames), the Duration/ID field is set to the value obtained from the Duration/ID field of the frame that elicited the response minus the time, in microseconds, between the end of the PPDU carrying the frame that elicited the response and the end of the PPDU carrying the frame.