IEEE P802.11
Wireless LANs

| Figure 8-4ae1 Update |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2011-03-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Stephen McCann | Research in Motion | 200 Bath Road, Slough, Berkshire, SL1 3XE, UK | +44 1753 667099 | smccann@rim.com |

Abstract

This document contains a proposed update to Figure 8-4ae1 based on TGae draft 2.0.

It addresses comments 1047, 1223, 1241, 1258, 1259, 1308, 1309, 1310 and 1337

Modify the following sub-clause:

## 3.3 Abbreviations and acronyms

Insert the following new acronym:

IQMF		Individually-addressed quality of service management frame

QMF		Quality of service management frame

MQMF		Multicast-addressed quality of service management frame

QACM		QMF Access Category Mapping

Modify the following sub-clause:

#### 8.4.2.ae1 QMF Policy element

The QMF Policy element defines a QMF access category mapping (QACM) of management frames and is used to advertise and exchange QMF policy between STAs. The use of the QMF Policy element is given in clause 10.ae1. See Figure 8-4ae1.

|  | Element ID | Length | QACM #1 (optional) |  | QACM #N (optional) |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | variable |  | variable |

Figure 8-4ae1—QMF Policy element format

The Element ID field is equal to the QMF Policy value in Table 8-51.

The Length is a 1-octet field whose value is dependent on the number and size of the QACM fields present in the element and is between 1 and 255.

The QACM field specifies a group of management frames and their associated access categories. See Figure 8-4ae2.

|  | QACM Header | Access Category Value (optional) | Action Value Bitmap (optional) |
| --- | --- | --- | --- |
| Octets: | 2 | 1 | Variable |

Figure 8-4ae2—QACM Field format

The format of the QACM Header subfield of the QACM field is defined in Figure 8-4ae3.

| B0 B1 | B2 B7 | B8 | B9 | B10 B11 | B12 B15 |
| --- | --- | --- | --- | --- | --- |
| QACM Field Type | QACM Field Length | I | G | ACI | Management Frame Subtype |

Figure 8-4ae3 – QACM Header subfield format

The QACM Field Type subfield is 2 bits in length and defines the structure of the QACM field. It is set to 0. Values 1, 2, and 3 are reserved.

The QACM Field Length subfield is 6 bits in length and defines the length in octets of the QACM field excluding the QACM Header subfield.

The Individually Addressed subfield (I) is one bit in length and when set to 1 indicates that the QACM applies to individually addressed management frames, otherwise it is set to 0.

The Group Addressed subfield (G) is one bit in length and when set to 1 indicates that the QACM applies to group addressed management frames, otherwise it is set to 0.

The combination of I=0 and G=0 is not allowed.

The ACI subfield is 2 bits in length. Indicated management frames are sent using the access category defined in the ACI subfield.

The Management Frame Subtype subfield is 4 bits in length. It indicates the subtype of management frames that are sent using the access category indicated in the ACI subfield. The valid values for this subfield are the subtypes in Table 8-1 that correspond to management frames.

The Access Category Value subfield is 8 bits in length and indicates the access category value as defined in 8.4.1.11 of Action frames that are sent using the access category indicated in the ACI subfield. The Access Category Value subfield is only included when the Management Frame Subtype subfield indicates Action or Action No Ack subtype as specified in 10.ae1.3QMF and the QACM Field Length subfield is greater than or equal to 2.

The Action Value Bitmap subfield is of variable length and indicates the action values as defined in 8.5 for the corresponding Access Category Value that are sent using the access category indicated in the ACI subfield. The Action Value Bitmap subfield is only included when the Management Frame Subtype subfield indicates Action or Action No Ack subtype and the QACM Field Length subfield is greater than or equal to 3. Each bit in the Action Value Bitmap subfield is mapped to the corresponding action value. The Action Value Bitmap subfield is zero padded to complete any incomplete octet. When included, the size in octets of the Action Value Bitmap field is found by subtracting 2 from the value of the QACM Field Length subfield.