IEEE P802.11
Wireless LANs

| Comment Resolution for VHT BSS Load element |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2011-03-21 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Daewon Lee | LG Electronics | LG R&D Complex 533, Hogye-1dong, Dongan-Gu, Anyang-Shi, Kyungki-Do, 431-749, Korea | +82-31-450-7897 | daewon.lee@lge.com |
| Yongho Seok | LG Electronics | LG R&D Complex 533, Hogye-1dong, Dongan-Gu, Anyang-Shi, Kyungki-Do, 431-749, Korea | +82-31-450-1497 | yongho.seok@lge.com |
| Illsoo Sohn | LG Electronics | LG R&D Complex 533, Hogye-1dong, Dongan-Gu, Anyang-Shi, Kyungki-Do, 431-749, Korea | +82-31-450-1882 | illsoo.sohn@lge.com |
| Eunsun Kim | LG Electronics | LG R&D Complex 533, Hogye-1dong, Dongan-Gu, Anyang-Shi, Kyungki-Do, 431-749, Korea | +82-31-450-1860 | esun.kim@lge.com |
| Youhan Kim | Atheros | 1700 Technology Drive, San Jose, CA 95110 | +1-408-830-5835 | youhan.kim@atheros.com |
| Matthew Fischer | Broadcom | 190 Mathilda Place, Sunnyvale, CA 94086 | +1-408-543-3370 | mfischer@broadcom.com |
| Brian Hart | Cisco Systems | 170 W Tasman Dr, San Jose, CA, 95134, USA | 1-408-5253346 | brianh@cisco.com |
| Jae Seung Lee | ETRI | 161 Gajeong-dong, Yuseong-gu, Daejeon, Korea | +82 42 860 1326 | jasonlee@etri.re.kr |
| Robert Stacey | Intel | 2111 NE 25th Ave, Hillsboro OR, 97124, USA | +1 503 724 0893 | robert.j.stacey@intel.com |
| Simone Merlin | Qualcomm | 5665 Morehouse Dr., San Diego, CA 92121 |  | smerlin@qualcomm.com |
| Raja Banerjea | Marvell | 5488 Marvell Lane, Santa Clara CA, 95054 | 1-408-222-3713 | rajab@marvell.com |
| Chunhui (Allan) Zhu | Samsung Electronics | 75 W. Plumeria Dr., San Jose, CA 95131 | +1-408-544-5667 | c.zhu@samsung.com |
| Bo Sun | ZTE Corporation | ZTE Building, #10 South Tangyan Rd., Xi'an, China | 86 29 88724130 | sun.bo1@zte.com.cn |
| Liwen Chu | ST-Ericsson | 2525 Augustine Drive, Santa Clara CA 95054 | +1.408.467.8436 | Liwen.Chu@st.com |
| Edward Au | Huawei Technologies | Huawei Industrial Base, Bantian Longgang, Shenzhen 518129, China | +1 773 782 6875 | edwardau@huawei.com |
| Vish Ponnampalam | Mediatek | 2860 Junction Ave, San Jose, CA 95134, USA | +1-408-526-1899 | vish.ponnampalam@mediatek.com |

Abstract

Submission for Draft P802.11ac_D0.1 comment resolution. The document address various comments related to VHT BSS Load element, which in principle was agreed in TGac.

Submission addresses Draft P802.11ac_D0.1 comments 1260, 1443, 1444, 1529, 1530, 1259, 1042, 1458, 707, 1771, 771, and 1152.

Introduction

## Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGac Draft.  This introduction is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGac Draft (i.e. they are instructions to the 802.11 editor on how to merge the text with the baseline documents).

TGac Editor:  Editing instructions preceded by “TGac Editor” are instructions to the TGac editor to modify existing material in the TGac draft.   As a result of adopting the changes, the TGac editor will execute the instructions rather than copy them to the TGac Draft.

Submission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or provide context.

| CID | Commenter Name | Subclause | Page | Line | Comment Type | Comment | SuggestedRemedy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1260 | Stephens, Adrian | 7.3.2.63 | 34 | 44 | TR | Has anybody put forward a compelling case for the need for this information? We currently have unspecified information having an unspecified effect. | Remove VHT BSS Load element, or make it useful. |
| 1443 | Kang, Byeongwoo | 7.3.2.63 | 34 | 44 | TR | BW utilization' and 'SS utilization' are not finalized. 11ac supports up to 8 SS and up to 160MHz as well as 80+80MHz Transmission. VHT BSS load element format (Figure 7-12) needs some modification. | Add definition of 'BW field' and 'SS utilization' |
| 1444 | Kang, Byeongwoo | 7.3.2.63 | 34 | 44 | TR | The performance of MU-MIMO can be enhanced if MU-MIMO capable STAs can choose their associatiation with BSS with more MU-MIMO capable advanced STAs. However, VHT BSS load element does not support this. | Add a field for 'the number of MU-MIMO capable stations' in VHT BSS load element format (Figure 7-12). |
| 1529 | SOHN, ILLSOO | 7.3.2.63 | 34 | 44 | TR | bandwidth utilization" and "spatial stream utilization" have not defined yet, which are important information indicating the load of network serving VHT STAs. | Define "bandwidth utilization field" and "spatial stream utilization". |
| 1530 | SOHN, ILLSOO | 7.3.2.63 | 34 | 44 | TR | In legacy BSS Load element, there exists a field that indicates the number of associated STAs. This information gives critical indication for the loading of BSS. For the same reason, VHT BSS also requires STA count but in this case MU-MIMO capable STA count. This information together with ''bandwidth utilization" and "spatial stream utilization" provides a method to provide an exact status of BSS load. | Define a field 'MU-MIMO capable STA count' in VHT BSS load element format. |
| 1259 | Stephens, Adrian | 7.3.2.63 | 34 | 46 | ER | "additional". I suppose this is in the same vein as "persil washes whiter". | Remove "additional" |
| 1042 | Seok, Yongho | 7.3.2.63 | 34 | 52 | TR | No details on Bandwidth Utilization and Spatial Stream Underutilization field. | Define the field. |
| 1458 | Lee, Daewon | 7.3.2.63 | 34 | 53 | TR | fill in TBD of bss load element | finalize section |
| 707 | Kneckt, Jarkko | 7.3.2.63 | 34 | 57 | ER | TBD should not exists in the standard | Define appropriate value for TBD fields. |
| 1771 | Lee, Jae Seung | 7.3.2.63 | 34 | 58 | ER | Duplicated Figure number. Figure number 7-12 is already used in the 802.11 baseline document | Change figure number 7-12 to other non duplicated number |
| 771 | Liu, Yong | 7.3.2.63 | 34 | 63 | TR | complete the TBD fields | as comment |
| 1152 | Stacey, Robert | 7.3.2.63 | 34 | 63 | TR | Remove editor's note |  |

Discussion:

In the past November meeting TGac agreed to support BSS load element for VHT, where the load element address some of the deficiencies of load element did not cover. The proposal is cover not only the time resource, which was the only resource the legacy system had control over, but also cover frequency and spatial resources the AP has control over.

In order to effectively give information over spatial and frequency utilization, the proposal to define a utility metric for spatial domain (which can be allocated to each user in case of MU-MIMO), and bandwidth metric for each of the bandwidth the BSS supports.

The VHT BSS load element should in general keep existing BSS load indication for legacy load balancing, and also support extra BSS load element to benefit 11ac devices. The desired characteristics of the element would be;

- Non duplicate information from legacy BSS load element

- Information on (under)utilized level of AP’s potential MU-MIMO capacity

- Information on bandwidth utilization

Figure  1. Example of resource utilization in a BSS

An example of under-utilized spatial streams is depicted in figure above. In order to define a spatial stream utility metric we need to define 3 states of medium {Idle, Busy : STA-to-AP, AP-to-legacy STA, non-self AP-to-STA, Partially Busy : self AP-to-11ac STA }. The new BSS load element can  indicate level of idleness (under-utilization) during partial busy durations.

Proposed Resolution:

Agree in principle to finalize the TBD fields as described in Draft P802.11ac_D0.1 comments 1260, 1443, 1444, 1529, 1530, 1259, 1042, 1458, 707, 1771, 771, and 1152.

Also agree to following editing instructions to complete the section.

## Editing instructions:

7.2.3.1 Beacon frame format

TGac editor: Insert the Extended BSS load element in Table 7-8 Beacon frame body after order 41 and before order Last:

Table 7-8—Beacon frame body

| Order | Information | Notes |
| --- | --- | --- |
| <ANA> | Extended BSS load element | The Extended BSS Load element is present if dot11QosOptionImplemented and dot11QBSSLoadImplemented and dot11VHTOptionImplemented are true. |

7.2.3.9 Probe Response frame format

TGac editor: Insert the Extended BSS load element in Table 7-15 Probe Response frame body after order 39 and before order Last-l:

Table 7-15—Probe Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| <ANA> | Extended BSS load element | The Extended BSS Load element is present if dot11QosOptionImplemented and dot11QBSSLoadImplemented and dot11VHTOptionImplemented are true. |

- 7.3.2 Information elements

TGac editor: Insert the following row in table 8-50, and renumber the reserved values accordingly:

Table 7-16—Element IDs

| Element | Element ID | Length (in octets) | Extensible |
| --- | --- | --- | --- |
| Extended VHT BSS Load | < ANA > | 7 | Yes |

TGac editor: Change the section title of 7.3.2.63 as shown below:

7.3.2.63 Extended VHT BSS Load element

TGac editor: Replace VHT BSS load element format figure in Figure7-12 as shown:

| Element ID | Length (5) | MU-MIMO capable STA count | Spatial Stream Under-Utility | 40MHz Busy Bandwidth Utility | 80MHz Busy Bandwidth Utility | 160MHz Busy Bandwidth Utility |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: 1 | 1 | 1 | 1 | 1 | 1 | 1 |

Figure 7-12—Extended BSS Load element format

TGac editor: On page 34 line 32, remove the Editor’s Note and insert a new paragraph as follows:

The MU-MIMO capable STA Count field is interpreted as an unsigned integer that indicates the total number of STAs with MU Rx capability currently associated with this BSS.

The spatial stream under-utility is defined as the percentage of time, linearly scaled with 255 representing 100%, that the AP has underutilized spatial domain resources for given busy time of the medium. When more than one channel is in use for the BSS, the spatial stream under-utility value is calculated only for the primary channel. This percentage is computed using the formula,

Spatial Stream Under-Utility = Integer( (max_supported_ss* channel_busy_time – utilized_ss_time)/(channel_busy_time* max_supported_ss) ) × 255),

where

channel_busy_time is defined to be the number of microseconds during which the CS mechanism, as defined in 9.2.0b.2 (CS mechanism) has indicated a channel busy indication.

utilized_ss_time is defined to be the sum of number of microseconds during which the CS mechanism has indicated channel busy indication due to transmission of MU-MIMO PPDU by the AP multiplied by number of spatial streams for the MU-MIMO transmission.

max_supported_ss is define to be the maximum number of spatial streams indicated by the VHT capability of the AP.

40, 80, or 160MHz Busy Bandwidth Utility is defined as the percentage of time, linearly scaled with 255 representing 100%, that the valid 40, 80, or 160 MHz bandwidth medium available to the BSS was busy. This percentage is computed using the formula,

40MHz Busy Bandwidth Utility = Integer( (busy_bandwidth_40/channel_busy_time) × 255 ),

80MHz Busy Bandwidth Utility = Integer( (busy_bandwidth_80/channel_busy_time) × 255 ),

160MHz Busy Bandwidth Utility = Integer( (busy_bandwidth_160/channel_busy_time) × 255 ),

where

channel_busy_time is defined to be the number of microseconds during which the CS mechanism, as defined in 9.2.0b.2 (CS mechanism) has indicated a channel busy indication.

busy_bandwidth_40, busy_bandwidth_80, and busy_bandwidth_160 is defined to be the sum of number of microseconds during which the CS mechanism has indicated channel busy for the valid 40MHz, 80MHz, 160MHz channel respectively. 80+80MHz transmission shall be considered 160MHz during utility computation context. The 40MHz channel was detected to be busy by the CS mechanism when any of the primary or the secondary parameter of the channel-list indicates busy. The 80MHz channel was detected to be busy by the CS mechanism when any of the primary, the secondary, or the secondary40 parameter of the channel-list indicates busy. The 160MHz  channel was detected to be busy by the CS mechanism when any of the primary, the secondary, the secondary40, or secondary 80 parameter of the channel-list indicates busy.

In case the AP does not support 160MHz, the 160MHz Busy Bandwidth Utility field shall be reserved. In case the AP does not support 80MHz, the 80MHz Busy Bandwidth Utility field shall be reserved. In case the AP does not support 40MHz, the 40MHz Busy Bandwidth Utility field shall be reserved.