IEEE P802.11
Wireless LANs

| Changes to clause 4 from LB172 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2011-03-13 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Santosh Pandey | Cisco Systems | 170 W Tasman Dr, San Jose, CA, 95134, USA | +1-408-8535990 | sanpande@cisco.com |

Abstract

Submission for comments on clause 4 in LB 170

The editing instructions are shown in bold italic. Four editing instructions are used: change, delete, insert, and replace. Change is used to make corrections in existing text or tables. The editing instruction specifies the location of the change and describes what is being changed by using strikethrough (to remove old material) and underscore (to add new material). Delete removes existing material. Insert adds new material without disturbing the existing material. Insertions may require renumbering. If so, renumbering instructions are given in the editing instruction. Replace is used to make changes in figures or equations by removing the existing figure or equation and replacing it with a new one. Editorial notes will not be carried over into future editions because the changes will be incorporated into the base standard.

This amendment’s baseline is Draft P802.11ae_D2.00.doc

4. General description

4.5 Overview of the services

4.5.6 Traffic differentiation and QoS support

Insert the following new subclause at the end of clause 4.5.6:

4.5.6.ae1 Management Frame QoS support

When operating with management frame QoS (MFQ), some management frames might be transmitted using an access category other than AC_VO in order to improve the quality of service of other traffic. This is achieved by the use of an MFQ policy. An MFQ policy defines the access categories of different management frames. Only QoS STAs are able to implement MFQ policy. A QoS STA that supports MFQ uses the default MFQ policy or the negotiated MFQ policy with the peer MFQ STA to transmit management frames to the peer MFQ STA. A QoS STA that supports MFQ uses AC_VO to transmit management frames to STAs that do not support MFQ.