IEEE P802.11
Wireless LANs

| Changes to subclause 10.ae1.1 from LB172 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2011-03-13 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Santosh Pandey | Cisco Systems | 170 W Tasman Dr, San Jose, CA, 95134, USA | +1-408-8535990 | sanpande@cisco.com |

Abstract

Submission for comments on subclause 10.ae1.1 in LB 170

The editing instructions are shown in bold italic. Four editing instructions are used: change, delete, insert, and replace. Change is used to make corrections in existing text or tables. The editing instruction specifies the location of the change and describes what is being changed by using strikethrough (to remove old material) and underscore (to add new material). Delete removes existing material. Insert adds new material without disturbing the existing material. Insertions may require renumbering. If so, renumbering instructions are given in the editing instruction. Replace is used to make changes in figures or equations by removing the existing figure or equation and replacing it with a new one. Editorial notes will not be carried over into future editions because the changes will be incorporated into the base standard.

This amendment’s baseline is Draft P802.11ae_D2.00.doc

10. MLME

Insert the following new subclauses at the end of clause 10:

- 10.ae1 Management Frame QoS

- 10.ae1.1 General

An MFQ STA shall set dot11MFQActivated and dot11QosOptionImplemented to true. The MFQ STA shall assign an Access Category to each management frame according to the access category assignments indicated in the MFQ policy configured using configuration procedures described in 10.ae1.2 (Management frame QoS policy configuration procedures).

An MFQ STA shall set the MFQActivated subfield in the Extended Capabilities element to one.

When the MFQActivated subfield is zero in the most recently received Extended Capabilities element from a destination STA or when an Extended Capabilities element has not been received from a destination STA, a transmitting MFQ STA shall transmit individually addressed management frames to that destination STA using the access category AC_VO.

If the MFQActivated subfield in the most recently received Extended Capabilities element from a destination STA has a value equal to one, but no Management Frame QoS Policy element (MFQ Policy element) has been received from the destination STA, then an MFQ STA shall transmit all IMFQ frames to the destination STA using the default MFQ policy access categories defined in Table 10-ae1. Otherwise, the MFQ STA shall transmit individually addressed management frames as defined in the MFQ policy element accepted from the destination MFQ STA. In either case, the transmitted management frames are IMFQ frames, and the transmitting MFQ STA shall indicate the access category used to transmit the frame in the ACI subfield of the sequence number field.

An MFQ STA in an unassociated state shall transmit all group addressed MFQ frames using default MFQ policy access categories defined in Table 10-ae1. An associated MFQ STA shall follow the MFQ policy dictated by its associated AP for transmitting group addressed management frames as described in 10.ae1.2 (Management frame QoS policy configuration procedures). The specific access category assignments of different management frames within a non-default MFQ policy are beyond the scope of this document.

- 10.ae1.1.1 Default management frame QoS policy

The default MFQ policy is as defined in Table 10-ae1. It defines the access category of each management frame based on management subtype value, category value and action value. The access category of management frames not included in this table is AC_BE.

EDITORIAL NOTE: Entries for "WNM", "Unprotected WNM", and "TDLS" are included in the table in anticipatino of 802.11v and 802.11z being integrated into 802.11REVmb prior to publication.

Table 10-ae1— Default MFQ policy

| Table 10-ae1— Default MFQ policy |  |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Management Frame Subtype value from Table 8-1 | Category Value from Table 8-36 | Action Value | MFQ Access Category |
| (Re)Association Request/Response | 0000-0011 | N/A | N/A | AC_VO |
| Probe Request (individually addressed) | 0100 | N/A | N/A | AC_VO |
| Probe Request (group addressed) | 0100 | N/A | N/A | AC_BE |
| Probe Response (in response to an individually addressed Probe Reques) | 0101 | N/A | N/A | AC_VO |
| Probe Response (in response to a group addressed Probe Reques) | 0101 | N/A | N/A | AC_VO |
| Timing Advertisement | 0110 | N/A | N/A | AC_BE |
| Beacon, ATIM, Disassociation, Authentication, Deauthentication | 1000-1100 | N/A | N/A | AC_VO |
| Spectrum management | 1101 | 0 | 0-3 | AC_BE |
| Spectrum management - channel switch announcement | 1101 | 0 | 4 | AC_VO |
| QoS | 1101 | 1 | 0-3 | AC_VO |
| DLS | 1101 | 2 | 0-2 | AC_BE |
| Block Ack | 1101 | 3 | 0-2 | AC_VO |
| Public | 1101 | 4 | 0-3,5-6,8-9 | AC_BE |
| Public - extended channel switch announcement | 1101 | 4 | 4 | AC_VO |
| Public - measurement pilot | 1101 | 4 | 7 | AC_VO |
| Public – TDLS Discovery Response | 1101 | 4 | 14 | AC_VO |
| Radio measurement | 1101 | 5 | 0-5 | AC_BE |
| Fast BSS Transition | 1101 | 6 | 0-4 | AC_VO |
| HT | 1101 | 7 | 0-3 | AC_VO |
| HT | 1101, 1110 | 7 | 4-7 | AC_VO |
| SA Query | 1101 | 8 | 0-1 | AC_VO |
| Protected Dual of Public Action | 1101 | 9 | 1-2,5-6,8-9 | AC_BE |
| Protected Dual of Public Action - extended channel switch announcement | 1101 | 9 | 4 | AC_VO |
| WNM | 1101 | 10 | 0-24 | AC_BE |
| Unprotected WNM | 1101 | 11 | 0-1 | AC_BE |
| TDLS | 1101 | 12 | 0-9 | AC_BE |
| Mesh Action - HWMP Mesh Path Selection | 1101 | 13 | 64 | AC_VI |
| Mesh Action | 1101 | 13 | 0-1, 128, 192-199 | AC_BE |
| Multihop Action | 1101 | 14 | 0-1 | AC_BE |
| Self Protected | 1101 | 15 | 0-5 | AC_VI |
| Reserved (used by WFA) | 1101 | 17 | All | AC_BE |
| Vendor-specific Protected | 1101 | 126 | N/A | AC_BE |
| Vendor-specific | 1101 | 127 | N/A | AC_BE |