IEEE P802.11
Wireless LANs

| D0.1 Comment Resolution – CID 145, 148, 281, 572, 588, 1612 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: March 14, 2011 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Youhan Kim | Atheros |  |  | youhan.kim@atheros.com |

Abstract

This document provides resolution for the following CIDs on TGac D0.1:

CID 145, 148, 281, 572, 588, 1612

Edits are based on D0.1.

| CommentID | Subclause | Page | Line | CommentType | Comment | SuggestedRemedy |
| --- | --- | --- | --- | --- | --- | --- |
| 145 | 22.2.2 | 74 | 26 | TR | Define CHAN_MAT for VHT. | Add "Contains a set of compressed beamforming feedback matrices as defined in 22.3.12.2 ( Beamforming feedback matrix V) based on the channel measured during the training symbols of the received PPDU. The number of elements depends on the number of spatial streams and the number of transmit chains" |
| 1612 | 22.2.2 | 74 | 26 | TR | For the CHAN_MAT parameter: the condition column is incomplete (or has an extra "and" at the end) and the value column is TBD. | check and revise |
| 572 | 22.2.2 | 74 | 27 | TR | Section 7.3.1.32 specifies that the compressed bamforming report field shall contain the beamforming feedback matrix V given in 20.3.12.2.5. Also, the delta SNR for each subcarrier is included in case of feedback for MU. | Change 'TBD' in Value to "Contains a set of compressed beamforming feedback matrices as defined in 20.3.12.2.5 (Compressed beamforming feedback matrix) based on the channel measured during the training symbols of the received PPDU." |

Response:

CID 145

AGREE IN PRINCIPLE.  The size of the feedback is clearly specified in 22.3.12.2, thus the last sentence not needed in this table.  Also, NDP is the only PPDU format for sounding in 11ac.

CID 1612, 572

See CID 145.

Proposed Resolution Text:

TGac editor: Edit Table 22-1 as shown below:

| CHAN_MAT | FORMAT is HT_MF and CHAN_MAT_TYPE is COMPRESSED_SV | See corresponding entry in Table 20-1 | N | Y |
| --- | --- | --- | --- | --- |
|  | FORMAT is HT_MF and CHAN_MAT_TYPE is NON_COMPRESSED_SV | See corresponding entry in Table 20-1 | N | Y |
|  | FORMAT is HT_MF and CHAN_MAT_TYPE is CSI_MATRICES | See corresponding entry in Table 20-1 | N | Y |
|  | FORMAT is VHT and CHAN_MAT_TYPE is COMPRESSED_SV | TBD Contains a set of compressed beamforming feedback matrices as defined in 22.3.12.2 (Beamforming feedback matrix V) based on the channel measured during the training symbols of the received NDP. | N | Y |
|  | Otherwise | Not present |  |  |

| CommentID | Subclause | Page | Line | CommentType | Comment | SuggestedRemedy |
| --- | --- | --- | --- | --- | --- | --- |
| 148 | 22.2.2 | 76 | 35 | TR | Define CH_OFFSET |  |
| 281 | 22.2.2 | 76 | 35 | TR | CH_OFFSET/VHT is missing an otherwise | Make it all a TBD? |
| 588 | 22.2.2 | 76 | 35 | TR | CH_OFFSET is currently TBD. Note that given the BW of the BSS, location of the Primary 20 MHz and the BW of the packet (CH_BANDWIDTH), the portion of the channel used for transmission is clearly defined (Primary20, Primary40, Primary80 or the entire 160/80+80 MHz channel). Thus, CH_OFFSET is not required and is redundant. | Delete the CH_OFFSET row from Table 22-1. |

Response:

CID 588

AGREE.  The following VHT PHY MIBs given in Table 22-23 is sufficient to determine the location of primary 20 MHz, primary 40 MHz and primary 80 MHz (for 160 MHz and 80+80 MHz BSS) channels within the spectrum used by the BSS:

- dot11CurrentChannelBandwidth

- dot11CurrentChannelCenterFrequencyIndex1

- dot11CurrentChannelCenterFrequencyIndex2

- dot11CurrentPrimary20MHzChannel

Since PPDUs with BW less than the BSS BW must be transmitted using the primary 20/40/80 MHz channels, CH_OFFSET is not necessary in the TX_VECTOR.  Hence CH_OFFSET should be deleted from Table 22-1.

CID 148, 281

See CID 588