IEEE P802.11
Wireless LANs

| LB 172 Clause 6.3 comment resolutions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2011-03-17 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Stephen McCann | Research in Motion | 200 Bath Road, Slough, Berkshire, SL1 3XE, UK | +44 1753 667099 | smccann@rim.com |

### Overview

This document updates TGae 2.0 Clause 6.3 to resolved comments received from LB 172.

It addresses comments 1271

## REV HISTORY:

R0 – initial revision

Modify the following sub-clause:

## 6.3 MLME SAP interface

### 6.3.7 Associate

#### 6.3.7.5 MLME-ASSOCIATE.response

##### 6.3.7.5.2 Semantics of the service primitive

Change the primitive parameter list as shown:

The primitive parameters are as follows:

MLME-ASSOCIATE.response(
					PeerSTAAddress,
					ResultCode,
					CapabilityInformation,
					AssociationID,
					EDCA Parameter Set,
					RCPI,
					RSNI,
					RMEnabledCapabilities,
					Content of FT Authentication Elements,
					SupportedOperatingClasses,
					DSERegisteredLocation,
					HTCapabilities,
					Extended Capabilities, 
					20/40 BSS Coexistence,

TimeoutInterval,
					BSSMaxIdlePeriod,
					TIMBroadcastResponse,
					QFM Policy,
					VendorSpecificInfo

)

Insert the following new row before VendorSpecificInfo in the parameter table:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| QFM Policy | QFM Policy element | As defined in 8.4.2.ae1 (QFM Policy element) | Indicates the QFM Policy parameters of the AP. This parameter is present when dot11QFMActivated [1271] is true, and is not present otherwise. |

### 6.3.8 Reassociate

#### 6.3.8.5 MLME-REASSOCIATE.response

##### 6.3.8.5.2 Semantics of the service primitive

Change the primitive parameter list as shown:

The primitive parameters are as follows:

MLME-REASSOCIATE.response(

PeerSTAAddress,

ResultCode,

CapabilityInformation,

AssociationID,

EDCAParameterSet,

RCPI,

RSNI,

RMEnabledCapabilities,

Content of FT Authentication Information elements,

SupportedOperatingClasses,

DSERegisteredLocation,

HT Capabilities,

Extended Capabilities,

20/40 BSS Coexistence,

TimeoutInterval,

BSSMaxIdlePeriod,

TIMBroadcastResponse,

FMSResponse,

DMSResponse,

QoSMapSet,

QFM Policy,

VendorSpecificInfo

)

Insert the following new row before VendorSpecificInfo in the parameter table:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| QFM Policy | QFM Policy element | As defined in 8.4.2.ae1 (QFM Policy element) | Indicates the QFM Policy parameters of the AP. This parameter is present when dot11QFMActivated [1271] is true, and is not present otherwise. |

6.3.11 Start

#### 6.3.11.2 MLME-START.request

##### 6.3.11.2.2 Semantics of the service primitive

Change the primitive parameter list as shown:

The primitive parameters are as follows:

MLME-START.request(

SSID,

BSSType,

BeaconPeriod,

DTIMPeriod,

CF parameter set,

PHY parameter set,

IBSS parameter set,

ProbeDelay,

CapabilityInformation,

BSSBasicRateSet,

OperationalRateSet,

Country,

IBSS DFS Recovery Interval,

EDCAParameterSet,

DSERegisteredLocation,

HT Capabilities,

HT Operation,

BSSMembershipSelectorSet,

BSSBasicMCSSet,

HTOperationalMCSSet,

Extended Capabilities,

20/40 BSS Coexistence,

Overlapping BSS Scan Parameters,

MultipleBSSID,

InterworkingInfo,

AdvertisementProtocolInfo,

RoamingConsortiumInfo,

Mesh ID,

Mesh Configuration,

QFM Policy,

VendorSpecificInfo

)

Insert the following new row before VendorSpecificInfo in the parameter table:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| QFM Policy | QFM Policy element | As defined in 8.4.2.ae1 (QFM Policy element) | This element is present when dot11QFMActivated [1271] is true, and is not present otherwise, and is provided by the SME to signal to the MLME the QFM Policy to be used for this BSS. |

Modify this sub-clause as follows:

### 6.3.ae1 QMF Policy

#### 6.3.ae1.1 Introduction

The following MLME primitives support the signaling of QMF Policy.

#### 6.3.ae1.3 MLME-QMFPOLICY.confirm

##### 6.3.ae1.3.1 Function

This primitive reports the QMFreceipt of a QMF Policy frame.

##### 6.3.ae1.3.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-QMFPOLICY.confirm (

Peer MAC Address,

Dialog Token,

QMF Policy parameters,

Result Code

)

| Name | Type |  | Valid Range |  | Description |  |
| --- | --- | --- | --- | --- | --- | --- |
| Peer MAC Address | MAC Address |  | Any valid individual MAC Address |  | The address of the peer MAC entity to which the QMF Policy is received. |  |
| Dialog Token | Integer |  | 1-255 |  | The dialog token to identify the QMF Policy transaction. |  |
| QMF Policy parameters |  | QMF Policy element |  | As defined in 8.4.2.ae1 (QMF Policy element) |  | These parameters indicate the QMF Policy the peer is requested to use. |
| Result Code | Enumeration |  | SUCCESS, REJECT |  | Reports the receipt of a QMF Policy Response frame. |  |

##### 6.3.ae1.3.3 When generated

This primitive is generated when a QMF Policy Response frame has been received from a peer entity.

##### 6.3.ae1.3.4 Effect of receipt

On receipt of this primitive, the SME evaluates the result code.

#### 6.3.ae1.4 MLME-QMFPOLICY.indication

##### 6.3.ae1.4.1 Function

This primitive indicates that a QMF Policy Response frame has been received from a peer entity.

##### 6.3.ae1.4.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-QMFPOLICY.indication (

Peer MAC Address,

Dialog Token,

QMF Policy parameters,

Result Code

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MAC Address | Any valid individual MAC Address | The address of the peer MAC from which the QMF Policy response was received |
| Dialog Token | Integer | 1-255 | The dialog token to identify the QMF Policy transaction. |
| QMF Policy parameters | QMF Policy element | As defined in 8.4.2.ae1 (QMF Policy element) | This element is optionally present. |
| Result Code | Enumeration | SUCCESS, REJECT, INVALID_PARAMETERS, TIMEOUT, TRANSMISSION_FAILURE, UNSPECIFIED_FAILURE | Reports the receipt of a QMF Policy response frame. |

##### 6.3.ae1.4.3 When generated

This primitive is generated by the MLME when a valid QMF Policy response frame is received from a peer entity.

##### 6.3.ae1.4.4 Effect of receipt

On receipt of this primitive, the QMF Policy response frame elements are provided to the SME to be processed.

#### 6.3.ae1.5 MLME-QMFPOLICY.response

##### 6.3.ae1.5.1 Function

This primitive requests the transmission of a QMF Policy frame to a peer entity. A QMF Policy frame may be sent unsolicited to a peer entity.

##### 6.3.ae1.5.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-QMFPOLICY.response (

Peer MAC Address,

Dialog Token,

QMF Policy parameters,

Result Code

)

| Name | Type |  | Valid Range | Description |  |
| --- | --- | --- | --- | --- | --- |
| Peer MAC Address | MAC Address |  | Any valid individual MAC Address | The address of the peer MAC entity to which the QMF Policy response is sent. |  |
| Dialog Token | Integer |  | 1-255 | The dialog token to identify the QMF Policy transaction. |  |
| QMF Policy parameters |  | QMF Policy element | As defined in 8.4.2.ae1 (QMF Policy element) |  | These parameters indicate the QMF Policy the peer is requested to use. |
| Result Code | Enumeration |  | SUCCESS, REJECT | Reports the outcome of the transaction. |  |

##### 6.3.ae1.5.3 When generated

This primitive is generated by the SME to request that an QMF Policy frame be sent to a

peer entity to convey QMF Policy information.

##### 6.3.ae1.5.4 Effect of receipt

On receipt of this primitive, the MLME constructs an QMF Policy frame containing the

set of QMF Policy elements specified. This frame is then scheduled for transmission.

#### 6.3.ae1.2 MLME-QMFPOLICYCHANGE.request

##### 6.3.ae1.2.1 Function

This primitive supports the signaling of QMF Policy between peer SMEs. An SME changes the QMF Policy within a peer SME.

##### 6.3.ae1.2.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-QMFPOLICYCHANGE.request (

Peer MAC Address,

Dialog Token,

QMF Policy parameters

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MAC Address | Any valid individual MAC Address | The address of the peer MAC entity to which the QMF Policy Change request is sent. |
| Dialog Token | Integer | 1-255 | The dialog token to identify the QMF Policy Change transaction. |
| QMF Policy parameters | QMF Policy element | As defined in 8.4.2.ae1 (QMF Policy element) | These parameters indicate the QMF Policy the peer is requested to use. |

##### 6.3.ae1.2.3 When generated

This primitive is generated by the SME when a STA wishes to request a change of the QMF Policy within a peer entity.

##### 6.3.ae1.2.4 Effect of receipt

On receipt of this primitive, the MLME constructs a QMF Policy Change request frame containing the set of QMF Policy parameters. This frame is then scheduled for transmission.

#### 6.3.ae1.3 MLME-QMFPOLICYCHANGE.confirm

##### 6.3.ae1.3.1 Function

This primitive reports the QMFreceipt of a QMF Policy Change frame.

##### 6.3.ae1.3.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-QMFPOLICYCHANGE.confirm (

Peer MAC Address,

Dialog Token,

QMF Policy parameters,

Result Code

)

| Name | Type |  | Valid Range |  | Description |  |
| --- | --- | --- | --- | --- | --- | --- |
| Peer MAC Address | MAC Address |  | Any valid individual MAC Address |  | The address of the peer MAC entity to which the QMF Policy Change is received. |  |
| Dialog Token | Integer |  | 1-255 |  | The dialog token to identify the QMF Policy Change transaction. |  |
| QMF Policy parameters |  | QMF Policy element |  | As defined in 8.4.2.ae1 (QMF Policy element) |  | These parameters indicate the QMF Policy the peer is requested to use. |
| Result Code | Enumeration |  | SUCCESS, REJECT, INVALID_PARAMETERS, TIMEOUT, TRANSMISSION_FAILURE, UNSPECIFIED_FAILURE |  | Reports the receipt of a QMF Policy Change response frame. |  |

##### 6.3.ae1.3.3 When generated

This primitive is generated when the QMF Policy Change has been received by the MAC.

##### 6.3.ae1.3.4 Effect of receipt

On receipt of this primitive, the SME evaluates the result code.

#### 6.3.ae1.4 MLME-QMFPOLICYCHANGE.indication

##### 6.3.ae1.4.1 Function

This primitive indicates that a QMF Policy Change request frame has been received from a peer entity.

##### 6.3.ae1.4.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-QMFPOLICYCHANGE.indication (

Peer MAC Address,

Dialog Token,

QMF Policy parameters,

Result Code

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MAC Address | Any valid individual MAC Address | The address of the peer MAC from which the QMF Policy Change request was received |
| Dialog Token | Integer | 1-255 | The dialog token to identify the QMF Policy Change transaction. |
| QMF Policy parameters | QMF Policy element | As defined in 8.4.2.ae1 (QMF Policy element) | This element is optionally present. |
| Result Code | Enumeration | SUCCESS, REJECT, INVALID_PARAMETERS, TIMEOUT, TRANSMISSION_FAILURE, UNSPECIFIED_FAILURE | Reports the receipt of a QMF Policy Change request frame. |

##### 6.3.ae1.4.3 When generated

This primitive is generated by the MLME when a valid QMF Policy Change Request frame is received from a peer entity.

##### 6.3.ae1.4.4 Effect of receipt

On receipt of this primitive, the QMF Policy Change Request frame elements are provided to the SME to be processed.

#### 6.3.ae1.5 MLME-QMFPOLICYCHANGE.response

##### 6.3.ae1.5.1 Function

This primitive requests the transmission of a QMF Policy Change frame to a peer entity.

##### 6.3.ae1.5.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-QMFPOLICYCHANGE.response (

Peer MAC Address,

Dialog Token,

QMF Policy parameters,

Result Code

)

| Name | Type |  | Valid Range | Description |  |
| --- | --- | --- | --- | --- | --- |
| Peer MAC Address | MAC Address |  | Any valid individual MAC Address | The address of the peer MAC entity to which the QMF Policy Change response is sent. |  |
| Dialog Token | Integer |  | 1-255 | The dialog token to identify the QMF Policy Change transaction. |  |
| QMF Policy parameters |  | QMF Policy element | As defined in 8.4.2.ae1 (QMF Policy element) |  | These parameters indicate the QMF Policy the peer is requested to use. |
| Result Code | Enumeration |  | SUCCESS, REJECT | Reports the outcome of the transaction. |  |

##### 6.3.ae1.5.3 When generated

This primitive is generated by the SME to request that a QMF Policy Change frame be sent to a peer entity to convey QMF Policy information.

##### 6.3.ae1.5.4 Effect of receipt

On receipt of this primitive, the MLME constructs a QMF Policy frame containing the set of QMF Policy elements specified. This frame is then scheduled for transmission.

#### 6.3.ae1.6 MLME-QMFPOLICYSET.request

##### 6.3.ae1.6.1 Function

This primitive supports the signaling of QMF Policy from the SME to the MLME.

#### 6.3.ae1.6.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-QMFPOLICYSET.request (

Peer MAC address,

QMF Policy parameters

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual MAC address | The address of the STA to which the QMF Policy is used for transmissions. |
| QMF Policy parameters | QMF Policy element | As defined in 8.4.2.ae1 (QMF Policy element) | This element is optionally present when dot11QMFPolicyActivated is true, and is not present otherwise. |

##### 6.3.ae1.6.3 When generated

This primitive is generated by the SME to request the MLME to set QMF Policy information.

##### 6.3.ae1.6.4 Effect of receipt

On receipt of this primitive, the MLME uses the supplied set of QMF Policy parameters when transmitting to the peer.

#### 6.3.ae1.7 MLME-QMFPOLICYSET.confirm

##### 6.3.ae1.7.1 Function

This primitive reports the result of a QMF Policy Set request operation.

##### 6.3.ae1.7.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-QMFPOLICYSET.confirm (

Result Code,

QMF Policy parameters

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Result Code | Enumeration | SUCCESS, REJECT, INVALID_PARAMETERS, UNSPECIFIED_FAILURE | Reports the outcome of a request to set QMF Policy. |
| QMF Policy parameters | QMF Policy element | As defined in 8.4.2.ae1 (QMF Policy element) | This element is optionally present when dot11QMFPolicyActivated is true, and is not present otherwise. |

##### 6.3.ae1.7.3 When generated

This primitive is generated by the MLME when the request to set QMF Policy completes.

##### 6.3.ae1.7.4 Effect of receipt

On receipt of this primitive, the SME evaluates the result code.