IEEE P802.11
Wireless LANs

| D0.1 Comment Resolution – Clause 22, Part 1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: April 14, 2011 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Youhan Kim | Atheros |  |  | youhan.kim@atheros.com |

Abstract

This document provides resolution for the following CIDs:

198, 199, 267, 298, 299, 301, 302, 303, 304, 305, 396, 565, 592, 612, 621, 636, 652, 1627

The comments are based on D0.1.

Edits for the proposed resolutions are based on D0.3.

| CommentID | Subclause | Page | Line | CommentType | Comment |  | SuggestedRemedy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 565 | 22.2.2 | 72 | 26 | TR | NON_HT_MODULATION should not be present in TXVECTOR or RXVECTOR when FORMAT is not NON_HT. | Indicate 'N' under both TXVECTOR and RXVECTOR. |  |

Response:

AGREE.

Proposed Resolution Text:

### 22.2.2 TXVECTOR and RXVECTOR parameters

| NON_HT_MODULATION | FORMAT is NON_HT | On transmit: indicates the subformat of the transmitted non-HT packet. On receive: indicates the estimated subformat of the received non-HT packet.(#279) Enumerated type: OFDM NON_HT_DUP_OFDM | Y | Y |
| --- | --- | --- | --- | --- |
|  | Otherwise | Not present | N (#565) | N (#565) |

| CommentID | Subclause | Page | Line | CommentType | Comment |  | SuggestedRemedy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 267 | 22.2.2 | 72 | 34 | TR | 1 to 4095, but with VHT-STF, VHT-LTFs, SIGB, 1 is not an allowed value | Determine the true min value and insert |  |

Response:

COUNTER.

Line 34 is for HT_MF format.  L_LENGTH is not present in TXVECTOR for VHT format.  Nevertheless, the minimum value of 1 for L_LENGTH is still incorrect for HT_MF.

Note that the shortest HT_MF packet is an NDP with only one HT-LTF symbol.  (NOTE: A VHT NDP with only one VHT-LTF symbol is not valid.  However, an HT NDP with only one HT-LTF can be used in certains cases, such as in calibration for implicit sounding.)  In case of a HT_MF NDP with only one HT-LTF symbol, there are 4 OFDM symbols (HT-SIG1/2 + HT-STF + HT-LTF) using 800 ns GI after the L-SIG.  This corresponds to L_LENGTH of 9 bytes.

Proposed Resolution Text:

### 22.2.2 TXVECTOR and RXVECTOR parameters

| L_LENGTH | FORMAT is NON_HT | Indicates the length of the PSDU in octets in the range of 1 to 4095. This value is used by the PHY to determine the number of octet transfers that occur between the MAC and the PHY. | Y | Y |
| --- | --- | --- | --- | --- |
|  | FORMAT is HT_MF | Indicates the value in the Length field of the L-SIG in the range of 9(#267) to 4095. | Y | Y |
|  | FORMAT is HT_GF | Not present | N | N |
|  | FORMAT is VHT | Not present | N | N |

| CommentID | Subclause | Page | Line | CommentType | Comment |  | SuggestedRemedy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 298 | 22.3.3 | 79 | 47 | TR | Confusing as written; should be more like "each element in a VHT PPDU can be generated …" | As in comment |  |
| 1627 | 22.3.3 | 79 | 60 | TR | segment parser is also applied to the contiguous 160MHz |  | "for non-contiguous transmission" needs to be changed into "for contiguous 160MHz and non-contiguous 80+80MHz transmission" |
| 299 | 22.3.3 | 79 | 64 | TR | IDFT is used inconsistently in different places to include pilot insertion (and duplication?) or not | Recommend IDFT excludes pilot insertion, and so add this explicitly as a step in this list and also in subsequent diagrams (e.g. blocks are renamed "pilot insertion, duplication and IDFT") |  |
| 302 | 22.3.3 | 80 | 7 | TR | Diagram for multiple segments for L-SIG and VHT-SIG-A and SIG-B is missing | Add (or constrain the figure labels) |  |
| 301 | 22.3.3 | 80 | 32 | TR | In figs 22-2 - 22-7, and also in the numbered list preceding, there are two distinct kinds of CSD yet are labelled the same - suggest that these be explicitly distinguished (i.e. CSD per STS and CSD per TX antenna) | As in comment |  |
| 303 | 22.3.3 | 80 | 35 | TR | "Multiply by 1st column of Pu" would probably be better if given its own distinct name. Also; by P99, [P]u is the first u columns of P, so "first column of [P]u likely needs more clarification. | Replace and introduce |  |
| 198 | 22.3.3 | 79 | 45 | TR | In transmitter diagrams in some places instead of FEC encoder BCC encoder can be noted. | As in comment |  |
| 304 | 22.3.3 | 80 | 65 | TR | Number of FEC encoders is missing an upper bound | Add |  |
| 305 | 22.3.3 | 81 | 6 | TR | "More outputs than inputs" - twice as many. For VHT we can be more specific | Be more specific |  |
| 592 | 22.3.3 | 82 | 21 | TR | Figure 22-2 is for both L-SIG and VHT-SIG-A. | Change "transmit process for the VHT-SIG-A" to "transmit process for the L-SIG and VHT-SIG-A". |  |
| 199 | 22.3.3 | 82 | 29 | TR | Remove "linear precoding" sicne it hints that there may be also non-linear precoding. | As in comment |  |

Response – 298, 1627:

AGREE.

Response – CID 299:

COUNTER.

Pilots need to be inserted before CSD per STS.  Hence, renaming the IDFT block to ‘pilot insertion, duplication and IDFT’ is not appropriate.  As for duplication, only Figure 22-2 (L-SIG and VHT-SIG-A) requires duplication of 20 MHz over 40/80/160 MHz.  Hence, propose to add a separate duplication block before the IDFT in Figure 22-2.

Response – CID 302:

AGREE.

The intent is not to have block diagram for all cases.  For example, block diagram for L-STF/L-LTF/VHT-STF/VHT-LTF fields are not present.  Propose to constraint the figure label instead of adding a separate figure for L-SIG and VHT-SIG-A for 80+80 MHz case.

Response – CID 301:

AGREE.

Suggest using “CSD per STS” and “CSD per chain” for shorter wording.

Response – CID 303:

COUNTER.

Agree that the notation of [P]u is confusing.  Since Figure 22-3 corresponds to VHT-SIG-B for SU packet, suggest clarifying in the figure label that this is for SU case only.   Also, there are not many other places where this block is referred to, thus coming up with a dedicated name does not seem necessary.  Suggest to modify the name to “Multiply by 1st column of P”.

Response – CID 198:

AGREE.

Note to Editor: This comment seems to be for p. 80, not p. 79.

Response – CID 304, 305:

AGREE.

Response – CID 592, 199:

AGREE.

NOTE: CID 306 has not been resolved yet.

Proposed Resolution Text:

### 22.3.3 Transmitter block diagram

Each element in a VHT PPDU can be generated using a subset of the following blocks (only select notable blocks are listed below)(#298):

- PHY Padding

- Scrambler

- Encoder parser

- FEC (BCC or LDPC(#198)) encoders

- Stream parser

- Segment parser (for contiguous 160 MHz and non-contiguous 80+80 MHz transmissions(#1627))

- BCC Interleaver

- Constellation mapper

- LDPC tone mapper

- Space time block encoder

- Cyclic shift diversity (CSD) per STS insertion(#301)

- Spatial mapper

- Duplicate over multiple 20 MHz(#299)

- Inverse discrete Fourier transform (IDFT)

- CSD per chain(#301) insertion

- Guard interval (GI) insertion

- Windowing

- Transmitter block diagram 1 (L-SIG and VHT-SIG-A using one frequency segment(#302))(#300)

- Transmitter block diagram 2 (VHT-SIG-B in SU packet(#303) using one frequency segment(#302))

- Transmitter block diagram 3 (Data field in SU packet using one frequency segment(#302))

Note:

- There may be 1 to 12(#304) FEC encoders when BCC encoding is used.

- The stream parser may have 1-8 outputs.

- For streams encoded using LDPC, the BCC interleavers are not used.

- For streams encoded using BCC, the LDPC tone mappers are not used,

- When STBC is used, the STBC block has twice as many(#305) outputs than inputs.

- When spatial mapping is used, there may be more transmit chains than space time streams.

- The number of inputs to the spatial mapper may be 1-8.

- Transmitter block diagram 4 (Data field in MU packet, with LDPC and BCC encoding using one frequency segment(#302))

- Transmitter block diagram 5 (contiguous 160 MHz)

- Transmitter block diagram 6 (non-contiguous 80+80 MHz)

Transmitter block diagram 1 (L-SIG and VHT-SIG-A using one frequency segment(#302)) through Transmitter block diagram 6 (non-contiguous 80+80 MHz) show example transmitter block diagrams. Specifically, Transmitter block diagram 1 (L-SIG and VHT-SIG-A using one frequency segment(#302)) shows the transmit process for the L-SIG and(#592) VHT-SIG-A field of a(#308) VHT packet. These transmit blocks are also used to generate the non-VHT portion of the VHT packet, except that the BCC encoder and interleaver are not used when generating the L-STF and L-LTF fields. Transmitter block diagram 2 (VHT-SIG-B in SU packet(#303) using one frequency segment(#302)) shows the transmit process for generating the VHT-SIG-B field of the VHT PPDUs. Transmitter block diagram 3 (Data field in SU packet using one frequency segment(#302)) shows the transmitter blocks used to generate the Data(#294) field of(#309) 20 MHz, 40 MHz and 80 MHz SU(#829) VHT format PPDUs. A subset of these transmitter blocks consisting of the constellation mapper and CSD blocks, as well as the blocks to the right of, and including, the spatial mapping block, are also used to generate the VHT-STF, VHT-LTF fields as illustrated in Error: Reference source not found. Transmitter block diagram 4 (Data field in MU packet, with LDPC and BCC encoding using one frequency segment(#302)) shows the transmit process for generating the Data field of DL MU-MIMO VHecoding(#199). Transmitter block diagram 5 (contiguous 160 MHz) and Transmitter block diagram 6 (non-contiguous 80+80 MHz) show the transmit process for generating the Data field of contiguous 160 MHz and non-contiguous 80+80 MHz VHT PPDUs, respectively.

| CommentID | Subclause | Page | Line | CommentType | Comment |  | SuggestedRemedy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 396 | 22.3.9.2.3 | 102 | 15 | TR | B3 is undefined for SU | Reserved 0 or reserved 1? |  |
| 612 | 22.3.9.2.3 | 102 | 57 | TR | If N_STS for the last user is 0, then B6 is reserved, not B4. | Change "B4 is reserved" to "B6 is reserved" |  |

Response – 396:

DISAGREE.

If an SU PPDU uses LDPC, then B3 is used to indicate whether extra OFDM symbol(s) were added during the LDPC encoding process.  Propose to edit the paragraph (mainly indentation) to prevent similar confusion in the future.

Response – CID 612:

AGREE.

Pilots need to be inserted before CSD per STS.  Hence, renaming the IDFT block to ‘pilot insertion, duplication and IDFT’ is not appropriate.  As for duplication, only Figure 22-2 (L-SIG and VHT-SIG-A) requires duplication of 20 MHz over 40/80/160 MHz.  Hence, propose to add a separate duplication block before the IDFT in Figure 22-2.

Proposed Resolution Text:

### 22.3.9.2.3 VHT-SIG-A definition

| B2-B3 | Coding | 2 | B2: For SU, B2 is set to 0 for BCC, 1 for LDPC For MU, if the NSTS field for user 1 is non-zero, then B2 indicates the coding used for user 1; set to 0 for BCC and 1 for LDPC. If the NSTS field for user 1 is set to 0, then this field is reserved and set to 1. B3: Set to 1 if LDPC PPDU encoding process (or at least one LPDC user’s PPDU encoding process) results in an extra OFDM symbol (or symbols) as described in Error: Reference source not found. Set to 0 otherwise. |
| --- | --- | --- | --- |
| B4-B7 | MCS | 4 | For SU: MCS index For MU: If the NSTS field for user 2 is non-zero, then B4 indicates coding for user 2: set to 0 for BCC, 1 for LDPC. If NSTS for user 2 is set to 0, then B4 is reserved and set to 1. If the NSTS field for user 3 is non-zero, then B5 indicates coding for user 3: set to 0 for BCC, 1 for LDPC. If NSTS for user 3 is set to 0, then B5 is reserved and set to 1. If the NSTS field for user 4 is non-zero, then B6 indicates coding for user 4: set to 0 for BCC, 1 for LDPC. If NSTS for user 4 is set to 0, then B6(#612) is reserved and set to 1. B7 is reserved and set to 1 |

| CommentID | Subclause | Page | Line | CommentType | Comment |  | SuggestedRemedy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 621 | 22.3.11 | 115 | 30 | TR | PHY padding computation for LDPC encoding in case of SU transmission is not defined. Equation (22-45) also applies to SU case as well in principle, with some additional clarifications. | Change "In case of MU LDPC encoding," to "In case of LDPC encoding," on line 30. On line 42, change "Equation (22-50)" to "Equation (22-50) in case of MU. N_{SYM_max_init} = N_{SYM,init} in case of SU where N_{SYM,init} is given by Equation 22-48)" |  |

Response:

AGREE.

Proposed Resolution Text:

### 22.3.11 Data field

In case of (#621) LDPC encoding, the PHY padding bits are calculated using Equation (22-45).

(22-45)

where

is the number of octets delivered by the MAC for user u and is given by Equation (22-88), and

is given by Equation (22-51) in case of MU transmissions.  In case of SU transmissions,  where  is given by Equation (22-48).(#621)

| CommentID | Subclause | Page | Line | CommentType | Comment |  | SuggestedRemedy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 636 | 22.3.11.11 | 132 | 28 | TR | Table 22-7 defines CSD for non-HT portion of packet for upto 8 antennas. No reason not to use it for non-HT duplicate packets as well. | Change the description for "T_{CS}^{i_{TX}}" to "represents the cyclic shift for transmitter chain i_{TX} with a value given in Table 22-7." |  |

Response:

AGREE.

Proposed Resolution Text:

### 22.3.11.11 Non-HT duplicate transmission

represents the cyclic shift for transmitter chain  with a value given in Table 22-7.(#636)

| CommentID | Subclause | Page | Line | CommentType | Comment | SuggestedRemedy |
| --- | --- | --- | --- | --- | --- | --- |
| 652 | J.1 | 173 | 34 | TR | Incorrect Channel center frequency index | Change "50" to "114" under "Channel center frequency index" |

Response:

AGREE.

Proposed Resolution Text:

### J.1 Country information and regulatory classes

Table J-2 – Operating classes in Europe

| Operating class | Global operating class (see Table J.4) | Channel starting frequency (GHz) | Channel spacing (MHz) | Channel set | Channel center frequency index | Behavior limits set |
| --- | --- | --- | --- | --- | --- | --- |
| <ANA> | TBD | 5 | 80 | - | 42 | VHT80MHzBehavior, IndoorOnlyBehavior, TransmitPowerControlBehavior |
| <ANA> | TBD | 5 | 80 | - | 58, 106, 122 | VHT80MHzBehavior, NomadicBehavior, TransmitPowerControlBehavior, DynamicFrequencySelectionBehavior |
| <ANA> | TBD | 5 | 160 | - | 50 | VHT160MHzBehavior, IndoorOnlyBehavior, NomadicBehavior, TransmitPowerControlBehavior, DynamicFrequencySelectionBehavior |
| <ANA> | TBD | 5 | 160 | - | 114 | VHT160MHzBehavior, NomadicBehavior, TransmitPowerControlBehavior, DynamicFrequencySelectionBehavior |
| <ANA>-255 | Reserved | Reserved | Reserved | Reserved |  | Reserved |
| NOTE—The channel spacing for operating classes <ANA> through <ANA> is for the supported bandwidth rather than the operating bandwidth. |  |  |  |  |  |  |