IEEE P802.11
Wireless LANs

| TGac Comment Resolution CID 187, 1655, 208, 1657, 1176, 209, 210, and 217 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2011-05-03 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Vinko Erceg | Broadcom | 16340 W. Bernardo Dr. San Diego, CA 92127 | +1 858 521 5885 | verceg@broadcom.com |

Abstract

This submission proposes resolutions to some of the CIDs submitted in the internal Task Group LB. The following CIDs are addressed: CID 187, 1655, 208, 1657, 1176, 209, 210, and 217.

The changes marked in this document are based on the TGac Draft 0.1

CID        Sec.         Pg.           Ln.                   Comment                               Proposal

| 187 | 11.20.3 | 63 | 64 | "frames" means PPDUs here? | Change to "PPDUs" |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Accept

Instruction to Editor: On pg. 63, ln 64-65, please modify the text as follows:

“NOTE—A STA need not set its NAV in response to 20/40/80 MHz frames PPDUs received on any channel that is not

or does not include the primary channel, even if it is capable of receiving those frames PPDUs.”

| 1176 | 22.3.11.5 | 120 | 34 | Equation (22-54) appears to be invalid for i_SS = 1 (at least the sum term doesn’t make sense) | Replace with conditional assignment for i_SS=1 and otherwise. Line 47 could then be removed too. |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Accept n Principle

| 1657 | 22.3.11.5 | 120 | 33 | There is no description about range of i_ss in Equation (22-54) | add in Equation (22-54) "i_ss=0, 1, … , N_ss" |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Accept n Principle, see resolution to CID 1176.

| 210 | 22.3.11.5 | 120 | 26-36 | Equations (22-53) and (22-4) seem to be in error. Instead of s in the denominator it should be S. Also first term in Eq (22-54) (summation) should be a function of k. | Please verify the equations. |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Accept n Principle, see resolution to CID 1176.

Instruction to Editor regarding CIDs 1176, 1657, and 210: On pg. 120, instead of Equation (22-54) please write:

i=i'=1iSS−1s+SkNESs+kmods, 2≤iSSNSSi=SkNESs+kmods, iSS=1

Instruction to Editor: remove the following sentence on pg120 ln 47:

For iSS=1, the first term in Equation has a value of 0.

Instruction to Editor: delete Equation (22-54)

| 217 | General |  |  | There are numerous TBDs in the spec, Please perform search for TBDs and define them | As in comment |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Reject

Reason for the rejection: Many, if not all, TBDs are being addressed in the comment resolution process of the Draft 0.1. Few that may remain should be identified during the LB 1.0 comment resolution process and corrected. I will personally include such a comment then.

| 208 | 22.3.11.5 | 120 | 27 | "Input k" is not clear. | Please better define "input k", its meaning. |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Accept n Principle

Resolution was already addressed in CID 1654 and implemented in D0.3.

| 1655 | 22.3.11.5 | 120 | 26 | There is no description about range of k in Equation (22-53) | add in Equation (22-53) "k=0, 1, … , N_CBPSS" |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Accept

Already implemented in D0.3

| 209 | 22.3.11.5 | 120 | 47 | Equation number is missing | Equation (22-54) |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Accept

Already implemented in D0.3