IEEE P802.11
Wireless LANs

| TGac Comment Resolution CID 187, 208, 1176, 209, 210, 217, 425 and 1656 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2011-05-03 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Vinko Erceg | Broadcom | 16340 W. Bernardo Dr. San Diego, CA 92127 | +1 858 521 5885 | verceg@broadcom.com |

Abstract

This submission proposes resolutions to some of the CIDs submitted in the internal Task Group LB. The following CIDs are addressed: CID 187, 208, 1176, 209, 210, 217, 425 and 1656.

The changes marked in this document are based on the TGac Draft 0.1

Note: Document title CIDs are now different than the resolved CIDs in the document due to revision 1 changes.

CID        Sec.         Pg.           Ln.                   Comment                               Proposal

| 187 | 11.20.3 | 63 | 64 | "frames" means PPDUs here? | Change to "PPDUs" |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Accept

Instruction to Editor: On pg. 63, ln 64-65, please modify the text as follows:

“NOTE—A STA need not set its NAV in response to 20/40/80 MHz frames PPDUs received on any channel that is not

or does not include the primary channel, even if it is capable of receiving those frames PPDUs.”

| 1176 | 22.3.11.5 | 120 | 34 | Equation (22-54) appears to be invalid for i_SS = 1 (at least the sum term doesn’t make sense) | Replace with conditional assignment for i_SS=1 and otherwise. Line 47 could then be removed too. |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree in principle. Refer to CID 1660 resolution.

| 210 | 22.3.11.5 | 120 | 26-36 | Equations (22-53) and (22-54) seem to be in error. Instead of s in the denominator it should be S. Also first term in Eq (22-54) (summation) should be a function of k. | Please verify the equations. |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Disagree. “s” in the denominator in EQ (22-53) is correct.

| 217 | General |  |  | There are numerous TBDs in the spec, Please perform search for TBDs and define them | As in comment |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Reject

Reason for the rejection: Many, if not all, TBDs are being addressed in the comment resolution process of the Draft 0.1. Few that may remain should be identified during the LB 1.0 comment resolution process and corrected. I will personally include such a comment then.

| 208 | 22.3.11.5 | 120 | 27 | "Input k" is not clear. | Please better define "input k", its meaning. |
| --- | --- | --- | --- | --- | --- |
| 425 | 22.3.11.5 | 120 | 27 | Range of k not described | Report range of k (esp. does it start at 0 or 1?) |
| 1656 | 22.3.11.5 | 120 | 33 | There is no description about range of k in Equation (22-54) | add in Equation (22-54) "k=0, 1, … , N_CBPSS" |

Proposed Resolution to CIDs 208, 425 and 1656: Agree in principle. Refer to CID 1654.

Already implemented in D0.3.

| 209 | 22.3.11.5 | 120 | 47 | Equation number is missing | Equation (22-54) |
| --- | --- | --- | --- | --- | --- |

Already implemented in D0.3