IEEE P802.11
Wireless LANs

| Comment Resolutions regarding to Information elements |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2011-05-05 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Chen Sun | NICT | 3-4, Hikarino-oka, Yokosuka, Kanagawa, Japan, 239-0847 | +81 45 847 5062 | sun@nict.go.jp |
| Hiroshi Harada | NICT |  |  | harada@nict.go.jp |

Abstract

This document present resolutions for comments 1179, 1180, 118.

| CID | Clause Number(C) | Page(C) | Line(C) | Type of Comment | Page | Line | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1179 | 8.4.2 | 27 | 18 | T | 27.18 | 18 | 8.4.2 | This csubclause lacks description of channel availability query and network channel control information elements | Add descriptions to this subclause. | Agree and revise as commented |
| 1180 | 8.4.2.1 | 27 | 28 | T | 27.28 | 28 | 8.4.2.1 | Table 8-51 is missing elements such as channel availability query element and network channel control element. | Add information elements. | Agree Add "Channel Availability Query element" and "Network Channel Control" elements into Table 8-51. |
| 1181 | 8.4.2.29 | 28 | 23 | T | 28.23 | 23 | 8.4.2.29 | Table 8-88 is missing elements such as channel availability query element and network channel control elements. | Add information elements. | Agree and revise as commented |

8.4 Management frame body components

8.4.2 Information elements

TGaf Editor: Insert rows in Table 8-65 as shown:

| Information Element | Element ID | Length (in octets) | Extensible |
| --- | --- | --- | --- |
| White Space Map (see 8.4.2.af1) | <ANA> | 4 to 255 | Yes |
| DSE Link Identifier (see 8.4.2.af2) | <ANA> | 12 | Yes |
| Channel Power Management Announcement (see 8.4.2.af3 (Channel Power Management)) | <ANA> | 3 to 257 |  |
| Channel Schedule Management (see 8.4.2.af4) | <ANA> | 3 to 255 |  |
| Geodatabase Inband Enabling Signal (see 8.4.2.af5) | <ANA> | 2 |  |
| Contact Verification Signal (see 8.4.2.af6) | <ANA> |  | Yes |
| Network Channel Control (see 8.4.2af7 Network Channel Control Element) | <ANA> | 3-255 | Yes |
| Channel Availability Query (see 8.4.2.af8 Channel Availability Query Element) | <ANA> | 3-255 | Yes |

8.4.2.29 Extended Capabilities element

TGaf Editor: Insert rows in Table 8-113 as shown:

| Bit | Information | Notes |
| --- | --- | --- |
| <ANA> | Network Channel Control | The STA sets the Network Channel Control field to 1 when the MIB attribute dot11NetworkChannelControlActivated is true, and set it to zero otherwise, see 8.4.2.af7 (Network Channel Control element). |
| <ANA> | Channel Availability Query | The STA sets the Channel Availability Query field to 1 when the MIB attribute dot11ChannelAvailabilityActivated is true, and set it to zero otherwise, see 8.4.2.af8 (Channel Availability Query element). |

TGaf Editor: Insert the following two subclauses as shown'

8.4.2.af7 Network Channel Control Element

The Network Channel Control element is used to request/respond to network channel control. It is transmitted by a STA and a dependent STA as part of the procedure to enable control of a dependent STA by an enabling STA for a particular 802.11 channel in TV bands under an operating class. The element is in the format shown in Figure 8-35af9.

|  |  |  |  |  |  |  |  |  | These three fields are repeated, as determined by the Length field |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Info ID |  | Length |  | Requester STA address | Responder STA Address | Reason Result Code | Network Channel Control Identifier | Operating Class | Channel Number | Transmit Power Constraint |
| Octets: | 1 |  | 1 |  | 6 | 6 | 1 | 2 | 1 | 1 | 1 |

Figure 8-42af9—Network Channel Control Element format

The Info ID field shall be set to the value for Network Channel Control defined in Table 8-65.

The Length field indicates the length of the remaining frame fields in octets, and the value is variable.

The RequesterSTAAddress field is the MAC address of the requesting STA that initiates the Network Channel Control process.

The ResponderSTAAddress field is the MAC address of the responding STA that grants Network Channel Control.

The Reason Result Code field is used to indicate the reason that a DSE Enablement Network Channel Control frame was generated. The length of the Reason Result Code field is 1 octet. The reason result codes that have been allocated are shown in Table 8-134 Table 8-45af3.

The Network Channel Control Identifier field is a 16-bit number assigned by an enabling STA to a dependent STA.

The Operating Class field indicates the channel set for which the network channel control request applies. The Operating Class and Channel Number fields together specify the channel frequency and channel bandwidth for which the channel control applies. Valid values for the Operating Class field are shown in Annex E.

The Network Channel Number field of control request indicates the network channel numbers for which dependent STA requests. The Network Channel Number field of the control response indicates the network channel granted by the enabling STA. The dependent STA can select a set of network channels from the list of available channels and the enabling STA can grant a subset of the network channels selected by the dependent STA. The channel number is defined within an operating class as shown in Annex E.

The Transmit Power Constraint gives the maximum transmit power in dBm for TV bands operation in the request frame and indicates the maximum allowable transmit power in dBm for TV band operation in the response frame. The field is coded as an unsigned integer in units of dBm.

8.4.2.af8 Channel Availability Query Element

The Channel Availability Query element is used to exchange the requests and responses for the Channel Availability Query process. It is transmitted by a STA as part of channel query. The format of the Channel Availability Query element is shown in Figure 8-42af10.

|  | Info ID | Requester STA address | Responder STA Address | Reason Result Code | Length | Channel Query Info | Device Class | Device Identification Information | Device Location Information | WSM Information |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 6 | 6 | 1 | 1 | 1 | variable | variable | variable | variable |

The Info ID field shall be set to the value for DSE Control defined in Table 8-65.

The Length field indicates the length of the remaining element fields in octets, and the value is variable. The minimum value of the Length field is 14.

The RequesterSTAAddress field is the MAC address of the requesting STA that initiates the channel query process. The length of the RequesterSTAAddress field is 6 octets.

The ResponderSTAAddress field is the MAC address of the responding STA that responds to the channel query requesting STA. The length of the ResponderSTAAddress field is 6 octets.

The Reason Result Code field is used to indicate the reason that a RLQP Channel Availability Query was generated. The length of the Reason Result Code field is 1 octet. The reason result codes that have been allocated are shown in Table 8-199 (Reason Result Code field values).

The Channel Query Info field is used to indicate the type of Channel Query request and associated parameters. The format of the Channel Query Info field is shown in Figure 8-404 (Channel Query Info field format).

Device Identification Information Present subfield set to 1 indicates that the Device Identification Information field is present in the Channel Availability Query element; otherwise it is set to 0 to indicate that the Device Identification Information field is not provided.

Device Location Information Present subfield set to 1 indicates that the Device Location Information field is present in the Channel Availability Query element; otherwise it is set to 0 to indicate that the Device Location Information field is not provided.

The Device Class field is used to indicate the characteristics of station requesting the channel availability query. The format of the Device Class field is specified in 8.2.6.1.1 (Device class).

The Device Identification Information field is used to indicate the identification of the station requesting the channel availability query. The format of the Device Identification Information field is specified in 8.2.6.1.2 (Device Identification Information).

The Device Location Information field is used to provide the location of the STA requesting the Channel Availability Query, which is provided in the format specified in 8.2.6.1.3 (Device Location Information).

The remaining fields are as described in the White Space Map element (see 8.4.2.106 (White Space Map element)).