IEEE P802.11
Wireless LANs

| Comment resolution regarding to enabling signal |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2011-07-19 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Chen Sun | NICT | 3-4, Hikarino-oka, Yokosuka, Kanagawa, Japan, 239-0847 | +81 45 847 5062 | sun@nict.go.jp |
| Zhou Lan | NICT | 3-4, Hikarino-oka, Yokosuka, Kanagawa, Japan, 239-0847 |  | lan@nict.go.jp |
| Yohannes Alemseged | NICT | 3-4, Hikarino-oka, Yokosuka, Kanagawa, Japan, 239-0847 |  | yohannes@nict.go.jp |
| Hiroshi Harada | NICT | 3-4, Hikarino-oka, Yokosuka, Kanagawa, Japan, 239-0847 |  | harada@nict.go.jp |

Abstract

This document presents resolution to comments received in the 1st letter ballot of 802.11af draft1.03

| CID | Page | Clause | Comment | Proposed Change |
| --- | --- | --- | --- | --- |
| 1182 | 36.42 | 8.4.2.af5 | The client devices listen enabling signal in the TV band and can contact the enabling STA and obtain the list of available channels in other frequency band. | Add additional field to indicate through which frequency band the client devices are allowed to contact the enabling STA and obtain the list of available channels. |

Discussion

The FCC regulations for unlicensed TV band devices (TVBDs) state that "To initiate contact with a fixed or Mode II device, a Mode I device may transmit on an available channel used by the fixed or Mode II TVBD or on a channel the fixed or Mode II TVBD indicates is available for use by a Mode I device on a signal seeking such contacts." This means that after the mode I TVBDs hearing an enabling signal the TVBDs can obtain the list of available channels, which is required by the FCC for starting operation, in the frequencies band indicated as available. Allowing the mode I TVBDs to contact the Mode I or fixed devices in frequency bands other than the TV bands potentially offloads the system overhead, incurred by the management frames for TVBD operation, in the TV band to other bands.

Propose Revised for CID 1182 per discussion and editing instructions in 11-11/656r4.

8.4.2.113 Geodatabase Inband Enabling Signal element

TGaf Editor: Change Figure 8-420q as follows:

|  | Element ID | Length | Enablement Band | Enablement Channel Operating Class | Enablement Channel Number |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 1 |

Figure 8-402q—Geodatabase Inband Enabling Signal element format

TGaf Editor: Insert the following sentences as the last paragraph of 8.4.2.133

The Enablement Band field indicates frequency bands such as TVWS or non-TVWS link including 2.4GHz, 5G, on which the dependent STA can contact the enabling STA to seek enablement to obtain the list of available channels. The values of Enablement Band field is given in Table 8-42af3.

Table 8-42af3 Definition of Enablement Band element

| Name | Band Identifier |
| --- | --- |
| TV band | 0 |
| Sub-1 GHz | 1 |
| 2.4 GHz | 2 |
| 3.6 GHz | 3 |
| 5 GHz | 4 |
| 60 GHz | 5 |
| Reserved | 6-255 |

The Enablement Channel Operating Class field gives the operating class of the channel in which the GDC dependent STA can seek enablement with the GDC enabling STA and obtain the WSM (cf. 8.4.2.160 (White Space Map element)). The Operating Class and Channel Number fields together specify the channel frequency and channel bandwidth for which the enablement applies. Values for the Operating Class field are shown in Annex E.1 (Country information and operating classes).

The Enablement Channel Number field gives the channel number of the channel in which the GDC dependent STA can seek enablement with the GDC enabling STA and obtain the WSM (cf. 8.4.2.160 (White Space Map element)). The channel number is defined within an operating class as shown in Annex E.1 (Country information and operating classes).