IEEE P802.11
Wireless LANs

| Comment Resolution for CID 106 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: May 05, 2011 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Wu Tianyu | Huawei |  |  | wutianyu@huawei.com |
| Edward Au | Huawei |  |  | Edward.au@huawei.com |

Abstract

This document provides resolution for CID 106 related the VHT Compressed Beamforming Report field the MU Exclusive Report field in TGac D0.1 and 422r1.

CID: 106

Comments (page and line numbers) are based on D0.1.

Proposed edits are based on D0.1 and 11-11-0422-01-00ac-d0-1-comment-resolution-vht-compressed-beamforming-report-field-and-mu-exclusive-report-field.

| CID | Subclause | Page | Line | Comment Type | Comment | SuggestedRemedy |
| --- | --- | --- | --- | --- | --- | --- |
| 106 | 7.3.1.32 | 22 | 38 | TR | The structure of the VHT compressed beamforming report field for 160M/80+80M is not clearly defined. | Clarify the spec text and clearly define the structure of the VHT compressed beamforming report field for 160M/80+80M. Details of the proposed text change are presented in our contribution. |

Proposed Resolution Text:

TGac editor: Edit subclauses 7.3.1.34 as follows:

#### 7.3.1.32 VHT Compressed Beamforming Report field

The VHT Compressed Beamforming Report field has the structure defined in Table 7-X3 and 7-X4, where Na is the number of angles used for the compressed beamforming feedback matrix subfield (see Table 7-X1).

Table 7-X3 – VHT Compressed Beamforming Report field (20MHz, 40MHz and 80MHz)

| Subfield | Size (bits) | Meaning |
| --- | --- | --- |
| Average SNR of Space-Time Stream 1 | 8 | Signal-to-noise ratio at the beamformee for space-time stream 1 averaged over subcarriers. See Table 7-X5. |
| … | … | … |
| Average SNR of Space-Time Stream Nc | 8 | Signal-to-noise ratio at the beamformee for space-time stream Nc averaged over subcarriers. See Table 7-X5 |
| Compressed Beamforming Feedback Matrix for subcarrier k = scidx(0) | Na×(b +b)/2 | Compressed beamforming feedback matrix as defined in Table 7-X1 |
| Compressed Beamforming Feedback Matrix for subcarrier k = scidx(1) | Na×(b +b)/2 | Compressed beamforming feedback matrix as defined in Table 7-X1 |
| Compressed Beamforming Feedback Matrix for subcarrier k = scidx(2) | Na×(b +b)/2 | Compressed beamforming feedback matrix as defined in Table 7-X1 |
| … | … | … |
| Compressed Beamforming Feedback Matrix for subcarrier k = scidx(Ns-1) | Na×( b +b)/2 | Compressed beamforming feedback matrix as defined in Table 7-X1 |

Table 7-X4 – VHT Compressed Beamforming Report field (160MHz and 80+80MHz)

| Subfield | Size (bits) | Meaning |
| --- | --- | --- |
| Average SNR of Space-Time Stream 1 on lower 80MHz subband | 8 | Signal-to-noise ratio at the beamformee for space-time stream 1 averaged over subcarriers of the lower 80MHz subband. See Table 7-X5 |
| … | … | … |
| Average SNR of Space-Time Stream Nc on lower 80MHz subband | 8 | Signal-to-noise ratio at the beamformee for space-time stream Nc averaged over subcarriers of the lower 80MHz. See Table 7-X5. |
| Average SNR of Space-Time Stream 1 on higher 80MHz subband | 8 | Signal-to-noise ratio at the beamformee for space-time stream 1 averaged over subcarriers of the higher 80MHz subband. See Table 7-X5 |
| … | … | … |
| Average SNR of Space-Time Stream Nc on higher 80MHz subband | 8 | Signal-to-noise ratio at the beamformee for space-time stream Nc averaged over subcarriers of the higher 80MHz subband. See Table 7-X5 |
| Compressed Beamforming Feedback Matrix for subcarrier k = scidx(0) | Na×(b +b)/2 | Compressed beamforming feedback matrix as defined in Table 7-X1 |
| Compressed Beamforming Feedback Matrix for subcarrier k = scidx(1) | Na×(b +b)/2 | Compressed beamforming feedback matrix as defined in Table 7-X1 |
| Compressed Beamforming Feedback Matrix for subcarrier k = scidx(2) | Na×(b +b)/2 | Compressed beamforming feedback matrix as defined in Table 7-X1 |
| … | … | … |
| Compressed Beamforming Feedback Matrix for subcarrier k = scidx(Ns-1) | Na×( b +b)/2 | Compressed beamforming feedback matrix as defined in Table 7-X1 |

where

is the estimated MIMO channel for subcarrier k

is column i of the beamforming matrix  for subcarrier k

is the noise plus interference power measured at the beamformee

is Average SNR of Space-Time Stream i reported in the VHT Compressed Beamforming Report field (Table 7-X3) for 20MHz, 40MHz and 80MHz. For160MHz and 80+80MHz,   is Average SNR of Space-Time Stream i on lower 80MHz subband in Table 7-X4 if k<0 and is Average SNR of Space-Time Stream i on higher 80MHz subband  if k>0.

### Discussion:

Adding one more average SNR fb for 160M and 80+80MHz is for better calculation of Per Tone SNR.

Table 7-11 – MU Exclusive Beamforming Report field

| Subfield | Size (Bits) | Meaning |
| --- | --- | --- |
| Delta SNR for space-time stream 1 for subcarrier k = scidx(0) | 4 | as defined in Equation (7-X3) |
| … | … | … |
| Delta SNR for space-time stream Nc for subcarrier k = scidx(0) | 4 | as defined in Equation (7- X3) |
| Delta SNR for space-time stream 1 for subcarrier k = scidx(1) | 4 | as defined in Equation (7- X3) |
| … | … | … |
| Delta SNR for space-time stream Nc for subcarrier k = scidx(1) | 4 | as defined in Equation (7- X3) |
| … | … | … |
| Delta SNR for space-time stream 1 for subcarrier k = scidx(Ns’-1) | 4 | as defined in Equation (7- X3) |
| … | … | … |
| Delta SNR for space-time stream Nc for subcarrier k = scidx(Ns’-1) | 4 | as defined in Equation (7- X3) |

The Delta SNR is calculated from:

In 160MHz contiguous and 80+80MHz transmission, the range of  could be very large because:

- Although theoretically, if all entries of Hk are iid Gaussian distributed, the mean and variance of  shall be the same for two 80MHz subband, this may not be true in practice espectially if there is strong correlation on frequency domain. There is chance that mean of  is quite different on two 80MHz subband.

- More importantly, there is also chance that two 80MHz subband see different interference level so that N (the noise plus interference power measured at the beamformee.) is very different on two 80MHz subband.  For example: lower 80MHz channel is clear and beamformee only see a noise power slightly above -95dBm, but on the higher 80MHz channel, the beamformee see an interference level of -80dBm. The CCA will indicate IDLE for both lower 80MHz and higher 80MHz but N for lower and higher 80MHz may have a difference as large as 15dB.

Combine reason 1 and 2, the dynamic range of  could be larger than 16dB but with only 4 bits for  quantization, we can not precisely express  if the dynamic range is larger than 16dB.

This can be solved by sending 2 average SNRs for each 80MHz subband as illustrated in the following figures:

Fig 1. 1 average SNR field