IEEE P802.11
Wireless LANs

| Comment resolutions regarding to Network channel control SAP and frame format |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2011-05-09 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Chen Sun | NICT | 3-4, Hikarino-oka, Yokosuka, Kanagawa, Japan, 239-0847 | +81 45 847 5062 | sun@nict.go.jp |
| Zhou Lan | NICT | 3-4, Hikarino-oka, Yokosuka, Kanagawa, Japan, 239-0847 |  | lan@nict.go.jp |
| Yohannes Alemseged | NICT | 3-4, Hikarino-oka, Yokosuka, Kanagawa, Japan, 239-0847 |  | yohannes@nict.go.jp |
| Hiroshi Harada | NICT | 3-4, Hikarino-oka, Yokosuka, Kanagawa, Japan, 239-0847 |  | harada@nict.go.jp |

Abstract

This document presents resolutions to the comments, 24, 91, 108, 197, 203, 431, 439, 604, 625, 895, 433, 440, 460, 524, 559, 568, 570, 914, 916, 601, 622, 698, 869, 874, 1040, 1044, 1222, 1234, 1099,  received in the 1st letter ballot of 802.11af draft. The proposed changes and the editor's instructions are given based on IEEE P802.11af™/D1.01.

| CID | Clause | Page(C) | Line(C) | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 24 | 6.3.af2.1.2 | 5 | 18 | If the network channel control public action frame is a directed frame (i.e. not broadcast) then a PeerMACAddress parameter is needed. | Add a "Peer MAC Address" parameter to the primitive, which is used to select the STA to which this frame will be sent | Agree. The network channel control public action frame is a directed frame. To resolve the comment, add PeerMACAddress parameter at request primitive and response primitive. Also, change the sentence at line 10 from “This primitive requests that a (Protected) Network Channel Control public action frame be sent by a STA” to “This primitive requests that a (Protected) Network Channel Control public action frame be sent by a STA to a specified peer MAC entity.” |

| CID | Clause | Page(C) | Line(C) | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 91 | 6.3.af2 |  |  | No protocol exchange figure is given | Supply a protocol exchange figure like Figure 6-af1 and Figure 6-af2 | Disagree. Figure 6-af1 is an example and should be moved out side of this normative session. Thus, adding a protocol exchange figure in 6.3.af2 is not needed. |

| CID | Clause | Page(C) | Line(C) | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 108 | 6.3.af2.1.2 | 5 | 14 | NetworkChannelControl includes Requester and Responder's addresses. In MLME-NEWORKCHANNELCONTROL.indication primitive, there is PeerMACAddress. Why doesn't MLME-NEWORKCHANNELCONTROL.request include PeerMACAddress? The same issues exist in other primitives. | Add PeerMACAddress in MLME-NEWORKCHANNELCONTROL.request or delete PeerMACAddress in MLME-NEWORKCHANNELCONTROL.indication. Change the other primitives accordingly. | Agree. The network channel control public action frame is a directed frame. To resolve the comment, add PeerMACAddress parameter at request primitive and response primitive. |

| CID | Clause | Page(C) | Line(C) | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 197 | 8.4.5.4 | 41 | 17 | The text does not require that the same Netw. Chan. Ctrl. Identifier is only assigned once (uniqueness) | Insert "uniquely" before "assigned" | same 203, 197 Agree. Revise the sentence as proposed by the commenter. |
| 203 | 8.5.8.af2 | 45 | 1 | The text does not require that the same Netw. Chan. Ctrl. Identifier is only assigned once (uniqueness) | Insert "uniquely" before "assigned" |  |

| CID | Clause | Page(C) | Line(C) | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 431 | 8.4.5.4 | 41 | 23 | Why is a required component of this protocol documented only in an Annex? | Move the definitions of all required fields into the main text. | Disagree To make the interface, frame format to be region-independent, the values of operating class are given in the Annex as in 802.11Revmb |
| 439 | 8.5.8.af2 | 45 | 7 | Why is a required component of this protocol documented only in an Annex? | Move the definitions of all required fields into the main text. |  |

| CID | Clause | Page(C) | Line(C) | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 604 | 8.4.5.4 | 41 | 35-36 | "The field can be ignored by the enabling STA if the Network Channel Control frame is sent from a dependent STA to an enabling STA as a control request." Does this sentence mean that a depenent STA may load arbitrary value in the Transmit Power Contraint field when it request a Network Channel Control? It would be better to explicit value when it is used as a request frame. | Change the sentence as "The field can be set to zero when a dependent STA requests an enabling STA a Network Channel Control." or remove Power field in request frame. | SAME: 625, 895, 604 Agree in principle that the sentence needs revision. To resolve the comment, the original sentence has been changed from "The Transmit Power Constraint field gives the maximum allowable transmit power in dBm for TV bands operation. The field can be ignored by the enabling STA if the Network Channel Control frame is sent from a dependent STA to an enabling STA as a control request." to "The Transmit Power Constraint gives the maximum transmit power in dBm for TV bands operation in the request frame and indicates the maximum allowable transmit power in dBm for TV band operation in the response frame." |
| 625 | 8.4.5.4 | 41 | 35-36 | "The field can be ignored by the enabling STA if the Network Channel Control frame is sent from a dependent STA to an enabling STA as a control request." Does this sentence mean that a depenent STA may load arbitrary value in the Transmit Power Contraint field when it request a Network Channel Control? It would be better to explicit value when it is used as a request frame. | Change the sentence as "The field can be set to zero when a dependent STA requests an enabling STA a Network Channel Control." or remove this power field in request frame. |  |
| 895 | 8.4.5.4 | 41 | 35-36 | "The field can be ignored by the enabling STA if the Network Channel Control frame is sent from a dependent STA to an enabling STA as a control request." Does this sentence mean that a depenent STA may load arbitrary value in the Transmit Power Contraint field when it request a Network Channel Control? It would be better to explicit value when it is used as a request frame. | Change the sentence as "The field can be set to zero when a dependent STA requests an enabling STA a Network Channel Control." or remove Power field in request frame. |  |

| CID | Clause | Page(C) | Line(C) | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 433 | 8.4.5.4 | 41 | 35 | The use of "can" is discouraged by the IEEE editors if it could possibly mean "may". That interpretation appears appropriate here. | Replace "can" with "may". | Agree in principle. To address comments 604, 625, 895 the sentence has been changed from "The Transmit Power Constraint field gives the maximum allowable transmit power in dBm for TV bands operation. The field can be ignored by the enabling STA if the Network Channel Control frame is sent from a dependent STA to an enabling STA as a control request." to "The Transmit Power Constraint gives the maximum transmit power in dBm for TV bands operation in the request frame and indicates the maximum allowable transmit power in dBm for TV band operation in the response frame. The field is coded as an unsigned integer in units of dBm." |

| CID | Clause | Page(C) | Line(C) | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 440 | 8.5.8.af2 | 45 | 10 | The statement "The Network Channel Number field of control request indicates the network channel numbers for which dependent STA requests." is confused. There is no "Network Channel Number" field. Is a "control request" a Network Channel Control frame? Should "numbers for which dependent STA requests" be rewritten as "numbers that the dependent STA requests"? | Rewrite this into one or more clear statements. Note also that the following sentence refers to the non-existent Network Channel Number field. | Agree in principle Change the sentence from "The Network Channel Number field of control request indicates the network channel numbers for which dependent STA requests. The Network Channel Number field of in the response frame indicates the network channel granted by the enabling STA." to "The Channel Number field of Network Channel Control request indicates the channel that the dependent STA requested to operate on. The Channel Number field in the Network Channel Control response frame indicates the channels that enabling STA permitted the client STA to operate on. |

| CID | Clause | Page(C) | Line(C) | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 460 | 6.3.af2.1.3 | 6 | 5 | "is associated to the enabling STA" implies that there is an assoiation that is peer-to-peer, not in an infrastructure. | If the intent is to use this only in IBSSs, then describe that fact. Since Public Action frames are intended for unassociated targets, perhaps replace "be sent to a STA that is associated to the enabling STA" with "be sent to a dependent STA". | Agree in principle with the need to rewrite the sentence. The network channel control frame is a public action frame and is not limited only to IBSSs. The sentence has been changed from "This primitive is generated by the STA management entity (SME) to request that a (Protected) Network Channel Control public action frame be sent to a STA that is associated to the enabling STA." to "This primitive is generated by the STA management entity (SME) to request that a (Protected) Network Channel Control frame be sent by a STA to the specified peer MAC entity." |

| CID | Clause | Page(C) | Line(C) | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 524 | 8.5.8.af2 | 45 | 1-2 | Network Channel Control frame is exchanged between an enabling STA and dependent STAs. It is not necessary to assign a duplicate identifier. If the operation is not correct, it should be justfied the reason why the enabler needs to assign another identifier. | Remove the following sentence, "The Network Channel Control Identifier field is a 16-bit number assigned by an enabling STA to a dependent STA." | Disagree Similar to comment 559, 568. (see 568 ion the next page.) Network channel control identifier give unique identity for each network channel control request. If one STA is enabled it can send different request at different time. Also, the purpose of the network channel control is different from the enablement. No change is made. |
| 559 | 8.4.5.4 | 41 | 17-18 | need for indentifier is unclear. the network channle controlframe is sent between enabling and dependent STA, thus indentify is uneccesary | remove indentifier |  |

| CID | Clause | Page(C) | Line(C) | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 568 | 8.4.5.4 | 41 | 17-18 | Network Channel Control frame are exchanged between an enabling STA and dependent STAs. No need to assign a duplicate identifier. If it is not, it should be justfied the reason why the enabler needs to assign another identifier. | remove the sentence, "The Network Channel Control Identifier field is a 16-bit number assigned by an enabling STA to a dependent STA." | Similar to comment 559. SAME: 570,568,601, 622, 916, 914 Network channel control identifier give unique identity for each network channel control request. If one STA is enabled it can send different request at different time. Also, the purpose of the network channel control is different from the enablement. No change is made. |
| 570 | 8.5.8.af2 | 45 | 1-2 | Network Channel Control frame are exchanged between an enabling STA and dependent STAs. No need to assign a duplicate identifier. If it is not, it should be justfied the reason why the enabler needs to assign another identifier. | remove the sentence, "The Network Channel Control Identifier field is a 16-bit number assigned by an enabling STA to a dependent STA." |  |
| 914 | 8.4.5.4 | 41 | 17-18 | Network Channel Control frame are exchanged between an enabling STA and dependent STAs. No need to assign a duplicate identifier. If it is not, it should be justfied the reason why the enabler needs to assign another identifier. | remove the sentence, "The Network Channel Control Identifier field is a 16-bit number assigned by an enabling STA to a dependent STA." |  |
| 916 | 8.5.8.af2 | 45 | 1-2 | Network Channel Control frame are exchanged between an enabling STA and dependent STAs. No need to assign a duplicate identifier. If it is not, it should be justfied the reason why the enabler needs to assign another identifier. | remove the sentence, "The Network Channel Control Identifier field is a 16-bit number assigned by an enabling STA to a dependent STA." |  |
| 601 | 8.4.5.4 | 41 | 16 | Network Channel Control frame are exchanged between an enabling STA and dependent STAs. Is the Network Channel Control Identifier the same as Enablement Identifier? If it is, there is no need to assign a duplicate identifier. If it is not, it should be justfied the reason why the enabler needs to assign another identifier. | Change the Network Channel Control Identifier as Enablement Identifier or justfy the nessicity of another identifer and specify the assignment of it. |  |
| 622 | 8.4.5.4 | 41 | 16 | Network Channel Control frame are exchanged between an enabling STA and dependent STAs. Is the Network Channel Control Identifier the same as Enablement Identifier? If it is, there is no need to assign a duplicate identifier. If it is not, it should be justfied the reason why the enabler needs to assign another identifier. | Change the Network Channel Control Identifier as Enablement Identifier or justfy the nessicity of another identifer and specify the assignment of it. |  |

| CID | Clause | Page(C) | Line(C) | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 698 | 6.3.af2.2.2 | 6 | 54 | Extra result codes for over the air transactions should be added, e.g. timeout and tranmission failure in 802.11ae 6.3.ae1.2.2. These changes should also be made to similar primitives. | Add the following result codes to the ResultCode enumeration "TIMEOUT, TRANSMISSION_FAILURE" | Agree Apply the changes as proposed by the commenter throughout the draft. |

| CID | Clause | Page(C) | Line(C) | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 869 | 6.3.af.2.2.2 | 6 | 23 | Why would the primitive use VendorSpecificInfo for information that is being defined by this amendment. Possibly already defined in 8.5.8.af2, and just needs the proper reference and adjustment to the primitive fields. | define the specifics that the amendment is wanting to be exchanged. | Disagree Similar: 869,874 The primitive can carry both information elements that are defined in 8.5.8.af2 and vendor specific elements as defined in 8.4.2.28. No change is made. |
| 874 | 6.3.af2.1 | 5 | 6 | The primivite(request, response, and confirm) utilizes a VendorSpecific parameter. As this is a new definition, It would seem plausible that the parameters could be specified, and a STANDARD way of getting the information be used. If this is a Vendorspecific thing, then use the generic Vendorspecific primitives and do not add this series of primitives that really boil down to just a vendor specific primitive. | make a standardize definition of the data being exchanged. | Disagree Similar: 869,874 The primitive can carry both information elements that are defined in 8.5.8.af2 and vendor specific elements as defined in 8.4.2.28. No change is made. |

| CID | Clause | Page(C) | Line(C) | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 1040 | 6.3.af2.1.3 | 6 | 5 | "associated to the enabling STA." the roles are unclear. Is the STA issuing the request always a DSE enabling STA? | If so, state this clearly. | Agree in principle The network channel control frame is a public action frame used by the STA to request permission to channel usage from the enabling STA. The sentence has been changed from "This primitive is generated by the STA management entity (SME) to request that a (Protected) Network Channel Control public action frame be sent to a STA that is associated to the enabling STA." to "This primitive is generated by the STA management entity (SME) to request that a (Protected) Network Channel Control frame be sent by a STA the specified peer MAC entity." |

| CID | Clause | Page(C) | Line(C) | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 1044 | 6.3.af2.3.2 | 8 | 26 | "A set of elements" - this is not correct. It is limited to vendor specific elements. | Insert "vendor specific" globally in this context. | Disagree It is explained in the Valid range column that they are vendor specific elements. This follows the current 11Remb format. Therefore, no change has been made. |

| CID | Clause | Page(C) | Line(C) | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 1222 | 6.3.af2.1.2 | 9 | 31 | The effect of receipt of .response should be the transmission of the network channel control response frame | Change to: On receipt of this primitive, the MLME schedules the transmission of network channel control. | Agree in principle The sentence has been changed from "On receipt of this primitive, the MLME schedules the network channel control." to "On receipt of this primitive, the MLME schedules the response to the specific peer MAC entity that has requested network channel control ". |

| CID | Clause | Page(C) | Line(C) | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 1234 | 8.4.5.4 | 41 | 34 | Format ambiguity on Transmit Power Constraint field. Are negative numbers allowed? Are fractional numbers allowed? Are scaling factors used? |  | Agree in principle with the commenter that the format should have been specified. To resolve this comment, the following sentence has been added. "The field is coded as an unsigned integer in units of dBm" Furthermore, to address comments 604, 625, 895 the sentence has been changed from "The Transmit Power Constraint field gives the maximum allowable transmit power in dBm for TV bands operation. The field can be ignored by the enabling STA if the Network Channel Control frame is sent from a dependent STA to an enabling STA as a control request." to "The Transmit Power Constraint gives the maximum transmit power in dBm for TV bands operation in the request frame and indicates the maximum allowable transmit power in dBm for TV band operation in the response frame. The field is coded as an unsigned integer in units of dBm." |

| CID | Clause | Page | Line | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 1099 | 44.33 | 33 | 8.5.8.af2 | "These three fields are repeated, as determined by the Length field" - this is an error. Action frames need to be parseable to determine the presence of any optional vendor specific and managment frame protection elements at the end of the frame. As such, the content part defined per action field needs to be self-delimiting so that the end of the action field can be located. | Add a Number of Operating Classes field before the repeating content. | Disagree. The fields Operation Class and Channel Number are given in the TLV format as in the Annex. They contains the length information thus the three fields are parseable. No change has been made |

6. Layer Management

6.3 MLME SAP Interface

TGaf Editor: Revise 8.3.81as follows.

6.3.81 Network channel control management

The following MLME primitives support the signaling of network channel control management.

6.3.81.1 MLME-NETWORKCHANNELCONTROL.request

6.3.81.1.1 Function

This primitive requests that a (Protected) Network Channel Control public action frame be sent by a STA to a specified peer MAC entity.

6.3.81.1.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-NETWORKCHANNELCONTROL.request (

Peer MAC Address,

Dialog Token,

NetworkChannelControl,

Protected,

VendorSpecificInfo

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual or group MAC address | The address of the peer MAC entity to which the measurement request is transmitted. |
| Dialog Token | Integer | 0-255 | The dialog token to identify the network channel control transaction |
| NetworkChannelControl | A set of information subfields | As described in 8.5.8.af2 | As described in 8.5.8.af2 |
| Protected | Boolean | true, false | Specifies whether the request is sent using a Robust Management frame. If true, the request is sent using the Protected Network Channel Control frame. If false, the request is sent using the Network Channel Control frame. |
| VendorSpecificInfo | A set of elements | As defined in 8.4.2.28 (Vendor Specific element) | Zero or more elements. |

6.3.af2.1.3 When generated

This primitive is generated by the STA management entity (SME) to request that a (protected) Network Channel Control public action frame be sent to a STA that is associated to the enabling STA by a STA to the specified peer MAC entity.

6.3.81.1.4 Effect of receipt

On receipt of this primitive, the MLME constructs and transmits a (Protected) Network Channel Control public action frame. This frame is then scheduled for transmission.

6.3.81.2 MLME-NETWORKCHANNELCONTROL.confirm

6.3.81.2.1 Function

This primitive reports the result of a request to network channel control.

6.3.af2.2.2 Semantics of the service primitive

The primitive parameters as follows:

MLME- NETWORKCHANNELCONTROL.confirm (

Dialog Token,

ResultCode,

VendorSpecificInfo

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Dialog Token | Integer | 0-255 | The dialog token to identify the network channel control transaction |
| ResultCode | Enumeration | SUCCESS, INVALID_PARAMETERS, UNSPECIFIED_FAILURE, TIMEOUT, TRANSMISSION_FAILURE | Reports the result of a network channel control request |
| VendorSpecificInfo | A set of elements | As defined in 8.4.2.28 (Vendor Specific element) | Zero or more elements. |

6.3.af2.2.3 When generated

This primitive is generated by the MLME when a Network Channel Control request completes.  Possible unspecified failure causes include an inability to schedule a Network Channel Control public action frame.

6.3.81.2.4 Effect of receipt

The SME is notified of the results of the network channel control procedure.

6.3.81.3 MLME-NETWORKCHANNELCONTROL.indication

6.3.81.3.1  Function

This primitive indicates that a (Protected) Network Channel Control public action frame was received from an STA.

6.3.81.3.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-NETWORKCHANNELCONTROL.indication (

PeerMACAddress,

Dialog Token,

NetworkChannelControl,

Protected,

VendorSpecificInfo

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerMACAddress | MACAddress | Any valid individual MAC address | The address of the peer MAC entity from which the Network Channel Control public action frame was received. |
| Dialog Token | Integer | 0-255 | The dialog token to identify the network channel control transaction |
| NetworkChannelControl | A set of information subfields | As described in 8.5.8.af2 | As described in 8.5.8.26 (Network Channel Control frame format) |
| Protected | Boolean | true, false | Specifies whether the request was received using a Robust Management frame. If true, the request was received using the Protected Network Channel Control public action frame. If false, the request was received using the Network Channel Control public action frame. |
| VendorSpecificInfo | A set of elements | As defined in 8.4.2.28 (Vendor Specific element) | Zero or more elements. |

6.3.81.3.3 When generated

This primitive is generated by the MLME when a valid (Protected) Network Channel Control public action frame is received.

6.3.81.3.4 Effect of receipt

On receipt of this primitive, the SME decides whether to accept the switch request.

6.3.81.4 MLME-NETWORKCHANNELCONTROL.response

6.3.81.4.1  Function

This primitive is used to schedule an accepted network channel control.

6.3.81.4.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-NETWORKCHANNELCONTROL.response (

Peer MAC Address,

Dialog Token,

NetworkChannelControl,

VendorSpecificInfo

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual or group MAC address | The address of the peer MAC entity to which the measurement request is transmitted. |
| Dialog Token | Integer | 0-255 | The dialog token to identify the network channel control transaction |
| NetworkChannelControl | A set of information subfields | As described in 8.5.8.af2 | As described in 8.5.8.26 (Network Channel Control frame format). |
| VendorSpecificInfo | A set of elements | As defined in 8.4.2.28 (Vendor Specific element) | Zero or more elements. |

6.3.81.4.3 When generated

This primitive is generated by the SME to schedule an accepted network channel control request.

6.3.af2.4.4 Effect of receipt

On receipt of this primitive, the MLME schedules the network channel control the response to the specific peer MAC entity that has requested network channel control.

8 Frame formats

8.4 Management frame body components

8.4.5 Registered Location Query Protocol elements

TGaf Editor: Revise 8.4.5.5 as follows.

- 8.4.5.5 RLQP Network Channel Control element

The Network Channel Control element of RLQP is used to request/respond to network channel control using the GAS protocol rather than using dedicated Public Action frames. The element is in the format shown in Figure 8-405 (RLQP Network Channel Control element format)

|  |  |  |  |  |  |  |  |  | These three fields are repeated, as determined by the length field |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Info ID |  | Length |  | Requester STA address | Responder STA Address | Reason Result Code | Network Channel Control Identifier | Operating Class | Channel Number | Transmit Power Constraint |
| Octets: | 1 |  | 1 |  | 6 | 6 | 1 | 2 | 1 | 1 | 1 |

Figure 8-405—RLQP Network Channel Control frame body format

The RLQP Info ID field shall be set to the value for DSE Control defined in Table 8-198 (Registered Location Query Protocol info ID definitions).

The Length field indicates the length of the remaining frame fields in octets, and the value is variable.

The RequesterSTAAddress field is the MAC address of the requesting STA that initiates the Network Channel Control process.

The ResponderSTAAddress field is the MAC address of the responding STA that grants Network Channel Control.

The Reason Result Code field is used to indicate the reason that a DSE Enablement frame was generated. The length of the Reason Result Code field is 1 octet. The reason result codes that have been allocated are shown in Table 8-226 (Reason Result Code field values).

The Network Channel Control Identifier field is a 16-bit number uniquely assigned by an enabling STA to a dependent STA.

NetworkChannelDescriptor describes the network channels, the applied spectrum mask and the maximum transmit power at the center frequency the network channel applies.

The Operating Class field indicates the channel set for which the network channel control request applies. The Operating Class and Channel Number fields together specify the channel frequency and channel bandwidth for which the channel control applies. Values for the Operating Class field are shown in Annex E.1 (Country information and operating classes).

The Channel Number field of control request indicates the network channel numbers for which dependent STA requests. The Channel Number field of the control response indicates the network channel granted by the enabling STA. The Channel Number field of Network Channel Control request indicates the channel that the dependent STA requested to operate on. The Channel Number field in the Network Channel Control response frame indicates the channels that enabling STA permitted the client STA to operate on. The dependent STA can select a set of channels from the list of available channels and the enabling STA can grant a subset of the channels selected by the dependent STA. The channel number is defined within an operating class as shown in Annex E.1 (Country information and operating classes).

The Transmit Power Constraint field gives the maximum allowable transmit power in dBm for TV bands operation. The field can be ignored by the enabling STA if the Network Channel Control frame is sent from a dependent STA to an enabling STA as a control request.

The Transmit Power Constraint gives the maximum transmit power in dBm for TV bands operation in the request frame and indicates the maximum allowable transmit power in dBm for TV band operation in the response frame.

- 8.5.8 Public Action details

TGaf Editor: Revise 8.5.8.26 as follows.

- 8.5.8.26 Network Channel Control frame format

The Network Channel Control frame is a Public Action frame. It is transmitted by a STA and a dependent STA as part of the procedure to enable control of a dependent STA by an enabling STA for a particular 802.11 channel in TV bands under an operating class. The format of the Network Channel Control frame body is shown in Figure 8-443 (Network Channel Control frame Action field format).

|  |  |  |  |  |  |  |  |  | These three fields are repeated, as determined by the length field |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Category | Action Value |  | Length |  | Requester STA address | Responder STA Address | Reason Result Code | Network Channel Control Identifier | Operating Class | Channel Number | Transmit Power Constraint |
| 1 Octets: | 1 |  | 1 |  | 6 | 6 | 1 | 2 | 1 | 1 | 1 |

Figure 8-443—Network Channel Control frame body format

The Category field is set to the value indicating the Public category, as specified in Table 8-49 (Category values) in 8.4.1.11 (Action field).

The Action Value field is set to the value indicating Network Channel Control frame, as specified in

Table 8-223 (Public Action field values) in 8.5.8.1 (Public Action frames).

The Length field indicates the length of the remaining frame fields in octets, and the value is variable.

The RequesterSTAAddress field is the MAC address of the requesting STA that initiates the Network Channel Control process.

The ResponderSTAAddress field is the MAC address of the responding STA that grants Network Channel Control.

The Reason Result Code field is used to indicate the reason that a DSE Enablement frame was generated. The length of the Reason Result Code field is 1 octet. The reason result codes that have been allocated are shown in Table 8-226 (Reason Result Code field values).

The Network Channel Control Identifier field is a 16-bit number uniquely assigned by an enabling STA to a dependent STA.

The Operating Class field indicates the channel set for which the network channel control request applies. The Operating Class and Channel Number fields together specify the channel frequency and channel bandwidth for which the channel control applies. Values for the Operating Class field are shown in Annex E.1 (Country information and operating classes).

The Channel Number field of control request indicates the network channel numbers for which dependent STA requests. The Channel Number field of the control response indicates the network channel granted by the enabling STA. The dependent STA can select a set of channels from the list of available channels and the enabling STA can grant a subset of the channels selected by the dependent STA. The channel number is defined within an operating class as shown in Annex E.1 (Country information and operating classes).

The Transmit Power Constraint field gives the maximum allowable transmit power in dBm for TV bands operation. The field can be ignored by the enabling STA if the Network Channel Control frame is sent from a dependent STA to an enabling STA as a control request.

The Transmit Power Constraint gives the maximum transmit power in dBm for TV bands operation in the request frame and indicates the maximum allowable transmit power in dBm for TV band operation in the response frame.