IEEE P802.11
Wireless LANs

| CID 3027, 3028, 3034 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2011-06-08 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Brian Hart | Cisco Systems |  |  | brianh@cisco.com |

Abstract

This document proposes resolutions to CIDs 3027 3028, and 3034 on Draft 3.0 of TGad

Assumes Draft 3.0 as a baseline.

| 3027 | Chu, Liwen | 9.34.1 | 262 | 8 | T | The fomula is wrong. Change it to "ClusterTimeOffsetn-1 = ((dot11BeaconPeriod TUs)/(ClusterMaxMem)) × (n-1)". | As proposed. |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3028 | Chu, Liwen | 9.34.1 | 262 | 17 | T | Chagge to "The maximum size of the Beacon SP duration field transmitted by a S-PCP/S-AP shall be the Beacon 23 Interval of the S-PCP/S-AP divided by ClusterMaxMem" | As proposed. |

Proposed resolution: Accept in principle.

Discussion: The formula “works” but agreed that it produces ClusterMaxMem-1 S-PCP/S-APs per BI yet the definition of ClusterMaxMem in 8.3.4.1 is “The ClusterMaxMem field defines maximum number of PCPs and/or APs, including the S-PCP/AP, that can participate in the cluster.” So there is an inconsistency, to be addressed at the positions noted and elsewhere.

Furthermore, there is a similar inconsistency with TXSS CBAP MaxMem (8.4.2.138 says “The TXSS CBAP MaxMem field is the number of TXSS CBAPs per BI.”) Yet the range and denominator is too small, so fix this at the same time

Change

8.4.2.138 ECPAC Policy element

The Available Time Cluster Offset Bitmap field is a bitmap where the (n-1)th bit, n = 1 to 32, indicates the availability of the nth Beacon SP. Values of n = 1 and greater than ClusterMaxMem are reserved (i.e., bit 0 and bits ClusterMaxMem+1 to 31 inclusive). Bit n-1 set to 0 indicates that ClusterTimeOffsetn-1 is determined to be already in use by a neighboring PCP/AP, excluding the recipient if sent within an individually addressed frame, in the ECPAC. Bit n-1 set to 1 indicates that ClusterTimeOffsetn-1 is not determined to be already in use by a neighboring PCP/AP, excluding the recipient if sent within an individually addressed frame, in the ECPAC.

8.4.2.139 Cluster Time Offset element

The Cluster Time Offset Index field is set to the value n-1 for a member PCP/AP of a Centralized PCP/AP cluster that adopted the nth Beacon SP (see 9.34). Values equal to 0 and greater than ClusterMaxMem are reserved.

9.34.1 General

ClusterTimeOffsetn-1 = ((dot11BeaconPeriod TUs)/ ClusterMaxMem) × (n-1)

and (n=2, 3…, ClusterMaxMem)

The maximum size of the Beacon SP duration field transmitted by a S-PCP/S-AP shall be the Beacon Interval of the S-PCP/S-AP divided by ClusterMaxMem.

9.34.3.4 Centralized PCP/AP cluster MAC requirements

ATXSS CBAP shall last from TBTT + 8×TXSS CBAP Offset + (n-1)×1024×Beacon Interval/TXSS BAP MaxMem until TBTT + 8×TXSS CBAP Offset + (n-1)×1024×Beacon Interval/TXSS CBAP MaxMem + 8×TXSS CBAP Duration for n = 1 to TXSS CBAP MaxMem, excluding any time that overlaps a BIH or an SP that has source and destination DBand AIDs set to 255 (such as for a Beacon SP). The most recently transmitted (if an S-AP) or received (if a member PCP/member AP or non-PCP/non-AP STA) value of TXSS CBAP Offset and TXSS CBAP Duration fields are used.

| 3034 | Chu, Liwen | 9.34.2.2 | 264 | 3 | T | It is not clear if a CCSR can give different CPAC Policy Detail to different S-AP in a CCSC? | Clarify it. |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution: Accept in principle.

Discussion: Since traffic flows, station density, and propagation can vary widely across a floor (and over time), the intent is indeed to allow the CCSR the flexibility to configure different S-APs  with different CPAC Policy Details in order to maximize efficiency. Add a sentence to this effect.

Change

Annex W (informative) Functions of the Centralized Coordination 2 Service Set

Via unspecified means over the DS, a Centralized Coordination Service Root (CCSR):

- Allows APs to enroll or unenroll from the CCSS

- Provides a Directory Service, wherein the CCSR responds to a Directory Service request that includes a MAC address with an indication of whether the MAC address is an S-AP within the CCSS

- Configures the Beacon Interval, ClusterMaxMem, Beacon SP duration , TXSS Offset, TXSS CBAP Duration and TXSS CBAP MaxMem (see 9.34.2.2 Centralized PCP/AP cluster formation) of each S-AP

- Aligns the rate of increment of TSFs of S-APs within the CCSS

- Configures the individual TSF offsets of each S-AP in order to minimize the temporal, spatial and frequency overlap of BIHs of the BSSs in the same CCSS

- Provides to each S-AP the Cluster Time Offset availability information from nearby co-channel S-APs

- Configures each S-AP with certain medium access policies for its Centralized PCP/AP cluster (see 9.34.3.4 Centralized PCP/AP cluster MAC requirements). The CCSR is permitted to configure different S-APs with different medium access policies.

- Configures each S-AP with a list of one or more channels in an Operating Class that the S-AP is excluded from operating upon and that the S-AP advertises via the Channel Usage procedures (see 10.23.14). The excluded channels are available to STAs that are operating in the area covered by the ECPAC, yet are not within the ECPAC.