IEEE P802.11
Wireless LANs

| Proposed Resolutions to Comments related to Secured Connectivity in 802.11af |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2011-6-20 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Chin-Sean Sum | NICT | 3-4, Hikarino-oka, Yokosuka, Kanagawa, Japan, 239-0847 | 046-847-5092 | sum@nict.go.jp |
| Zhou Lan |  |  |  | lan@nict.go.jp |
| Chen Sun |  |  |  | sun@nict.go.jp |
| Yohannes Alemseged |  |  |  | yohannes@nict.go.jp |
| Hiroshi Harada |  |  |  | harada@nict.go.jp |

Abstract

This document presents resolutions to the comments, 1173, 1174, 1175, 1185 and 1186 received in Letter Ballot 171 of 802.11af draft.

| 1173 | 10.12.6 | 59 | 7 | Security is an important requirement when establishing the link from a STA to the database to obtain available channels. Clause 8 has covered security features for the 802.11 air interface. However, since the link between a STA to the database may also involve other medium, further explanations are needed on top of the RLQP. | Include additional explanations on how security features can be enabled when connecting STAs and the RLS through the DS | Accept in principle. Additional information on secured connectivity is given in DCN 11/879r0. |
| --- | --- | --- | --- | --- | --- | --- |

| 1174 | 10.12.6.2.2 | 60 | 40 | The relationship and difference between the terms "RLS", "RLQP Advertisement Server" and "External databases" should be clarified further. | Further explain the terms in the text or the definitions section. Also, "NOTE" in 10.12.6.2.2 and "NOTE1" in 10.12.6.2 should be revised to remove redundant information and to clarify the difference/similarity among the terms. | Accept in principle. Delete NOTE1 in page 59 line 37. Add after the last sentence in NOTE in page 60 line 40, as the following “The RLS is not necessarily capable to transmit or receive WLAN signals, but it is connected to the STA that advertises RLQP by any connectivity means, wireless or wireline.” |
| --- | --- | --- | --- | --- | --- | --- |

| 1175 | 10.12.6.2.1 | 59 | 26 | RLQP is used to transport information such as list of available channels between APs across the DS by employing Ethertype 890d. Although the details are not in the scope of TGaf, the current draft is over simplifying the explanation. | Explain further how the two APs can be connected by referring to the relevant existing specifications. | Accept in principle. Resolve as in DCN 11/879r0. |
| --- | --- | --- | --- | --- | --- | --- |

| 1185 | 10.12.6.2 | 59 | 26 | "When RLQP is transmitted over the DS between the enabling STA and the dependent AP, it uses Ethertype 89-0d frames, as defined in Annex U." If there is no RLS the dependent AP can also use RRB in 11revmb to obtain the list of available channels via another AP that has access to the database | Add procedure or explaination to allow a dependent AP to obtain a list of available channels via another AP. | Accept in principle. Resolve as in DCN 11/879r0. |
| --- | --- | --- | --- | --- | --- | --- |

| 1186 | 10.12.6.2 | 59 | 32 | FCC rules say "TV band devices shall incorporate adequate security measures to ensure that they are capable of communicating for purposes of obtaining lists of available channels only with databases operated by administrators authorized by the Commission, and to ensure that communications between TV band devices and databases between TV band devices are secure to prevent corruption or unauthorized interception of data. This requirement includes implementing security for communications between Mode I personal portable devices and fixed or Mode II devices for purposes of providing lists of available channels." | Security procedure for the transportation of list of available channels should be described. | Accept in principle. Resolve as in DCN 11/879r0. |
| --- | --- | --- | --- | --- | --- | --- |