IEEE P802.11
Wireless LANs

| ANQP TDLS Capability Information |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2011-07-18 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Stephen McCann | Research in Motion | 200 Bath Road, Slough, Berkshire, SL1 3XE, UK | +44 1753 667099 | smccann@rim.com |

### Abstract

This submission proposes a mechanism for TDLS Capability information discovery using ANQP.  This enables the discovery of TDLS capability in a peer STA, at layer 2, prior to the commencement of TDLS.

Note: The original concept was accepted into the TGz draft, then deleted and moved into the TGu draft and then finally deleted due to outstanding cross referencing errors with the final TGz draft at the end of the TGu sponsor ballot process.  It is felt that this message can now be correctly added to 802.11REVmb D9.0.

This document is based on:

11-09-1218-07-000z-tdls-peer-discovery.doc.

11-10-0466-03-000u-tdls-discovery-information.doc

Modify Table 8-176 in the following clause:

8.4.4 Access Network Query Protocol (ANQP) elements

| Table 8-176—ANQP information ID definitions |  |  |
| --- | --- | --- |
| Information Name | Information ID | ANQP Info Element (clause) |
| Reserved | 0-255 | n/a |
| ANQP Query list | 256 | 8.4.4.1 |
| ANQP Capability list | 257 | 8.4.4.2 |
| Venue Name information | 258 | 8.4.4.3 |
| Emergency Call Number information | 259 | 8.4.4.4 |
| Network Authentication Type information | 260 | 8.4.4.5 |
| Roaming Consortium list | 261 | 8.4.4.6 |
| IP Address Type Availability information | 262 | 8.4.4.8 |
| NAI Realm list | 263 | 8.4.4.9 |
| 3GPP Cellular Network information | 264 | 8.4.4.10 |
| AP Geospatial Location | 265 | 8.4.4.11 |
| AP Civic Location | 266 | 8.4.4.12 |
| AP Location Public Identifier URI | 267 | 8.4.4.13 |
| Domain Name list | 268 | 8.4.4.14 |
| Emergency Alert Identifier URI | 269 | 8.4.4.15 |
| TDLS Capability | 270 | 8.4.4.17 |
| Emergency NAI | 271 | 8.4.4.16 |
| Reserved | 272– 56796 | n/a |
| Access Network Query Protocol vendor-specific list | 56797 | 8.4.4.7 |
| Reserved | 56798 – 65535 | n/a |

Add the following clause and renumber successive clauses:

8.4.4.17 TDLS Capability information

The TDLS Capability information contains information that a STA uses to discover the TDLS capabilities of a peer STA. The format of the TDLS Capability information is provided in Figure 8-380a.

|  | Info ID |  | Length | OI | TDLS Capability information |  |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 2 |  | 2 | variable | variable |  |
|  |  | Figure 8-380a—TDLS Capability information format |  |  |  |  |

The Length field is set to the sum of OI field length and TDLS Capability information field length.

The OI field is defined in 8.4.1.31 and indicates the type of TDLS Capability information.

The TDLS Capability information contains information that a STA may use to establish a TDLS link and is defined as an XML schema (see Annex H.1).  An example of the TDLS Capability information is described in 10.24.3.2.9.

Modify Table 10-10 in the following clause:

10.24.3.2 ANQP procedures

| Table 10-10—ANQP usage |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | BSS |  | IBSS |
| Info Name | ANQP Info Element (subclause) | Element Type | AP | Non-AP STA | STA |
| ANQP Query list | 8.4.4.1 | Q | T, R | T, R | T, R |
| ANQP Capability list | 8.4.4.2 | Q | T, R | T, R | T, R |
| Venue Name information | 8.4.4.3 | S | T | R | ---- |
| Emergency Call Number information | 8.4.4.4 | S | T | R | ---- |
| Network Authentication Type information | 8.4.4.5 | S | T | R | ---- |
| Roaming Consortium list | 8.4.4.6 | S | T | R | ---- |
| Access Network Query Protocol vendor-specific list | 8.4.4.7 | Q, S | T, R | T, R | T, R |
| IP Address Type Availability information | 8.4.4.8 | S | T, R | T, R | T, R |
| NAI Realm list | 8.4.4.9 | S | T | R | T, R |
| 3GPP Cellular Network information | 8.4.4.10 | S | T | R | ---- |
| AP Geospatial Location | 8.4.4.11 | S | T | R | T, R |
| AP Civic Location | 8.4.4.12 | S | T | R | T, R |
| The AP Location Public Identifier URI | 8.4.4.13 | S | T | R | T, R |
| Domain Name list | 8.4.4.14 | S | T | R | ---- |
| Emergency Alert Identifier URI | 8.4.4.15 | S | T | R | T, R |
| TDLS Capability | 8.4.4.17 | Q, S | T | R | T, R |
| Emergency NAI | 8.4.4.16 | S | T | R | ---- |
| Symbols Q element is an ANQP Query S element is an ANQP Response T ANQP element may be transmitted by MAC entity R ANQP element may be received by MAC entity --- ANQP element is neither transmitted nor received by MAC entity |  |  |  |  |  |

Add the following clause:

10.24.3.2.9 TDLS Capability procedures

The TDLS Capability element (8.4.4.17) can be used in a ANQP query to discover TDLS capabilities (e.g. another TDLS capable STA) using GAS. An example of TDLS Capability using ANQP is given in Figure 10-26a. The discovery mechanism shall work as follows:

1)	STA1 determines the MAC address of STA2.

2)	STA1 sends a ANQP request to STA2. The ANQP request includes a TDLS Capability element.

3)	STA2 receives the ANQP request frame and includes the TDLS Capability element in the ANQP response frame to STA1.

Figure 10-26a—Example TDLS Capability exchange using ANQP

Insert a new row into clause B.4.22 as follows:

- B.4.22 Interworking (IW) with External Networks extensions

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| IW2.2.22 | Advertisement Protocol ID, Vendor Specific | 8.4.2.95 | IW2.2:O | Yes, No, N/A |
| IW2.2.23 | TDLS Capability | 8.4.4.17 | IW2.2.1:O | Yes, No, N/A |
| IW2.3 | GAS Initial Request frame | 8.5.8.12 | IW2:M | Yes, No, N/A |

Add the following clause at the end of Annex H:

H.1 TDLS Capability information

TDLS Capability information allows TDLS peer STA’s to discover their capabilities. An example of TDLS Capability information could be expressed in an extensible XML format as follows:

<TDLS capability>

<mode> tdls </mode>

<wireless info>

<mac address> 00:01:02:03:04:05 </mac address>

<bssid> 00:0a:0b:0c:0d:0e </bssid>

</wireless info>

< network info>

< DHCP> No</DHCP>

< IP address>192.168.2.1 </IP address>

< Subnet> 255.255.255.0 </subnet>

<Hostname> myphone </Hostname>

</network info>

</TDLS capability>

The minimum required information for TDLS Capability is contained in “wireless info”.  The “mac address” field contains the MAC address of the transmitting STA and the “bssid” identifies the BSS in which the STA is a member. The “network info” could optionally be used to aid the peer devices in establishing a TDLS link.