IEEE P802.11
Wireless LANs

| CID 13006 Proposed Resolution |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2011-07-18 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Stephen McCann | Research in Motion | 200 Bath Road, Slough, Berkshire, SL1 3XE, UK | +44 1753 667099 | smccann@rim.com |

### Abstract

This submission proposes a solution to CID 13006:

“"Editor's Note: What is the ""this method"" cited below?” in Clause 10.24.3.2.6, REVmb Draft 9.0

This document is based on:

Draft P802.11REVmb_D9.1-546.pdf.

Modify the row in Table 10-10 as follows:

| - Table 10-10 ANQP usage |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | BSS |  | IBSS |
| Info Name | ANQP Info Element (subclause) | Element Type | AP | Non-AP STA | STA |
| AP Civic Location | 8.4.4.12 | S | T | R | T, R |
| AP Location Public Identifier URI | 8.4.4.13 | S | T | R | T, R |
| Domain Name list | 8.4.4.14 | S | T | R | ---- |

Modify the following clause:

10.24.3.2.6 AP Geospatial Location procedure

A STA when dot11InterworkingServiceActivated is true may retrieve an AP's Geospatial location using ANQP procedures in 10.24.3.2.  A STA in the associated state should retrieve Geospatial location information from the AP using the procedures in 10.11.9.6.

Modify the following clause:

10.24.3.2.7 AP Civic Location procedure

A STA when dot11InterworkingServiceActivated is true may retrieve an AP's Civic location using ANQP

procedures in 10.24.3.2.  A STA in the associated state should retrieve Civic location information from the AP using the procedures in 10.11.9.9.

Add the following clause and renumber it appropriately

10.24.3.2.7a AP Location Public identifier URI procedure

A STA when dot11InterworkingServiceActivated is true may retrieve an AP's Location Public identifier URI using ANQP procedures in 10.24.3.2.  A STA in the associated state should retrieve Location Public identifier URI information from the AP using the procedures in 10.11.9.10.

Due to security concerns, there are some URI schemes that should be cautiously processed when received by a STA.  For example, URIs using the scheme names "data:" and "http:" may direct applications (e.g. a browser) on the STA to internet pages that contain active scripts. Therefore, URIs received via this ANQP procedure should not be processed in a general manner, as these scripts may be inadvertantly activated.  Instead of listing all the types of URIs that might be misused or potentially have harmful affects, Section 3.3 IANA registers acceptable URI schemes.