IEEE P802.11
Wireless LANs

| D1.0 Comment Resolution – Various MAC comments |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2011-07-18 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Reza Hedayat | Cisco Systems | 2200 E. G. Bush Turnpike, Richardson, TX 75082, USA |  | rehedaya at cisco dot com |

Baseline document: 11ac D1.0.

The document proposes resolutions for the following CIDs:

MAC: 	2786, 3621, 2008, 3493, 2107, 3304, 2946, 3090, 3485, 2955, 3091, 2295, 3373, 2009, 3745, 3495.

| 2786 | Lee, Jae Seung | N | 79.00 | 9.19.2.2 |  |  |  | "SU PPDU", "MU PPDU" are not defined in the draft. | add definition of "SU PPDU" and "MU PPDU" in the definition section | Agree. See Doc 1020r0. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree.

TGac Editor: Please add the following

3.2 Definitions specific to IEEE 802.11

Single-user (SU) physical layer convergence procedure (PLCP) protocol data unit (PPDU):

A PPDU that is intended for a STA.

Multi-user (MU) physical layer convergence procedure (PLCP) protocol data unit (PPDU):

A PPDU that carries independent data streams for one or more STA using MU-MIMO technique.

| 3261 | Reuss, Edward | Y | 79.48 | 9.19.2.2 |  |  |  | "associated with secondary ACs (secondary ACs)" What does this mean? | Clarify or fix this notation. | Agree in principle. See Doc 1020r0. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree in principle. See the proposed editing instructions that is provided below CID 2008.

| 2008 | Aboul-Magd, Osama | N | 79.00 | 9.19.2.2 |  |  |  | The statement, "the sharing of the EDCA TXOP occurs when an EDCAF (associated with the primary AC) has obtained access to the medium..." is misleading and implies that primary AC is determined before the associated EDCAF gained access to the medium. An AC becomes a primary AC only after its associated EDCAF gained access to the medium. | Need to rewrite to reflect the actual sequence of events. | Agree. See Doc 1020r0. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree. The definition of primary AC depend on which EDCAF has obtained access to the medium. See the proposed editing instructions that is provided below CID 3485.

| 3493 | Stacey, Robert | Y | 79.47 | 9.19.2.2 |  |  |  | Is the EDCAF sharing access with another EDCAF or is the EDCAF just obtaining taffic (permitting traffic) from the queues associated with another EDCAF? Also, I think it should be explicitly stated that this is the sharing is actually in MU PPDUs. | "The sharing of the EDCA TXOP occurs when an EDCAF (associated with the primary AC) has obtained access to the medium and includes traffic from queues associated with other ACs in MU PDDUs transmitted during the TXOP." | Agree in principle. See Doc 1020r0. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree in principle. See the proposed editing instructions that is provided below CID 3485.

| 2107 | Au, Edward Kwok Shum | Y | 79.00 | 9.19.2.2 |  |  |  | a) and b) shouldn't be transmitted in a TXOP with the TXOP limit value of 0. Only one PPDU can be transmitted. | combine a) and b) to one item | Agree in principle. See Doc 1020r0. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree in principle. See the proposed editing instructions that is provided below CID 3485.

| 3304 | Rosdahl, Jon | Y | 79.62 | 9.19.2.2 |  |  |  | This reads as if both an SU PPDU and a MU PPDU can both be transmitted | Combine a and b with an "or" | Agree in principle. See Doc 1020r0. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree in principle. See the proposed editing instructions that is provided below CID 3485.

| 2946 | LU, KAIYING | Y | 79.62 | 9.19.2.2 |  |  |  | either a) or b) will happen in a real case. So it's better to combine a) and b) using similar description as d); Or delete b) and describe the MU case in another place. | as comment | Agree. See Doc 1020r0. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree. See the proposed editing instructions that is provided below CID 3485.

| 3090 | Merlin, Simone | Y | 80.09 | 9.19.2.2 |  |  |  | Include VHT beamforming frames: | add: "any frame required for VHT sounding protocol defined in 9.30.5 VHT sounding protocol" | Agree. See Doc 1020r0. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree. Bullet e) is proposed to be expanded to include VHT sounding protocol. See the proposed editing instructions that is provided below CID 3485.

| 3485 | Stacey, Robert | Y | 79.56 | 9.19.2.2 |  |  |  | A normative statement is needed that permits an NDPA-NDP-VHT Compressed Beamforming Report sequence with TXOP Limit 0. Note also that the paragraph in REVmb 8.0 at P597L18 does not list this sequence either. For TG discussion: do we permit the MU sequence (multiple beamformees) when the TXOP limit is 0? | Perhaps expand bullet e) to include a reference to 9.30.5. Clarify rules regarding MU sounding sequence. Delete the normative paragraph in Clause 8, P597L18 of REVmb 8.0 - the paragraph in 9.19.2.2 is sufficient. | Agree. See Doc 1020r0. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree. Bullet e) is proposed to be expanded to include VHT sounding protocol, SU and MU, however it is up to debate whether to permit frame exchanges required for MU. Also the duplicated material is proposed to be deleted from 8.4.2.31, but a reference to 9.19.2.2 is given. See the proposed editing instructions below.

TGac Editor: Please make the following changes

9.19.2.2 EDCA TXOPs

There are three modes of EDCA TXOP defined, the initiation of the EDCA TXOP, the sharing of the

EDCA TXOP, and the multiple frame transmission within an EDCA TXOP. An initiation of the TXOP occurs

when the EDCA rules permit access to the medium. The sharing of the EDCA TXOP occurs when an EDCAF

has obtained access to the medium, making the associated AC the primary AC, and includes traffic from queues associated with other ACs in MU PDDUs transmitted during the TXOP. A multiple frame transmission within the TXOP occurs when an EDCAF retains the right to access the medium following the completion of a frame exchange sequence, such as on receipt of an ACK frame.

The TXOP limit duration values are advertised by the AP in the EDCA Parameter Set element in Beacon and Probe Response frames transmitted by the AP.

A TXOP limit value of 0 indicates that the TXOP holder may transmit or cause to be transmitted (as responses)

the following within the current TXOP:

a) Either an SU PPDU carrying a single MSDU, MMPDU, A-MSDU, or A-MPDU at any rate, subject to

the rules in 9.7 (Multirate support), or an MU PPDU carrying A-MPDUs to different users at any rate, subject to the rules in 9.7 (Multirate support)

b) Any required acknowledgments

c) Any frames required for protection, including one of the following:

1) An RTS/CTS exchange

2) CTS to itself

3) Dual CTS as specified in 9.3.2.8 (Dual CTS protection)

d) Any frames required for beamforming as specified in 9.26 (Sounding PPDUs), or any frame required for VHT sounding protocol defined in 9.30.5 VHT sounding protocol

e) Any frames required for link adaptation as specified in 9.25.2 (Scheduled PSMP)

f) Any number of BlockAckReq and BlockAck frames

TGac Editor: Please make the following changes

8.4.2.31 EDCA Parameter Set element

Change the tenth paragraph as follows:

The value of the TXOP Limit field is specified as an unsigned integer, with the least significant octet transmitted first, in units of 32 s. A TXOP Limit field value of 0 indicates that the TXOP holder may transmit or cause to be transmitted (as responses) within the current TXOP the cases that are listed in 9.19.2.2 (EDCA in TXOPs).

| 2955 | LU, KAIYING | Y | 80.24 | 9.19.2.2 |  |  |  | Curent TXOP limit exceeding exception description is not complete. The TXOP limit could also be exceeded due to sounding process or dynamic bandwidth operation. | Pls specifiy the corresponding TXOP limit exceeding exception due to sounding process and dynamic bandwidth operation. | Agree in principle. See Doc 1020r0. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree in principle. The clarification for including sounding process is added to 9.19.2.2. See proposed editing instructions below CID 3091. However, allowing TXOP limit exceed due to dynamic bandwidth operation requires more debate in TGac. If this is allowed it actually defeats the purpose of bandwidth sensing that is proposed in TGac, since even a single MPDU could cause interference in the recipient’s side. It is therefore proposed not to allow TXOP limit exceed due to dynamic bandwidth operation.

| 3091 | Merlin, Simone | Y | 79.39 | 9.19.2.2 |  |  |  | "9.19.2.2 EDCA TXOPs. When the TXOP limit is non-zero,(#1346) a STA shall fragment an individually addressed(#1359) MSDU so that the transmission of the first MPDU of the TXOP does not cause the TXOP limit to be exceeded at the PHY rate selected for the initial transmission attempt of that MPDU. The TXOP limit may be exceeded, when using a lower PHY rate than selected for the initial transmission attempt of the first MPDU, for a retransmission of an MPDU, for the initial transmission of an MPDU if any previous MPDU in the current MSDU has been retransmitted, or for group addressed(#1359) MSDUs. When the TXOP limit is exceeded due to the retransmission of an MPDU at a reduced PHY rate, the STA shall not transmit more than one MPDU in the TXOP" Sounding feedback 'shall' be sent as a response to a BR Poll, so it should be already implicit that it can exceed the TXOP, without fragmentation; it may be useful to include a clarification in this sentence. | include a clarification in this sentence. | Agree. See Doc 1020r0. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree.

TGac Editor: Please change the 9th paragraph after 9.19.2.2 as follows

9.19.2.2 EDCA TXOPs

When the TXOP limit is non-zero, a STA shall fragment an individually addressed MSDU so that the

transmission of the first MPDU of the TXOP does not cause the TXOP limit to be exceeded at the PHY rate

selected for the initial transmission attempt of that MPDU. The TXOP limit may be exceeded, when using a

lower PHY rate than selected for the initial transmission attempt of the first MPDU, for a retransmission of an

MPDU, for the initial transmission of an MPDU if any previous MPDU in the current MSDU has been

retransmitted, for group addressed MSDUs, or for responding to NDPA or Beamforming Report Poll. When the TXOP limit is exceeded due to the retransmission of an MPDU at a reduced PHY rate, the STA shall not transmit more than one MPDU in the TXOP.

| 2295 | Fischer, Matthew | Y | 80.32 | 9.19.2.2 |  |  |  | ambiguous reference - save what value? | change "save the value" to "save the value of the address" | Agree in principle. See Doc 1020r0. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree.

TGac Editor: Please make the following changes

9.19.2.2 EDCA TXOPs

A STA shall save the TXOP holder address for the BSS in which it is associated, which is the MAC address

from the Address 2 field of the frame that initiated a frame exchange sequence except when this is a CTS

frame, in which case the TXOP holder address is the Address 1 field. If the TXOP holder address is obtained

from a control frame, the STA shall save the value of the address with the Individual/Group bit forced to 0. If an RTS frame is received with the RA address matching the MAC address of the STA and the MAC address in the TA field in the RTS frame matches the saved TXOP holder address, then the STA shall send the CTS frame after SIFS,

without regard for, and without resetting, its NAV. When a STA receives a frame addressed to it that requires

an immediate response, except in the case of an RTS, it shall transmit the response independent of its NAV.

The saved TXOP holder address shall be cleared when the NAV is reset or when the NAV counts down to 0.

| 3373 | Rosdahl, Jon | Y | 80.50 | 9.19.2.2a |  |  |  | "should"? So it's OK to transmit the frames in the secondary AC first? | Clarify | Agree. See Doc 1020r0. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree. See the proposed editing instructions that is provided below CID 3495.

| 2009 | Aboul-Magd, Osama | N | 80.51 | 9.19.2.2a |  |  |  | in the statement, "…the frames in the primary AC queue should be transmitted to the destination first.." | Change "should" to "shall" | Agree. See Doc 1020r0. |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree. See the proposed editing instructions that is provided below CID 3495.

| 3745 | Wu, Tianyu | Y | 80.52 | 9.19.2.2a |  |  |  | If a TXOP is set up by RTS and CTS for MU-MIMO transmission, the RTS to the STAs with primary AC shall be transmitted first. The bandwidth of RTS sent to the STAs with secondary AC shall not be greater than the bandwidth of the CTS responded by the STAs with primary AC. | Change it as suggestted. Please refer to our proposal "Bandwidth request using RTSs for MU-MIMO" for details. | Disagree. See Doc 1020r0. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution: Disagree. Currently the RTS/CTS procedure in VHT does not mandate sending multiple RTSs to multiple recipients.

| 3495 | Stacey, Robert | Y | 80.47 | 9.19.2.2a |  |  |  | This section should limit the sharing description to the MU PDDU, not the EDCA TXOP. Frames from secondary ACs may be included in an MU PDDU provided the duration of the PPDU is determined by primary AC traffic. If we state this simple rule then there is no need for the diagram showing multiple PPDUs and no need to discussion TXOPs with mixtures of SU and MU PPDUs, etc. | Rework section (or move it into 9.19.2.4). Introductory text should state that sharing is achieved when primary AC traffic is transmitted in an MU PPDU and resources permit traffic from secondary ACs to be included without increasing the duration of the MU PPDU. Add a rule: "Secondary AC traffic may be included in an MU PPDU carrying primary AC traffic. The inclusion of secondary AC traffic in an MU PPDU shall not increase the duration of the PPDU beyond that required to transport the primary AC traffic alone." If a diagram is necessary it should show a single MU PPDU where the duration is determined by primary AC traffic and secondary traffic is included with padding to the PPDU duration. (I personally don't think the diagram is necessary). Note also that we can't and shouldn't prevent implementations doing silly things like limiting the primary AC traffic into one stream to free up streams for secondary AC traffic or decreasing the MCS of the primary AC traffic to increase the PPDU duration. | Agree in principle. See Doc 1020r0. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree in principle.

TGac Editor: Please make the following changes

9.19.2.2a Sharing an EDCA TXOP

This mode only applies to an AP that supports DL MU-MIMO. The AC whose EDCAF is granted an EDCA TXOP becomes the primary AC . TXOP sharing is achieved when primary AC traffic is transmitted in an MU PPDU and resources permit traffic from secondary ACs to be included, targeting up to four STAs . The inclusion of secondary AC traffic in an MU PPDU shall not increase the duration of the PPDU beyond that required to transport the primary AC traffic alone. If a destination is targeted by frames in the queues of both primary AC and secondary AC, the frames in the primary AC queue shall be transmitted to the destination first, among a series of downlink transmission within a TXOP. The decision of which secondary ACs and destinations are selected for TXOP sharing, as well as the order of transmissions, are implementation specific and is out of scope of this specification.

NOTE—Each A-MPDU shall contain frames from the same AC as defined in 8.6.3 (A-MPDU contents).

When sharing, the TXOP duration is bounded by the TXOP limit of the primary AC.

An illustration of TXOP sharing is shown in Figure 9-ac1. In this figure, the AP has frames in queues of three of its ACs. It is assumed that the TXOP was obtained by AC_VI and is shared by AC_VO and AC_BE. It is also assumed that these frames are targeting three STAs, STA-1 to STA-3.