IEEE P802.11
Wireless LANs

| CIDs 4030 and 4038 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2011-08-22 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Brian Hart | Cisco Systems |  |  | brianh@cisco.com |

Abstract

This document proposes resolutions to CIDs 4030 and 4038 on Draft 4.0 of TGad

Assumes Draft 4.0 as a baseline.

| 4030 | Hunter, David | 3.2 | 37 | 41 | There is no definition of "PCP/AP cluster". Presumably this is a "PCP cluster or an AP cluster. If so, then need to define "PCP cluster" and "AP cluster". | Define "PCP cluster", "AP cluster", and also "cluster", including the requirements for being a cluster. If a "PCP/AP cluste" may be a cluster of PCPs and legacy APs, then specify that fact also. What functions make a cluster different from a set or group? Specify also how "AP cluster" changes the legacy requirements for an 802.11 AP, or specify that clusters shall only be supported in DBand channels. | Accept in principle. As per 11/yyyyr0, add definition for PCP/AP cluster and explicitly limit PCP/AP clustering to the DBand, so we can be silent about legacy APs. The requirements for being in a cluster are complicated and so continue to be defined once in 9.34. |
| --- | --- | --- | --- | --- | --- | --- | --- |

3.2 Definitions specific to IEEE 802.11

PCP/AP cluster: A collection of nearby co-channel PCPs and/or APs that adopt specific mechanisms to improve spatial sharing and interference mitigation in the DBand

9.34 PCP/AP Clustering

9.34.1 General

A PCP/AP may use the PCP/AP clustering mechanism in the DBand to improve spatial sharing and interference mitigation with other co-channel BSSs. There are two types of clustering:

- Decentralized PCP/AP clustering that involves a single S-PCP/S-AP in the BSA of the S-PCP/S-AP; and

- Centralized PCP/AP clustering where there can be multiple S-APs in the BSA of any one S-AP, and all S-APs are coordinated via a single centralized coordination service set.

| 4038 | Hunter, David | 4.3.4.4 | 42 | 19 | A CCSR is an S-AP or another entity having a globally administered MAC address. Since all STAs are required to have "globally administered" (RAC) MAC addresses, this implies that some of the operating entities defined in this draft do not contain a STA. | Define which operating entities (PCP?) do not contain at least one STA. Why are there any operating entities being proposed that do not depend on the functionality of a STA? | Decline. A PCP does contain a STA.. In 11ad only the CCSR does not contain a STA.. Note 802.11 has “always” included operating entities that do not depend on the functionality of a STA: for instance the portal or external networks (11u). These are typically described in informative annexes, and that is the approach adopted here too. Clause 3, clause 4 and Annex Y collectively provide a rich definition and explanation of the CCSR. |
| --- | --- | --- | --- | --- | --- | --- | --- |