IEEE P802.11
Wireless LANs

| Comment resolution regarding to NCC PICS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2011-09-09 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Chen Sun | NICT | 3-4, Hikarino-oka, Yokosuka, Kanagawa, Japan, 239-0847 | +81 45 847 5062 | sun@nict.go.jp |
| Zhou Lan | NICT | 3-4, Hikarino-oka, Yokosuka, Kanagawa, Japan, 239-0847 |  | lan@nict.go.jp |
| Yohannes Alemseged | NICT | 3-4, Hikarino-oka, Yokosuka, Kanagawa, Japan, 239-0847 |  | yohannes@nict.go.jp |
| Ha Nguyen Tran | NICT | 3-4, Hikarino-oka, Yokosuka, Kanagawa, Japan, 239-0847 |  | haguen@nict.go.jp |
| Hiroshi Harada | NICT | 3-4, Hikarino-oka, Yokosuka, Kanagawa, Japan, 239-0847 |  | harada@nict.go.jp |

Abstract

This document presents resolution to comments received in the 1st letter ballot of 802.11af draft1.03

| CID | Page | Clause | Comment | Proposed Change |
| --- | --- | --- | --- | --- |
| 1146 | 102.26 | B.4.af | The cited references and name of the feature contains a specification for different behaviour for a STA in different roles. It may be that a STA cannot meaningully support all roles (e.g. both AP and non-AP parts), and so would always have to answer "no". What you want is that the STA supports the "relevant" bits related to its role. | For features where different roles perform different behaviours, splits into sub-functions such as Network Channel Control as AP and Network Channel Control as non-AP STA. |

Discussion

The network channel control

Discussion After obtaining the WSM and before starting network operation, the NCC requesting STA may select its preferred frequencies from the WSM and request usage of the selected frequencies for multiple WLAN network channels usage.

The responding ST responding STA may grant permission of using the selected frequencies for multiple WLAN network channels to the NCC requesting STA by using the NCC response frame. The selected network channels can be determined by this STA if this STA has the capability to manage spectrum usage. Also if the responding STA employs RLQP, it can send the request from the NCC requesting STA to the RLSS and return the response obtained from RLQP to the NCC requesting STA.

It can be seen that the STA might have different behaviors. The commenter suggests splitting PICS WS6 into subsections for different behaviors

Propose Revised for CID 1146 per discussion and editing instructions in 11-11/1203r1.

Editing Instructions

B.4.27 TVWS functions

TGaf Editor: Revise PICS of WS6 as shown

| Item | Protocol capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| *WS6 | Network Channel Control (NCC | 6.3.95 (Network channel control management), 8.4.5.4 (RLQP Network Channel Control element), 8.5.8.30 (Network Channel Control frame format), 10.12.7 (Network channel control (NCC)) | CF29:M | Yes □No □N/A □ |
| WS6.1 | NCC requesting STA | 10.12.7.1 (NCC requesting STA) | CF29:M | Yes □No □N/A □ |
| WS6.2 | NCC responding STA | 10.12.7.2 (NCC responding STA) | CF29:M | Yes □No □N/A □ |