IEEE P802.11
Wireless LANs

| ANQP Neighbor Report |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2011-09-21 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Stephen McCann | Research in Motion | 200 Bath Road, Slough, Berkshire, SL1 3XE, UK | +44 1753 667099 | smccann@rim.com |

### Abstract

This submission suggests the addition of a Neighbor Report ANQP-element.  This enables the neighbor report to provide information about neighboring APs to a STA whilst in a pre-associated state.  This is of benefit to STAs that which to discover alternative APs prior to association and determine potential candidates for handover.

Modify the table in the following clause:

- Access Network Query Protocol (ANQP) elements(11u)

| - ANQP-element definitionsinput.doc(11u) |  |  |
| --- | --- | --- |
| ANQP-element name | Info ID | ANQP- (Ed)element (clause) |
| Reserved | 0 – 255 | n/a |
| Query List | 256 | Error: Reference source not found |
| Capability List | 257 | Error: Reference source not found |
| Venue Name | 258 | Error: Reference source not found |
| Emergency Call Number | 259 | Error: Reference source not found |
| Network Authentication Type | 260 | Error: Reference source not found |
| Roaming Consortium | 261 | Error: Reference source not found |
| IP Address Type Availability | 262 | Error: Reference source not found |
| NAI Realm | 263 | Error: Reference source not found |
| 3GPP Cellular Network | 264 | Error: Reference source not found |
| AP Geospatial Location | 265 | Error: Reference source not found |
| AP Civic Location | 266 | Error: Reference source not found |
| AP Location Public Identifier URI | 267 | Error: Reference source not found |
| Domain Name | 268 | Error: Reference source not found |
| Emergency Alert Identifier URI | 269 | Error: Reference source not found |
| TDLS Capability(#13018) | 270 | Error: Reference source not found |
| Emergency NAI | 271 | Error: Reference source not found |
| Neighbor Report | 272 | 8.4.4.18 (Neighbor Report ANQP-element) |
| Reserved | 272– 56796 | n/a |
| Vendor Specific | 56797 | Error: Reference source not found |
| Reserved | 56798 – 65535 | n/a |

Add the following clause and possibly re-number as appropriate

      - Neighbor Report ANQP-element (11u)

The Neighbor Report ANQP-element provides zero or more neighbor reports about neighboring APs.  This is of benefit to a STA in a pre-associated state.

|  | Info ID | Length | Neighbor Report (optional) |  |
| --- | --- | --- | --- | --- |
| Octets: | 2 | 2 | variable |  |
| Figure 8-x18 Neighbor Report ANQP-element format(11u)(#12178) |  |  |  |  |

The Info ID field is equal to the value in ANQP-element definitions corresponding to the Neighbor Report ANQP-element.

The Length is a 2-octet field whose value is equal to the number of octets in the Neighbor Report field.

The format of the Neighbor Report field is defined in 8.4.2.39.

Modify the table in the following clause:

- ANQP procedures(11u)

- General(Ed)

| - ANQP usageinput.doc(11u) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | BSS |  | IBSS |
| ANQP-element Name | ANQP-element (subclause)(Ed) | ANQP-element Type | AP | Non-AP STA | STA |
| Query List | 8.4.4.1 (Query List ANQP-element (11u)) | Q | T, R | T, R | T, R |
| Capabililty List | 8.4.4.2 (Capabililty List ANQP-element(11u)) | S | T, R | T, R | T, R |
| Venue Name | 8.4.4.3 (Venue Name ANQP-element (11u)) | S | T | R | — |
| Emergency Call Number | 8.4.4.4 (Emergency Call Number ANQP-element (11u)) | S | T | R | — |
| Network Authentication Type | 8.4.4.5 (Network Authentication Type ANQP-element(11u)) | S | T | R | — |
| Roaming Consortium | 8.4.4.6 (Roaming Consortium ANQP- element(11u)) | S | T | R | — |
| Vendor Specific | 8.4.4.7 (Vendor Specific ANQP-element(11u)) | Q, S | T, R | T, R | T, R |
| IP Address Type Availability | 8.4.4.8 (IP Address Type Availability ANQP-element (11u)) | S | T, R | T, R | T, R |
| NAI Realm | 8.4.4.9 (NAI Realm ANQP-element(11u)) | S | T | R | T, R |
| 3GPP Cellular Network | 8.4.4.10 (3GPP Cellular Network ANQP-element(11u)) | S | T | R | — |
| AP Geospatial Location | 8.4.4.11 (AP Geospatial Location ANQP-element(11u)) | S | T | R | T, R |
| AP Civic Location | 8.4.4.12 (AP Civic Location ANQP-element(11u)) | S | T | R | T, R |
| (#13006)AP Location Public Identifier URI | 8.4.4.13 (AP Location Public Identifier URI ANQP-element(11u)) | S | T | R | T, R |
| Domain Name | 8.4.4.14 (Domain Name ANQP-element(11u)) | S | T | R | — |
| Emergency Alert Identifier URI | 8.4.4.15 (Emergency Alert URI ANQP-element(11u)) | S | T | R | T, R |
| TDLS Capability (#13018) | 8.4.4.17 (TDLS Capability ANQP-element(#13018)) | Q, S | T,R | T,R | T, R |
| Emergency NAI | 8.4.4.16 (Emergency NAI ANQP-element(11u)) | S | T | R | — |
| Neighbor Report | 8.4.4.18 (Neighbor Report ANQP-element) | S | T | R | - |
| Symbols Q element is an ANQP query S element is an ANQP response T ANQP-element may be transmitted by MAC entity R ANQP-element may be received by MAC entity — ANQP-element is neither transmitted nor received by MAC entity |  |  |  |  |  |

Modify the table in the following clause:

| - Interworking (IW) with external networks extensions(11u)input.doc |  |  |  |  |
| --- | --- | --- | --- | --- |
| Item | Protocol capability | References | Status | Support |
|  | Are the following Interworking with External Networks capabilities supported? |  |  |  |
| IW1 | Interworking capabilities and Information | 8.4.2.94 (Interworking (Ed)element(11u)), 10.24.2 (Interworking capabilities and information) | CF20:M | Yes  No  N/A  |
| IW1.1 | Interworking (Ed)element | 8.4.2.94 (Interworking (Ed)element(11u)) | IW1:M | Yes  No  N/A  |
| IW1.2 | Access network type(#13159) | 8.4.2.94 (Interworking (Ed)element(11u)) | IW1:M | Yes  No  N/A  |
| IW1.3 | Venue type(#13203) | 8.4.2.94 (Interworking (Ed)element(11u)) | IW1:M | Yes  No  N/A  |
| IW1.4 | HESSID | 8.4.2.94 (Interworking (Ed)element(11u)) | IW1:M | Yes  No  N/A  |
| IW2 | Generic Advertisement Service(Ed) | 10.24.3 (Interworking procedures: generic advertisement service (GAS)) | CF20:M | Yes  No  N/A  |
| IW2.1 | Advertisement Protocol element | 8.4.2.95 (Advertisement Protocol element(11u)) | IW2:M | Yes  No  N/A  |
| *IW2.2 (#12155) | GAS Protocol | 10.24.3.1 (GAS Protocol) | IW2:M | Yes  No  N/A  |
| *IW2.2.1 | GAS (#13074)frames(#13151) | 8.5.8 (Public Action details(11k)) | IW2:M | Yes  No  N/A  |
| IW2.2.2 | Access Network Query Protocol | 8.4.4 (Access Network Query Protocol (ANQP) elements(11u)) | IW2.2:M | Yes  No  N/A  |
| IW2.2.3 | Query List | 8.4.4.1 (Query List ANQP-element (11u)) | IW2.2.1:M | Yes  No  N/A  |
| IW2.2.4 | Capabililty List | 8.4.4.2 (Capabililty List ANQP-element(11u)) | IW2.2.1:M | Yes  No  N/A  |
| IW2.2.5 | Venue Name | 8.4.4.3 (Venue Name ANQP-element (11u)) | IW2.2.1:O | Yes  No  N/A  |
| IW2.2.6 | Emergency Call Number | 8.4.4.4 (Emergency Call Number ANQP-element(11u)) | IW2.2.1:O | Yes  No  N/A  |
| IW2.2.7 | Network Authentication Type | 8.4.4.5 (Network Authentication Type ANQP-element (11u)) | IW2.2.1:O | Yes  No  N/A  |
| IW2.2.8 | Roaming Consortium | 8.4.4.6 (Roaming Consortium ANQP-element(11u)) | IW2.2.1:O | Yes  No  N/A  |
| IW2.2.9 | IP Address Type Availability | 8.4.4.8 (IP Address Type Availability ANQP-element (11u)) | IW2.2.1:O | Yes  No  N/A  |
| IW2.2.10 | NAI Realm | 8.4.4.9 (NAI Realm ANQP-element(11u)) | IW2.2.1:O | Yes  No  N/A  |
| IW2.2.11 | 3GPP Cellular Network | 8.4.4.10 (3GPP Cellular Network ANQP-element(11u)) | IW2.2.1:O | Yes  No  N/A  |
| IW2.2.12 | AP Geospatial Location | 8.4.4.11 (AP Geospatial Location ANQP-element(11u)) | IW2.2.1:O | Yes  No  N/A  |
| IW2.2.13 | AP Civic Location | 8.4.4.12 (AP Civic Location ANQP-element(11u)) | IW2.2.1:O | Yes  No  N/A  |
| IW2.2.14 | AP Location Public Identifier URI | 8.4.4.13 (AP Location Public Identifier URI ANQP-element(11u)) | IW2.2.1:O | Yes  No  N/A  |
| IW2.2.15 | Domain Name | 8.4.4.14 (Domain Name ANQP-element(11u)) | IW2.2.1:O | Yes  No  N/A  |
| IW2.2.16 | Emergency Alert URI | 8.4.4.15 (Emergency Alert URI ANQP-element(11u)) | IW2.2.1:O | Yes  No  N/A  |
| IW2.2.17 | Emergency NAI | 8.4.4.16 (Emergency NAI ANQP-element(11u)) | IW2.2.1:O | Yes  No  N/A  |
| IW2.2.18 | Vendor Specific | 8.4.4.7 (Vendor Specific ANQP-element(11u)) | IW2.2.1:O | Yes  No  N/A  |
| IW2.2.19 | MIH IS | 8.4.2.95 (Advertisement Protocol element(11u)), (#12156) 10.24.4 (Interworking procedures: IEEE 802.21 MIH support) | IW2:O | Yes  No  N/A  |
| IW2.2.20 | MIH Event and Command Services Discovery | 8.4.2.95 (Advertisement Protocol element(11u)), (#12156) 10.24.4 (Interworking procedures: IEEE 802.21 MIH support) | IW2.2:O (#12155) | Yes  No  N/A  |
| IW2.2.21 | Emergency Alert System (EAS) | 8.4.2.95 (Advertisement Protocol element(11u)), 8.4.2.99 (Emergency Alert Identifier (Ed)element(11u)) | IW2.2:O (#12155) | Yes  No  N/A  |
| IW2.2.22 (#12157) | Advertisement Protocol ID, Vendor Specific | 8.4.2.95 (Advertisement Protocol element(11u)) | IW2.2:O (#12155) | Yes  No  N/A  |
| IW2.2.23 (#13018) | TDLS Capability | 8.4.4.17 (TDLS Capability ANQP-element(#13018)) | IW2.21:O | Yes  No  N/A  |
| IW2.2.24 (#13018) | Neighbor Report | 8.4.4.18 (Neighbor Report ANQP-element) | IW2.21:O | Yes  No  N/A  |
| IW2.3 | GAS Initial Request frame | 8.5.8.12 (GAS Initial Request frame format(11u)) | IW2:M | Yes  No  N/A  |
| IW2.4 | GAS Initial Response frame | 8.5.8.13 (GAS Initial Response frame format(11u)) | IW2:M | Yes  No  N/A  |
| IW2.5 | GAS Comeback Request frame | 8.5.8.14 (GAS Comeback Request frame format(11u)) | IW2:M | Yes  No  N/A  |
| IW2.6 | GAS Comeback Response frame | 8.5.8.15 (GAS Comeback Response frame format(11u)) | IW2:M | Yes  No  N/A  |
| IW3 | QoS Mapping from External Networks | 10.24.9 (Interworking procedures: support for QoS mapping from external networks), 9.19.4.2 (Contention-based admission control procedures), 9.19.4.3 (Controlled-access admission control) | CF20:O | Yes  No  N/A  |
| IW3.1 | QoS Map Set element | 8.4.2.97 (QoS Map Set (Ed)element (11u)) | IW3:M | Yes  No  N/A  |
| IW3.2 | Transport of QoS Map Set | 10.24.9 (Interworking procedures: support for QoS mapping from external networks) | IW3:M | Yes  No  N/A  |
| IW3.3 | QoS Map Configure | 8.5.3.6 (QoS Map Configure frame format(11u)) | IW3:M | Yes  No  N/A  |
| IW4 | MIH Support | 6.4 (MAC state generic convergence function (MSGCF) (11u)), 10.24.4 (Interworking procedures: IEEE 802.21 MIH support) | CF20:O | Yes  No  N/A  |
| IW4.1 | MAC State Generic Convergence Function Support | 6.4 (MAC state generic convergence function (MSGCF) (11u)) | IW4:M | Yes  No  N/A  |
| IW4.2 | Informational events | 6.4.5 (Convergence function informational events(11u)) | IW4:M | Yes  No  N/A  |
| IW4.3 | ESS status reporting | 6.4.7 (ESS status reporting(11u)) | IW4:M | Yes  No  N/A  |
| IW4.4 | Network configuration | 6.4.8 (Network configuration(11u)) | IW4:M | Yes  No  N/A  |
| IW4.5 | Network events | 6.4.9 (Network events(11u)) | IW4:M | Yes  No  N/A  |
| IW4.6 | Network command interface | 6.4.10 (Network command interface(11u)) | IW4:M | Yes  No  N/A  |
| IW4.7 | Mobility management | 6.4.11 (MAC state SME SAP - mobility management(11u)) | IW4:M | Yes  No  N/A  |
| IW4.8 | Network configuration | 6.4.8 (Network configuration(11u)) | IW4:M | Yes  No  N/A  |
| IW5 | Extended channel switch enabled | 8.4.2.60 (20/40 BSS Intolerant Channel Report element(11n)), 10.1.4 (Acquiring synchronization, scanning) | (CF15 AND DSE9):M | Yes  No  N/A  |
| IW6 | Expedited Bandwidth Request | 8.4.2.96 (Expedited Bandwidth Request (Ed)element(11u)) | CF20:O | Yes  No  N/A  |
| IW7 | SSPN Interface | 10.24.5 (Interworking procedures: interactions with SSPN) | CF20:O | Yes  No  N/A  |