IEEE P802.11
Wireless LANs

| LB 178 TGac Comment Resolution CIDs 2073, 2074, 2318, 2322, 2199, 2243, 2345, 2470, 2471, 2472, 2473, 2477, 2481, 2482, 2616, 2704, 2921, 2961, 3161, 3272, 3611, 3667, 3687, and 3817 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2011-10-06 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Vinko Erceg | Broadcom | 16340 W. Bernardo Dr. San Diego, CA 92127 | +1 858 521 5885 | verceg@broadcom.com |
| Jun Zheng | Broadcom | 16340 W. Bernardo Dr. San Diego, CA 92127 |  | junz@broadcom.com |

Abstract

This submission proposes resolutions to some of the CIDs submitted in TGac LB 178.

The changes marked in this document are based on the TGac Draft 1.1.

Revision History:

rev0 – CIDs 2243 and 2472 deferred

rev1 – CIDs 2073, 2074, 3687 and 2704 are deferred

CID        Sec.         Pg.           Ln.                   Comment                               Proposal

| 2073 | 22.3.18.1 | 181 | 1 | In the channelization in TGac Draft D1.0, there is no condition when the regions between -40dBr and -20dBr in two frequency segments are mutually overlapped, so the definition of this sentence is not needed. | Remove the first sentence in P181. |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree in Principle

Discussion: In the US channel 144 will be used thus allowing three 80 MHz channels next to each other. Also there is a discussion with FCC to release channel 68-96 for unlicensed usage.Therefore, there is (will be) a condition when the regions between -40dBr and -20dBr in two frequency segments are mutually overlapping.

NOTE: For some regulatory domains this configuration is not applicable (CID 3162).

Deferred

| 2074 | 22.3.18.1 | 181 | 19 | In TGac channelization, there is no case when the center frequency of the two 80 MHz channels are separated by 160MHz. In U.S. channelization, there are five 80MHz channels, which center frequencies are 5210, 5290, 5530, 5610, and 5775 MHz. All of the possible values of frequency difference between two segments are: 240, 245, 320, 400, 485, and 565 MHz. Therefore, Figure 22-20 is in an impossible pattern and shall be replaced with one of possible patterns. | Replace Figure 22-20 to new one which center frequency difference between two frequency segments is not 160 MHz but another value (for example, 320MHz). |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree in Principle, see resolution to CID 2073.

Deferred

| 2318 | 22.1.1 | 106 | 27 | "MU transmission supports up to four users with up to four space-time streams per user" would result in 16 spatial streams in total. | Clarification of spatial stream support. My understanding is, that maximum of 8 spatial streams can be allocated to subsets of spatial streams for different users. |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree in Principle

Instructions to Editor: Please modify the text on pg 124 ln 27 as follows:

“An MU transmission supports up to four users with up to four space-time streams per user with the total number of spatial streams not exceeding eight.”

| 2322 | 22.3.18.2 | 182 | 25 | Table 22-19: only 80 MHz Non-HT duplicate, no other "duplicate" Modes in table | Information on other bandwidths (20, 160 MHz) for Non-HT duplicate |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree in Principle, resolved by CID 3225 in 11/1189r2.

| 2199 | 22.1.1 | 106 | 26 | "spatial streams" should be "space-time streams" |  |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree in Principle

Instructions to Editor: Please modify the text on pg 124 ln 26 as follows:

“The VHT PHY extends the number of spatial space-time streams supported to eight and provides support for multi-user (MU) transmissions.”

| 2345 | 22.1.1 | 106 | 54 | "respond to TXBF sounding (provide compressed V feedback)" | "initiate TXBF sounding (send frame exchange including NDP) and respond to TXBF sounding (provide compressed V feedback)" |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree in Principle

Instructions to Editor: Please modify the text on pg 124 ln 54 as follows:

“Initiate transmit beamforming sounding (by sending an NDPA frame followed by a VHT NDP frame) and Rrespond to transmit beamforming sounding (provide compressed beamforming(#2690) feedback)”

| 2470 | 22.3.18.1 | 181 | 1 | What if two segments have rather different peak PSDs? (There is no spec on segment power mismatch) Need to place peak of each 80 MHz mask on peak of 80MHz segment PSD, then perform the combining as written. | As in comment. Add a requirement on segment power alignment |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree in Principle

Instructions to Editor: Please modify the text on pg 201 ln 41-46 as follows:

“Figure 22-21 shows an example of a transmit spectral mask for a non-contiguous transmission using two 80 MHz channels where the center frequency of the two 80 MHz channels are separated by 160 MHz. If the transmit spectral mask specified in dBr, in conjunction with the transmit power, results in a transmit spectral mask requirement of less than -59

dBm/MHz at a given frequency offset, then the transmit spectral mask requirement at that frequency offset shall be -59 dBm/MHz. Different center frequency separation between the two 80 MHz frequency segments of the spectral mask as well as different peak levels of each 80 MHz frequency segment of the spectral mask are possible, in which case a similar procedure in determining the spectral mask as in Figure 22-21 is followed.”

| 2471 | 22.3.18.2 | 182 | 9 | Given we have separate line items for non-HT duplicate, we need to identify the format for 20/40/80/160 (VHT, or HT/VHT?). And non-ht dup should be 40 MHz non-ht dup. Also need a flatness requirement for 160 MHz non-ht-dup | As in comment |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree in Principle, resolved by CID 3225 in 11/1189r2.

| 2477 | 22.3.18.5.2 | 183 | 13 | N … and N is the number of data plus pilot tones | N_ST (see Table 22-4) |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree in Principle

Instructions to Editor: Please modify the text on pg 203 ln 64-65 as follows:

“When RF LO is in the center of the transmitted PPDU BW, the power measured at the center of transmission BW using resolution BW 312.5 KHz(#3333) shall not exceed the average power per subcarrier of the transmitted PPDU, or equivalently, (P – 10log10(NST)), where P is the total transmit power. and N is the number of data plus pilot tones.”

| 2481 | 22.3.18.5.3 | 183 | 40 | (19-89) uses N_ST which is per segment, so cannot apply to 80+80 | REtain 19-89, but test EVM per segment: "as well as each segment of a 80+80 MHz transmission" |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree in Principle

Discussion: It may be desirable to test the two segments of discontiguous 80+80 MHz transmission either separately or together, dependending on the testing instrumentation setup.

Instructions to Editor: Please modify the text on pg 205 ln 23 as follows:

NOTE – In the case the transmit modulation accuracy test is performed simultaneously for the two frequency segments of the non-contiguous 80+80 MHz transmissions, NST in Equation (19-89) represents the total number of subcarriers of both 80 MHz frequency segments.

The test shall be performed over at least 20 frames..”

| 2482 | 22.3.18.5.4 | 184 | 6 | "each freq segment may ..." yet 19-89 applies to one segment only | e.g. may -> shall |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Disagree

Discussion: It may be desirable to test the two segments of discontiguous 80+80 MHz mode separately or together, dependending on the testing instrumentation setup. See also resolution to CID 2481.

| 2616 | 22.1.1 | 106 | 32-34 | The VHT PHY currently supports only 80+80 MHz non-contiguous channel widths. It will be useful to extend this for other channel width combinations while considering operations in overlapping BSSs. | Consider non-contiguous channel bonding for lower channel widths (e.g., 20+ 20, 20+ 40, 40+40 MHz) |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Disagree, see resolution to CID 2938 in document 11/1042r1.

| 2921 | 22.1.1 | 106 | 65 | MCSs 8 and 9 (transmit and receive) are only applicable with certain Nss | Change "MCSs 8 and 9 (transmit and receive)" to "MCSs 8 and 9 (transmit and receive) when operating with appropriate Nss" |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree in Principle

Instructions to Editor: Please modify the text on pg 124 ln 65 as follows:

— MCSs 8 and 9 (transmit and receive) when operating with appropriate NSS and channel width

| 2961 | 22.1.1 | 106 | 50 | In 802.11ac D1.0, 2 or more spatial streams are optional features. However, in 802.11n, 2 spatial streams are mandatory for APs. At this point, 802.11ac does not remain the technical backward compatibility with 802.11n. | Need to remain the technical backward compatibility with 802.11n |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Disagree

Discussion: Single stream 11ac AP can interoperate with multiple stream 11n STAs in single stream modes.  The 11n STAs will know the 11ac AP capability based on the AP’s Supported MCS Set in the HT Capabilities element.

| 3161 | 22.3.18.2 | 182 | 19 | Null sub-carriers should be excluded | change to "-250 to -130, -126 to -6, 6 to 126, and 130 to 250" |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree in Principle, resolved by CID 3225 in 11/1189r2.

| 3272 | 22.3.18.1 | 180 | 180 | I'm concerned about the spectral pedestal at only -28 dBr extending about 80 MHz beyond the band edge when using the 160 MHz mode in the channels adjacent to the band edges. | Consider modifying the spectral mask requirements for the wider bandwidth modes when operating on channels adjacent to the regulatory band edges. |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Disagree

Discussion: 802.11a and 802.11n sprectral mask was not modified for the band edge. Usually transmit power is lowered to meet the regulatory requirements. Also both 11ac and regulatory requirements must be met as indicated by NOTE 1.

| 3817 |  |  |  | I'm concerned about the spectral pedestal at only -28 dBr extending about 80 MHz beyond the band edge when using the 160 MHz mode in the channels adjacent to the band edges. | Consider modifying the spectral mask requirements for the wider bandwidth modes when operating on channels adjacent to the regulatory band edges. |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Disagree, duplicate of CID 3272.

| 3611 | 182 | 22.3.18.3 | 49 | "their phase shall not be required to be correlated." Who is the subject of this shall statement? The writers of the spec? The authors of an implementation? The implementaiton itself? As stated it is meaningless. The only relevant normative language that is possible is for the receiver, where it shall be able to receive a signal from a transmitter that uses separate LOs for the two segments. | Ensure that the normative statement on the receiver is present, then turn the cited para into a note and reword to avoid ridicule. Adam Ant afficianados may disagree. |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree in Principle

Instructions to Editor: Please modify the text on pg 203 ln 32-35 as follows:

“NOTE - If two separate RF LOs are used to generate the lower and upper 80 MHz frequency portions of a transmit signal with TXVECTOR parameter CH_BANDWIDTH set to CBW160 or CBW80+80,  the signal phase of the two 80 MHz frequency portions might not be correlated. shall not be required to be correlated.”

| 2473 | 22.3.18.3 | 182 | 48 | "If two separate LOs .." is not (easily) externally visible | Remove this condition or convert to a note since it is described fully elsewhere |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree in Principle, see resolution to CID 3611.

| 3667 | 22.3.18.5.2 | 183 | 24 | What about non-contiguous transmission using nonadjacent 80 MHz channels where the RF LO does not fall outside both channels? (Or, indeed, what about two LO as allowed by the text previously?) | Add sentence "For non-contiguous transmission using nonadjacent 80 MHz channels, any LO falling inside a channel shall follow the limits above" |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree in Principle

Instructions to Editor: Please modify the text on pg 203 ln 58 as follows:

“TX LO leakage shall meet the following requirements for all bandwidths except non-contiguous 80+80 MHz(#3333) where the RF LO is falling outside both frequency segments:”

Instructions to Editor: Please modify the text on pg 204 ln 7 as follows:

“For non-contiguous 80+80 MHz transmissions, any RF LO falling inside a frequency segment shall follow the limits above. For non-contiguous 80+80 MHz transmission using nonadjacent 80 MHz channels where the RF LO falls outside both channels frequency segments, the RF LO shall follow the spectral mask requirements as defined in 22.3.19.1 22.3.18.1.”

| 3687 | 22.3.18.5.4 | 184 | 40 | "Compute the average of the RMS of all errors in a frame" is a bit vague and the sentense order is not entirely correct | Change to "Compute the RMS of the constellation errors in a frame." or "Compute the RMS of the Euclidean distances in a frame." |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree in Principle

Instructions to Editor: Please modify the text on pg 205 ln 21 as follows:

h) 	Compute the average of the RMS of all errors in a frame as given by Equation (19-89).

h) 	Compute the RMS of the constellation errors in a frame as given by Equation (19-89).

Deferred

| 2704 | 22.3.18.5.4 | 184 | 40 | Looking at Equation (19-89), we are computing the RMS of errors, not the 'average of RMS'. | Remove 'average of' |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree in Principle, see resolution to CID 3687.

Deferred

| 2243 | 22.3.18.2 | 182 | 36 | Should the number of streams be mentioned here? Equal to the number of antennas? |  |
| --- | --- | --- | --- | --- | --- |

Proposed resolution:

Deferred

| 2472 | 22.3.18.2 | 182 | 37 | Q = I implies requirements on number of ports etc. e.g. need to borrow the language at P183L34-36 for this section too | As in comment |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: TBD

Deferred