IEEE P802.11
Wireless LANs

| White Space Map for UK |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-03-10 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Yongho Seok | LG Electronics | Mobile Comm. Lab, LG R&D Complex 533, Hogye1, Dongan, Anyang, Korea | +82-31-450-1947 | yongho.seok@lge.com |
| Wookbong Lee | LG Electronics | Mobile Comm. Lab, LG R&D Complex 533, Hogye1, Dongan, Anyang, Korea | +82-31-450-1883 | wookbong.lee@lge.com |

Abstract

Editing instructions are based on P802.11af Draft 1.06.

## Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGaf Draft.  This introduction is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGaf Draft (i.e. they are instructions to the 802.11 editor on how to merge the text with the baseline documents).

TGaf Editor: Editing instructions preceded by “TGaf Editor” are instructions to the TGaf editor to modify existing material in the TGaf draft.  As a result of adopting the changes, the TGaf editor will execute the instructions rather than copy them to the TGaf Draft.

Discussion:

In “Draft regulatory requirements for white space devices in the UHF TV band” updated on 16, Feb, 2012, the OFCOM asks to provide a single time validity for the parameters to the a master WSD and a slave WSD.

Also, the OFCOM provides the definition of the channel number and the maximum power level for UK.

Propose Adding Channel Number and Maximum Power Level for UK discussion and editing instructions in 11-12/0383r1.

Change #1.

TGaf Editor: Change Table 8-14b Device Class definition value fields as follows:

| Name | Type | Length (octets) | Value | Scope |
| --- | --- | --- | --- | --- |
| Device Class | <ANA> | 1 | The Device Class field is set to an integer that specifies the device’s TVWS band mode of operation as follows: 0: Personal/portable non-AP STA, GDCnonAPSTA, 1: Personal/portable AP STA, GDCAP, 2: Fixed STA, GDCFixedSTA, 3-255: Reserved. | CAQ |

Change #2.

TGaf Editor: In section “8.4.5.2 RLQP Channel Availability Query element”, add following text after line 29 of page 50:

The Device Class field is used to indicate the characteristics of station requesting the channel availability query. The format of the Device Class field is specified in 8.2.6.1.1 (Device class). The element is present only when the reason result code field value is 1.

Change #3.

TGaf Editor: Change Table 8-14d Device Identification Information Value field as follows:

8.2.6.1.2 Device Identification Information

| Name | Length (octets) | Value | Scope |
| --- | --- | --- | --- |
| Device Serial Number | 4 | The Device Serial Number field is set to the manufacturer's serial number. This field is present only if the Device Class field (Table 8-14b (Device Class definition)) is equal to 1 (Personal/portable AP STAGDCAP) or 2 (Fixed STAGDCFixedSTA); otherwise, this field is not present. | CAQ, US |

Change #4.

TGaf Editor: Change Table 8-14k WSM information value fields and texts below as follows:

| Name | Length (octets) | Value | Scope |
| --- | --- | --- | --- |
| Device Class | 1 | The Device Class field is defined in 8.2.6.1.1. The Device Class field is set to a value identifying the Device Class used by the WSM as follows: 0: GDCnonAPSTA, 1: GDCAP, 2: GDCFixedSTA, 3-255: Reserved. Single TLV comprised of fields in Table 8-14b (Device Class definition). | WSM,US |