IEEE P802.11
Wireless LANs

| TGac Comment Resolution Misc PHY CIDs |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-03-13 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Vinko Erceg | Broadcom | 16340 W. Bernardo Dr. San Diego, CA 92127 | +1 858 521 5885 | verceg@broadcom.com |
| Jun Zhen | Broadcom | 16340 W. Bernardo Dr. San Diego, CA 92127 |  |  |

Abstract

This submission proposes resolutions to some of the CIDs submitted in the 802.11ac LB 187. The following CIDs are addressed: CID 4079 4120 4123 4145 4290 4302 4458 4699 4762 4768 4783 4993 5102 5232 5233 5234 5235 5257 5394 5456.

The changes marked in this document are based on the TGac Draft 2.0

CID       Sec.          Pg.            Ln.                   Comment                                        Proposal

| 4079 | 22.3.1 | 175 | 15 | "the PSDU is processed" Doesn't allow for the MU case | "a PSDU (in the SU case) or multiple PSDUs (in the MU case) are processed" |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree.

Instruction to Editor: modify the text on pg. 175 ln. 15 as follows:

“During transmission, a PSDU (in the SU case) or multiple PSDUs (in the MU case) are the PSDU is processed (i.e., scrambled and coded) and appended to the PLCP preamble to create the PPDU. At the receiver, the PLCP preamble is processed to aid in demodulation and delivery of the PSDU.”

| 4120 | 22.6.4.4 | 292 | 8 | The PMD interface carries a number of useless parameters, i.e. those that are handled solely in the PLCP sublayer. Such PLCP processes include: 1. Coding 2. SIG field formatting, decoding 3. TXOP power save 4. Dynamic bandwidth signalling in the service field / TA field | Remove the following parameters from Table 22-63 and at 297.04: STBC, FEC_CODING, GROUP_ID, PARTIAL_AID, CH_BANDWIDTH_IN_NON_HT, DYN_BANDWIDTH _IN_NON_HT |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree.

Instruction to Editor: Delete the following entries from the table on pg. 292:

STBC,  FEC_CODING, GROUP_ID, PARTIAL_AID, CH_BANDWIDTH_IN_NON_HT, DYN_BANDWIDTH_IN_NON_HT

| 4123 | 22.6.5.7.2 | 295 | 46 | "This primitive shall provide the following parameter: PMD_TXPWRLVL.request (TXPWR_LEVEL)" When we have two segments with different power limits, this interface is inadquate. | Add an optional TXPWR_LEVEL_2 parater, present when operating in a 80+80MHz BSS, plus description at 294.54. |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Disagree.

Discussion: there is no definition for different power levels in the specification for two discontiguous segments.

| 4145 | D.2.5 | 332 | 25 | "CCA-ED thresholds are never different from the values in PHY clauses for operation in license-exempt bands." What is the purpose of this statement? Is it attempting to constrain future standardization? If so, it can have no effect, and is not relevant to the standard itself. | Remove cited sentence. |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Disagree.

Discussion: An example of CCA-ED thresholds that are different from the PHY clauses is in E.2.2 3650–3700 MHz in the United States.

| 4290 | 8.3.3.2 | 38 | 20 | "Specifies ... entity" is spurious | Delete. Ditto P40L37 |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree.

Instruction to Editor: delete the following sentence on pg. 38 ln 20:

“Specifies the parameters within the VHT Transmit Power Envelope element that are indicated by the MAC entity.”

Instruction to Editor: delete the following sentence on pg. 40 ln 37:

“Specifies the parameters within the VHT Transmit Power Envelope element that are indicated by the MAC entity.”

| 4302 | 8.4.1.32 | 42 | 28 | "PHY rate in 0.5 Mbps units" - description of rounding behavior is missing from section 8 where it most naturally belongs | As in comment |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree in Principle.

Instruction to Editor: Modify the text on pg. 42 ln. 28 as follows:

“The Rate field contains a 2-octet unsigned integer that specifies the PHY rate in 0.5 Mb/s units. When the PHY rate is not multiples of 0.5Mb/s, a rounding operation is used to obtain the Rate field value in integer.”

| 4458 | 10.22.6.4.2 | 139 | 27 | "may TX" | "may TX up to" or better "PPDUs of bandwidth up to 40 ... 80+80 MHz on a 40 ... 80+80 Mhz direct link respectively |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Disagree.

Discussion: One may say up to maximum signal bandwidth, but in this paragraph possible bandwisths are listed.

| 4699 | 8.3.3.1 | 37 | 44 | This paragraph duplicates information in other places and may be wrong (I can't work out how you get 2356) | Delete the paragraph |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Disagree.

Discussion: It is appropriate to have the sentence in this subclause because it deals with management frame formats. Also I could not find any other place in the specification referring to 2356 octets.

“If a management MPDU is sent using a VHT PPDU, the size of the MPDU is constrained by the maximum

MPDU size supported by the recipient. Otherwise, the maximum management MPDU size is 2356 octets.”

| 4762 | 22 |  |  | Is N_SD per segment or for all segments? In Table 22-5 it's per-segment (80 and 80+80 are the same), but in clause 22.5 it's total (160 and 80+80 are the same) | Fix either Table 22-5 or Tables 22-53 to 22-60, and make sure whatever is chosen aligns with the uses of N_SD elsewhere in clause 22 |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree in Principle.

Instruction to Editor: In Tables 22-53 to 22-60 divide N_SD values by 2 and indicate “N_SD per frequency segment” instead of “N_SD” on top of tables.

| 4768 | 22.3 |  |  | 22.3.19.6 says that the PMD_RSSI.ind is returned during reception of VHT-LTF but Figure 22-27 shows it as being returned just after reception of L-STF. Which is it? | Fix the text, or the figure, or both |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree in Principle.

Discussion: Setion 22.3.19.6 RSSI states:

The RSSI parameter returned in the RXVECTOR shall be calculated from the values of the RSSI parameter

provided by the PMD_RSSI.indication primitive during the reception of the VHT-LTFs such that RSSI

parameter returned in the RXVECTOR shall be a monotonically increasing function of the received power.

Instruction to Editor: Fix Fig 22-27 so that PMD_RSSI.ind is returned at the end of VHT training field in the figure.

| 4783 | 8.5.23.2 | 82 |  | The VHT Compressed Beamforming frame must not contain any vendor-specific elements as this could otherwise make the rule on whether it needs to be segmented ambiguous | Add a paragraph "No vendor-specific elements shall be present in a VHT Compressed Beamforming frame". Also need to specify no use of +HTC? |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Reject.

Discussion:  The ambiguity being referred to, it is assumed, is the following:

A BFER requests BFEE FB and sets a DUR field value in a frame exchange to cover the transmission of the FB frame.  If the BFER is unaware of whether any VS IEs would be present, then the BFER might guess at an incorrectly low value of FB duration – and if the VS IEs might push the total length beyond the maximum MPDU length, then a segmenting might occur that was unexpected, complicating the DUR prediction even further. However, because there are no restrictions on the selected MCS of the FB frame, the DUR is generally unpredictable anyway. So an any attempt to plug this hole of predictability is futile without plugging all of the holes.

| 4993 | D.2 | 332 | 26 | Annex D.2 proposes to change the requirements for CCA-ED, stating the CCA-ED thresholds are "never different" from the values in the PHY clauses, until some amendment makes them different by explicit references in Annex E. | Delete the second sentence, and change the ending of the first sentence to "where they differ from the values in PHY clauses, otherwise they are as stated in PHY clauses." |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree in principle.

Instruction to Editor:

CCA-ED thresholds for operation in specific bands are given in E.2 Band-specific operating requirements

where they could differ from the values in PHY clauses. CCA-ED thresholds for operation in license-exempt bands are stated in PHY clause 22.3.19.5.1.

| 5102 | 20.3.11.3 | 158 | 16 | Clarify requirement | Replace "are defined not to be present" with "shall not be present" |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Accept.

Instruction to Editor: On pg. 158, ln. 16, replace "are defined not to be present" with "shall not be present” .

| 5232 | 22.6.44 | 292 | 62 | CHAN_MAT includes pilot tones | The parameter CHAN_MAT is defined as a (N_SD + N_SP) complex matrices of size (N_RX x N_STS). Note that with the modified modulation of the pilot tones in VHT-LTF, the channel on the pilot tones can not be estimated as a (N_RX x N_STS) matrix. We propose to only exchange the matrices on the data tones. |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Accept.

Instruction to Editor: On pg. 292, ln. 62 replace "(N_SD + N_SP)" with "N_SD" .

| 5233 | 22.6.44 | 292 | 62 | Should CHAN_MAT in Table 22-63 have the same format as CHAN_MAT in Table 22-1? | I assume there is a close connection between CHAN_MAT in Table 22-63 and CHAN_MAT in Table 22-1. Instead of saying "complex matrices", should we desribed CHAN_MAT in Table 22-63 as "compressed beamforming feedback matrices"? |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Reject.

Discussion:  According to the following definition of “PMD_CHAN_MAT.indication” in 22.6.5.11.2:

“The CHAN_MAT parameter contains the channel response matrices that were measured during the reception

of the current PPDU”,

This parameter is inherited from 11n describing the channel matrix from CSI feedback. It really means the entire channel estimation “seen” by receiver, rather than the compressed beamforming matrix that is going to used by beamformer for data steering. Therefore, there is no close connection between “CHAN_MAT” in Table 22-63 and CHAN_MAT in table 22-1.

| 5234 | 22.6.44 | 292 | 62 | EXPANSION_MAT is missing from Table 22-63 | The corresponding table for HT (Table 20-28 in 11mb) contains the parameter "EXPANSION_MAT". Propose to include the same row. Note that PMD_EXPANSION_MAT.request is used in Figure 22-26. |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Accept.

Instruction to Editor: Add the following row to Table 20-28:

| EXPANSION_MAT | PMD_TX_PARAMETERS.request | NSD complex matrices of size (NTX × NSTS) |
| --- | --- | --- |

| 5235 | 22.6.5.2.2 | 293 | 45 | Clarification | Replace "0 bits" with "bits with value 0" |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Accept.

Instruction to Editor: On pg. 293, ln. 45: replace “0 bits” with “bits with value 0”.

| 5257 | 8.4.1.32 | 42 | 28 | The Rate field cannot indicate a rate up to the maximum VHT rate | Add. "If MCS selector is set to 3,4,5 or 6, the Rate field contains a 2-octet unsigned integer that specifies the PHY rate in X Mb/s units;" with some X > 0.5 |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Reject.

Discussion:  SPEC has the following wording in line 28 of Section 8.4.1.32:

“The Rate field contains a 2-octet unsigned integer that specifies the PHY rate in 0.5 Mb/s units.”

2-octet is enough to specify maximum phyrate up for 11AC, i.e. 6.93Gbps with 0.5Mb/s units, whose value equals to 13867 (< 2^16-1).

| 5394 | 22.6.4.4 | 293 | 15 | CBW80+80 is missing | Add "CBW80+80" to the list of enumerated type for CH_BANDWIDTH_IN_NON_HT in Table 22-63. |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Accept.

Instruction to Editor: On pg. 293, ln. 15 replace “CBW20, CBW40, CBW80 and CBW160” with “CBW20, CBW40, CBW80, CBW160 and CBW80+80”.

| 5456 | 22.6.4.4 | 293 | 15 | CBW80+80 should be one of the enumerated types for CH_BANDWIDTH_IN_NON_HT. | Add CBW80+80. |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Accept.

Instruction to Editor: Same as CID 5394.