IEEE P802.11
Wireless LANs

| LB187 - CID 4775 Comment Resolution |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-05-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Wei Shi | CSR | Churchill House, Cambridge, CB4 0WZ, UK |  | wei.shi@csr.com |

Abstract

This document discusses CID 4775 for LB187 and proposes a resolution.

## Comment:

| CID | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 4775 |  |  | A non-VHT STA using L_LENGTH to determine the end of the PPDU will only be able to do this with about 4 us accuracy. This is not sufficient accuracy to determine exactly when the end of the SIFS following the PPDU is, i.e. the slot boundary | Add a NOTE to clarify how non-VHT STAs can accurately determine the next slot boundary | Revised: See editing instruction under Proposal section in 12/0514r2. |

## Discussion

The comment says “non-VHT” but it actually applies to any STA that does not understand short GI. The problem raised is where precisely the end of a frame is which is used as a reference for SIFS, slot boundaries etc.

In IEEE Std 802.11-2012 Clause 9.3.2.3.3 states SIFS is the time from the end of the last symbol. Likewise, Clause 9.3.7 also states timings are from the end of the last symbol. So the definition is very clear in Clause 9. However, in 22.6.5.6.1 PMD_TXEND.confirm is defined to be:

This primitive, generated by the PMD entity, indicates the end of PPDU transmission by the PMD layer. It is generated at the 4 μs boundary following the trailing boundary of the last symbol transmitted.

PMD_TXEND.confirm is used to generate PHY-TXEND.confirm which according to 20.3.22 is emitted at the end of the last symbol and timing references (e.g. SIFS) in 9.3.7 is referenced from PHY-TXEND.confirm. So there is ambiguity here.

The simplest thing to do is to remove the 4 us boundary wording in 22.6.5.6.1 and 22.6.5.6.3 for PMD_TXEND.confirm because the intention in the baseline is for PHY-TXEND.confirm to be at the end of the last symbol of a PPDU. This will remove the ambiguity in 11ac.

4 us boundaries

3.6 us

3.6 us

3.6 us

End of the last symbol

4us boundary after end of the last symbol

Data Sym

Data Sym

Data Sym

## Proposal:

Revised.

TGac editor: Modify current text in 22.6.5.6.1 in D2.2 as follows:

This primitive, generated by the PMD entity, indicates the end of PPDU transmission by the PMD layer. It is

generated at the 4 μs boundary following the trailing boundary of the last symbol transmitted.

TGac editor: Modify current text in 22.6.5.6.3 in D2.2 as follows:

This primitive shall be generated by the PMD entity at the 4 μs boundary following the trailing boundary at the end of the last symbol transmitted.