IEEE P802.11
Wireless LANs

| Resolution for Comments on link adaption |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-05-02 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | Email |
| Bin Chen | Huawei | 3601 Algonquin Road, Rolling Meadows, IL 60008 | +1 (847) 818 1778 | Bin.Chen@huawei.com |
| Yi Luo | Huawei | Huawei Base Bantian Shenzhen |  | Roy.luoyi@huawei.com |
| David Xun Yang | Huawei |  |  |  |
| Osama | Huawei |  |  |  |
| Wangxuehuan | Huawei |  |  |  |
| Eric.Zhangxingxin | Huawei |  |  |  |

Abstract: Resolutions of D2.0 comments on Link Adaptations (9.28.3): 4229, 4230, 5360, 4957

##### CIDs 4229, 4957

| 4229 | Bin Chen | 9.28.3 | 124.41 | "the received PPDU from which the MRQ was triggered" is ambiguous. What is the meaning of MRQ was triggered? Does that means this PPDU is not the PPDU carrying the MRQ? This comment is related to the previous one. If there is a clear description about MRQ in non-NDPA frame, this would be also cleared. | add a (s) after the PPDU. As below. "the received PPDU(s) from which the MRQ was triggered", and need more description about what is the meaning of "trigger" |  |
| --- | --- | --- | --- | --- | --- | --- |
| 4957 | Nir Shapira | 9.28.3 | 122.65 | Wording "associated PPDU" is confusing. | Suggest using "PPDU carryig the MRQ" instead |  |

Discussions:

It is difficult to compute the MFB based on just the PPDU carrying MRQ, and the responder may use the frames other than the MRQ frame to compute the MFB to get more precise and stable estimation, i.e., multiple PPDUs may be used for MFB computation.

Proposed resolution:

Change 1

TGac Editor: please change text in section 9.28.3, page 122 line 65 as below:

On receipt of a VHT variant HT Control field with the MRQ field set to 1, an MFB responder computes the

MCS, N_STS and SNR estimates based on the PPDU(s) from which the MRQ was triggered (i.e. whose RXVECTOR parameters GROUP_ID, N_STS, FEC_CODING, BEAMFORM and STBC match those on the PPDU carrying the MRQ) and labels the result of this computation with

Change 2

TGac Editor: please change text in section 9.28.3, page 124 line 41 as below:

In a solicited MFB response to an MRQ that was not carried in an NDPA frame, the MFB is computed based on RXVECTOR parameters CH_BANDWIDTH, GROUP_ID, NUM_STS, N_TX, FEC_CODING, BEAMFORM

and STBC of the received PPDU(s) from which the MRQ was triggered and may additionally be based on other factors which are not part of the RXVECTOR. The N_STS subfield of the MFB subfield of VHT variant HT Control field shall be set to an equal or smaller value than the RXVECTOR parameter NUM_STS of the received PPDU from which the MRQ was triggered; If the N_STS subfield is set to a smaller value than the RXVECTOR parameter NUM_STS, the MFB responder shall estimate the recommended MCS under the assumption that the MFB requester will transmit the first N_STS space-time streams in the corresponding PPDU carrying MRQ.

##### CIDs 5360

| 5360 | Yi Luo | 9.28.3 | 122.46 | MFB shall be computed based on NDP following a NDPA frame if the MRQ is in the NDPA frame. But for a non-NDPA frame with MRQ set, there is no description for the MFB computation. So, does that mean in a non-NDPA frame with MRQ set, the responder may compute MFB based on that non-NDPA frame or it can compute MFB based on other PPDUs which have same TX features (i.e., bandwidth, group_id, Num-STS, and so on)? | Add a sentence to make it clear. "An MFB responder sending MFB in response to MRQ in an non-NDPA frame, may compute the MFB based on the non-NDPA PPDU or the other PPDU(s) " |  |
| --- | --- | --- | --- | --- | --- | --- |

Discussions:

There is already some text about “response to MRQ in a non-NDPA frame”, but it is on the text a little bit far below that about “response to MRQ in a NDPA frame”. So, just move the text about “MRQ in a NDPA frame” to the place near by “MRQ in a non-NDPA frame”.

Proposed resolution:

Change 1

TGac Editor: please remove text in section 9.28.3, page 122 line 46~47 as below:

Change 2

TGac Editor: please add a paragraph in section 9.28.3, page 122 line 37 as below:

NOTE—The value of GID-L and GID-H fields allows to identify whether the unsolicited feedback is estimated from an SU or MU PPDU.

In a solicited MFB response to an MRQ that was carried in an NDPA frame, the MFB shall be computed based on the NDP following the NDPA frame.

In a solicited MFB response to an MRQ that was not carried in an NDPA frame, the MFB is computed based on RXVECTOR parameters CH_BANDWIDTH, GROUP_ID, NUM_STS, N_TX, FEC_CODING, BEAMFORM

and STBC of the received PPDU from which the MRQ was triggered and may additionally be based on other factors which are not part of the RXVECTOR. The N_STS subfield of the MFB subfield of VHT variant HT Control field shall be set to an equal or smaller value than the RXVECTOR parameter NUM_STS of the received PPDU from which the MRQ was triggered; If the N_STS subfield is set to a smaller value than the RXVECTOR parameter NUM_STS, the MFB responder shall estimate the recommended MCS under the assumption that the MFB requester will transmit the first N_STS space-time streams in the corresponding PPDU carrying MRQ.

##### CIDs 4230

| 4230 | Bin Chen | 9.28.3 | 125.22 | If an MRQ is included in the last PPDU in a TXOP and there is not enough time for a response, the recipient can transmit the response MFB in a subsequent TXOP. If the MFB is computed based on some later PPDU(s) which doesn't carry the MRQ. In this case, the requester can't know that it can use the MFB for longer time than it appears as a solicited MFB for the MRQ. this may trigger a new MRQ and cause overhead. | need a mechanism to let the requester know this situation, so that the requester can use the MFB appropriately. |  |
| --- | --- | --- | --- | --- | --- | --- |

Discussions:

It is a corner case that the responder will response the MFB with long delay, so it is not necessary to to solve it by changing the frame format.

Proposed resolution:

Reject.