IEEE P802.11
Wireless LANs

| TGac Comment Resolution Misc PHY CIDs |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-08-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Vinko Erceg | Broadcom | 16340 W. Bernardo Dr. San Diego, CA 92127 | +1 858 521 5885 | verceg@broadcom.com |

Abstract

This submission proposes resolutions to some of the PHY CIDs submitted in the Task Group LB 188. The following CIDs are addressed: CID 6179, 6182, 6183, 6184, 6358, 6359, 6360, 6379, 6406, 6355, 6354, 6168, 6668, 6664, 6665, 6667, 6666, 6619, 6620, 6621, 6622, 6624, 6842, 6843, 6844, 6845, 6846, 6786, 6487, 6491, 6492, and 6447.

The changes marked in this document are based on TGac Draft 3.0

Revision Notes:

Rev0: CIDs 6406, 6622, 6491, and 6492 were deferred (dependent on other CID resolutions)

CID        Sec.            Pg.          Ln.                   Comment                                  Proposal

| 6179 | 22.2.4.1 | 192 | 31 | An 80 MHz mask PPDU includes non-HT20, HT20 and HT40 transmissions (P4L55). But Clauses 18 and 20 do not define masks for 80 MHz mask PPDUs. Similarly for 160 MHz. Thus, it seems that simply requiring non-HT and HT transmissions to comply with Clause 18 and 20 masks is not adequate. Given that the mask requirements between Clause 18, 20 and 22 are identical, it would be simpler to have mask requirements for VHT STAs to be all referred to 22.3.18.1 for all formats (non-HT/HT/VHT). Note that 22.3.18.1 already lists HT format PPDUs for the 160 MHz mask. Also, non-HT20 is listed in 22.3.18.1 as well. | Add "+ 22.3.18.1 - 20.3.20.1" to the box for Clause 20. Add "+ 22.3.18.1 - 18.3.9.3" to the box for Clause 18. |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree.

Instruction to Editor: On page 192 figure, add "+ 22.3.18.1 - 20.3.20.1" to the box for Clause 20. Add "+ 22.3.18.1 - 18.3.9.3" to the box for Clause 18.

| 6182 | 22.3.18.1 | 265 | 62 | Figure 22-1 (P192L31) and P2L48 state that 20 MHz mask PPDUs of non-HT or HT20 format shall follow the TX mask specified in Clause 18 and 20, respectively. | Change 'For 20 MHz mask PPDU of non-HT, HT or VHT format' to 'For 20 MHz mask PPDU of VHT format'. |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree in Principle.

Instruction to Editor: On pg. 2-3, modify the text as follows:

“

a Clause 20 20 MHz high-throughput (HT) PPDU with the TXVECTOR parameter

CH_BANDWIDTH equal to HT_CBW20 and the CH_OFFSET parameter equal to

CH_OFF_20 transmitted using the 20 MHz transmit spectral mask defined in Clause 20; or

a Clause 22 20 MHz very high throughput (VHT) PPDU with TXVECTOR parameter

CH_BANDWIDTH equal to CBW20 transmitted using the 20 MHz transmit spectral mask

defined in Clause 22;

a Clause 18 PPDU transmitted by a VHT STA using the transmit spectral mask defined in Clause 22; or

a Clause 20 20 MHz high-throughput (HT) PPDU with the TXVECTOR parameter

CH_BANDWIDTH equal to HT_CBW20 and the CH_OFFSET parameter equal to

CH_OFF_20 transmitted by a VHT STA using the 20 MHz transmit spectral mask defined in Clause 22. ”

| 6168 | 3.2 | 3 | 21 | HT/VHT 20 MHz transmissions are allowed under 40 MHz mask PPDU, but why is non-HT 20 MHz transmissions not allowed under 40 MHz mask PPDU? Note that non-HT 20 MHz transmissions are allowed under 80/160 MHz mask PPDUs (P4L54, P5L5). | Add a bullet allowing non-HT 20 MHz transmissions to be allowed under 40 MHz mask PPDU. |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree in principle, see resolution to CID 6183.

| 6183 | 22.3.18.1 | 266 | 33 | It would be simpler to have all mask requirements for VHT STAs be referred to 22.3.18.1 for all formats (non-HT/HT/VHT). | Change 'For 40 MHz mask PPDU of HT, non-HT duplicate or VHT format' to 'For 40 MHz mask PPDU of non-HT, non-HT duplicate, HT or VHT format'. |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree in Principle (see also CID 6168).

Instruction to Editor: On pg. 266, ln 33, modify the text as follows:

“For a 40 MHz mask PPDU of HT, non-HT duplicate or VHT format, the interim transmit spectral mask shall

For a 40 MHz mask PPDU of non-HT, non-HT duplicate, HT or VHT format, the interim transmit spectral mask shall..”

Instruction to Editor: On pg. 3, modify the text as follows:

“

a Clause 22 20 MHz VHT PPDU with the TXVECTOR parameter CH_BANDWIDTH equal

to CBW20 transmitted using the 40 MHz transmit spectral mask defined in Clause 22; or

a 40 MHz very high throughput (VHT) PPDU with the TXVECTOR parameter

CH_BANDWIDTH equal to CBW40 transmitted using the 40 MHz transmit spectral mask

defined in Clause 22.;

a 40 MHz high-throughput (HT) PPDU (TXVECTOR parameter CH_BANDWIDTH equal to

HT_CBW40) transmitted by a VHT STA using the 40 MHz transmit spectral mask defined in Clause 22;

a 20 MHz non-HT PPDU (TXVECTOR parameter CH_BANDWIDTH set to CBW20) transmitted using the 40 MHz transmit spectral mask defined in Clause 20; or

a 20 MHz non-HT PPDU (TXVECTOR parameter CH_BANDWIDTH set to CBW20) transmitted by a VHT STA using the 40 MHz transmit spectral mask defined in Clause 22. “

| 6184 | 22.3.18.1 | 267 | 29 | 80 MHz mask PPDU includes HT format (P4L55). | Change 'For an 80 MHz mask PPDU of non-HT, non-HT duplicate or VHT format' to 'For an 80 MHz mask PPDU of non-HT, non-HT duplicate, HT or VHT format'. |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree in principle.

Instruction to Editor: On pg. 267, ln 28, modify the sentence as follows:

“For an 80 MHz mask PPDU of non-HT, non-HT duplicate, HT or VHT format, the interim transmit spectral..”

Also make the following changes for the 160MHz case:

Instruction to Editor: On pg. 267, ln 65, modify the sentence as follows:

“For a 160 MHz mask PPDU of non-HT, HT, non-HT duplicate, HT or VHT format, the interim transmit spectral..”

| 6358 | 22.3.21 | 281 | 6 | It is not clear what "optional feature of partial AID" does point out on the context of receiver procedure. | Delete it. |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree in principle.

Instruction to Editor: On pg. 281, ln 6, please modify the text as follows:

“This receive procedure and state machine do not describe the operation of optional features, such as LDPC, or STBC or partial AID.”

Instruction to Editor: On pg. 284, ln 38, please modify the text as follows:

“NOTE 1—This procedure does not describe the operation of optional features , such as LDPC, or STBC or partial AID.”

| 6359 | 22.3.21 | 283 | 25 | "VHT-SIG" -> "VHT-SIG-B" | As in comment. |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree.

Instruction to Editor: On pg. 283, ln 25, please modify the text as follows:

“If the VHT-SIG-B field is decoded and the VHT-SIG-B CRC field is checked and valid, a ..”

| 6668 | 22.3.21 | 283 | 25 | Describe what happens when VHT-SIG-B CRC field is not checked. | As in comment |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree in Principle. See also resolution to CID 6359.

Instruction to Editor: On pg. 283, ln 25, please modify the text as follows:

“If the VHT-SIG-B field is decoded and the VHT-SIG-B CRC field is either checked and valid or not checked, a PHY-RXSTART.indication(RXVECTOR) primitive shall be issued.”

| 6360 | 22.3.21 | 284 | 43 | Figure 22-34 illustrates the PLCP receive procedure for an SU transmission. The title should be revised to indicate this is an example of SU case. | Change the title of Figure 22-34 to "PLCP receive procedure for an SU transmission" |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree.

Instruction to Editor: On pg. 284, ln 43, please modify the text as follows:

“Figure 22-34—PLCP receive procedure for an SU transmission”

| 6379 | 22.3.16 | 265 | 42 | 18.3.8.7 only mentions slot times for 20/10/5 MHz channel spacing. In Annex E VHT appears to fall under 80/160 MHz channel spacing. | Need to add some text in the 11ac spec to define slot times for VHT. E.g. "as if the channel spacing were 20MHz." |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree in Princuple.

Instruction to Editor: On pg. 265, ln 42, please modify the text as follows:

18.3.8.7 Slot time

“The slot time is defined in 18.3.8.7 (Slot time). The slot time for the OFDM PHY shall be 9 μs for 20 MHz, 40 MHz, 80 MHz, 160 MHz and 80+80 MHz channel spacing, shall be 13 μs for 10 MHz channel spacing, and shall be 21 μs for 5 MHz channel spacing.

| 6406 | 22.3.21 | 281 | 53 | when the GID and Partial AID of a frame indicate STA is not a target STA, the STA can discard this frame, and it is better to describe what to do in this situation, and make the PLCP receive procedure more clear. | When a STA discard a frame because of GID or Pratial AID not matching, the PHY shall indicate this action to MAC by PHY-RXEND.indication primitive with a error condition(such as "Filtered" or "Discard"). Propose to add a paragraph like: "The PLCP may filter out the PPDU if: -- The Group ID indicates an SU PPDU and the Partial AID is not equal to 0 and does not match its partial AID, or -- MembershipStatusInGroupID[GroupID] is equal to 0 , or -- MembershipStatusInGroupID[GroupID] is equal to 1 and the MU[UserPositionInGroupID[Group ID]] NSTS field in VHT-SIG-A1 is zero. If the PPDU is filtered out, the PLCP shall issue a PHY-RXEND.indication(Filtered) primitive unless the TXOP_PS_NOT_ALLOWED field in VHT-SIG-A1 is equal to 0, in which case, the PLCP shall issue a PHYRXEND.indication(TXOP_PS_Filtered) primitive." |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Defer, dependent on the resolution outcome of CID 6520 and CID 6521.

| 6355 | 22.3.18.1 | 267 | 28 | As defined in section 3.2, an 80 MHz mask PPDU does include HT PPDUs. | Add "HT," just after "non-HT,". |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree, see resolution to CID 6184 of this document.

| 6354 | 22.3.14 | 265 | 5 | channel starting frequency is not 5 000 kHz but 5 000 MHz. Ditto in P265L14 and P265L24. These errors are from the 12/0527r1made by the commentor. | Change "kHz" to "MHz". |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree.

Instruction to Editor: On pg. 265, on lines 5, 14, and 24 change “kHz” to “MHz”

| 6664 | 22.3.18.4.4 | 272 | 57 | After step c) the following is missing which was part of the 11n specification:"d) The frame shall be derotated according to estimated frequency offset." | As in comment |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree in Principle.

Instruction to Editor: On pg. 272, ln 57, please modify the text as follows:

“a) Start of PPDU shall be detected.

b) Transition from L-STF to L-LTF shall be detected and fine timing shall be established.

c) Coarse and fine frequency offsets shall be estimated.

d) Symbols in a packet shall be derotated according to estimated frequency offset.

fd) For each VHT-LTF symbol, transform the symbol into subcarrier received values, estimate the

phase from the pilot subcarriers, and derotate the subcarrier values according to the estimated phase.”

| 6665 | 22.3.18.4.2 | 271 | 28 | It should be clarified that if 20 MHz mask is used for a 20in40 MHz or 20in80 MHz transmission (and similarly 40 MHz mask for a 40in80 MHz transmission), then it is ok to ignore the LO leakage when applying the spectral mask requirements. | As in comment |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree in principle.

Instruction to Editor: On pg. 265, ln 59, please modify the text as follows:

“NOTE 3—For rules regarding TX center frequency leakage levels see section 22.3.18.4.2 (Transmit center frequency

leakage). The spectral mask requirements in this section do not apply to the RF LO.

| 6667 | 22.3.21 | 283 | 25 | Write "VHT-SIG CRC field is.." instead of "VHT-SIG-B CRC field is.." | As in comment |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree, resolved in CID 6359 of this document.

| 6666 | 22.3.21 | 281 | 64 | "the PHY shall decode VHT-SIG-B" but is this true only for MU capable devices? | Please clarify |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Disagree.

Discussion: MU is evident from the sentence: “If the Group ID field in VHT-SIG-A has a value indicating an MU PPDU (see 9.17a (Group ID and partial AID in VHT PPDUs)), the PHY shall decode VHT-SIG-B.”

| 6619 | 22.3.19 | 273 | 22 | As written, this requirement may be impossible to satisfy. There should be two separate requirements: - number of SS equal to number of Tx antennas - number of input ports of DUT equal to this number The second is a requirement on the DUT, not the number of SS | Change sentence to: "The number of spatial streams under test shall be equal to the number of utilized transmitting STA antenna (output) ports. The number of utilized Device under Test input ports shall be equal to this number of spatial streams" |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Disagree.

Discussion: The proposed text reads same as spec text: “The number of spatial streams under test shall be equal to the number of utilized transmitting STA antenna (output) ports and also equal to the number of utilized Device Under Test input ports.” Note underlined text, which has same meaning as second sentence in the proposed change.

| 6620 | 22.3.19.3 | 275 | 14 | The results of nonadjacent channel rejection will be different depending on whether the center frequency of the interfering channel is put at 2 x W or more that that. It should be clear which value to use here. | Change "2 x W or more" to "2 x W" |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Disagree

Discussion: “2 x W or more” was defined since in some instances available channelization may not allow for exactly 2 x W, especially for wider BWs.

| 6621 | 22.3.20 | 278 | 17 | NOTE 1 states that VHT Training symbols are per user. This is not correct. | Delete reference to VHT Training symbol in current text. Add new sentence "The VHT Training symbols are a function of the total number of users" |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree in principle.

Instruction to Editor: On pg. 278, ln 17, please modify the text as follows:

“NOTE 1—for MU the A-MPDU is per user in the MAC sublayer; the VHT Training Symbols, VHT-SIG-B, and CPSDU are per user in the PHY PLCP sublayer; and the VHT Training Symbols, VHT-SIG-B, and Data are per user in the PHY PMD sublayer in Figure 22-32. The VHT Training symbols are a function of the total number of users”

“NOTE 1—for MU the A-MPDU is per user in the MAC sublayer; the VHT-SIG-B and CPSDU are per user and VHT Training Symbols are a function of the total number of users in the PHY PLCP sublayer; and the VHT-SIG-B and Data are per user and VHT Training Symbols are a function of the total number of users in the PHY PMD sublayer in Figure 22-32.”

| 6622 | 22.2.30 | 278 | 60 | Typo | Change "data passed frmo" to "data is passed from" |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Defer, dependent on the resolution outcome of CIDs 6442 and 6206.

| 6624 | 22.3.21 | 282 | 25 | Equation (22-104) uses "L_LENGTH". This is not defined for VHT-format PPDUs (see RXVECTOR) | Either explicitly refer to bits in L-SIG or make L_LENGTH part of RXVECTOR |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Disagree.

Discussion: True, the commenter of CID 6624 is correct that the L_LENGTH isn’t defined in the RXVECTOR for VHT, but since the RXTIME computation of equation 22-104 is done in the PHY, there is no need to pass it through the RXVECTOR.

| 6842 | 22.3.18.4.4 | 272 | 57 | After step c) the following is missing which was part of the 11n specification:"d) The frame shall be derotated according to estimated frequency offset." | As in comment |
| --- | --- | --- | --- | --- | --- |

Duplicate of CID 6664

| 6843 | 22.3.18.4.2 | 271 | 28 | It should be clarified that if 20 MHz mask is used for a 20in40 MHz or 20in80 MHz transmission (and similarly 40 MHz mask for a 40in80 MHz transmission), then it is ok to ignore the LO leakage when applying the spectral mask requirements. | As in comment |
| --- | --- | --- | --- | --- | --- |

Duplicate of CID 6665

| 6844 | 22.3.21 | 281 | 64 | "the PHY shall decode VHT-SIG-B" but is this true only for MU capable devices? | Please clarify |
| --- | --- | --- | --- | --- | --- |

Duplicate of CID 6666

| 6845 | 22.3.21 | 283 | 25 | Write "VHT-SIG CRC field is.." instead of "VHT-SIG-B CRC field is.." T | As in comment |
| --- | --- | --- | --- | --- | --- |

Duplicate of CID 6667

| 6846 | 22.3.21 | 283 | 25 | Describe what happens when VHT-SIG-B CRC field is not checked. | As in comment |
| --- | --- | --- | --- | --- | --- |

Duplicate of CID 6668

| 6786 | 22.3.19.3 | 275 | 21 | "must" is a term that is specifically deprecated in IEEE standards. | Replace "must" with "shall" on both lines 21 and 29. |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree

Instruction to Editor: On pg. 275, ln 21, please modify the text as follows:

“The center frequency of the interfering signal must shall also be at least 160 MHz away from the center frequency..”

| 6487 |  |  |  | "a value not following Equation (22-20)" is strange. How does a value follow an equation? | Add ", i.e. one which isn't a multiple of 3" |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree in principle

Instruction to Editor: On pg. 281, ln 42, please modify the text as follows:

“An invalid L-SIG Length field value is defined as a value that does not satisfy not following Equation (22-20).”

| 6491 | 22.3.21 | 281 |  | The use of PMD_RCPI.ind, PMD_CHAN_MAT.ind and PMD_CBW.ind is not described in the textual walk-through in 22.3.21 | Add suitable text to walk-through |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Defer, dependent on the resolution outcome of CIDs 6442 and 6206.

| 6492 | 22.3.21 | 285 |  | The use of PMD_RCPI.ind, PMD_CHAN_MAT.ind and PMD_CBW.ind is not described in the state machine in 22.3.21 | Add suitable material to figure 22-35 |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Defer, dependent on the resolution outcome of CIDs 6442 and 6206.

| 6447 | 22.3.16 | 265 | 42 | 18.3.8.7 only defines the slot time for 20, 10 and 5 MHz channel spacing, but Annex E makes it clear the channel spacing for VHT is 80 or 160 MHz | Add "as if the channel spacing were 20 MHz" |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree in Principle, same as resolution to the CID 6379.